Register abstraction layer (RAL) model

1-Ethernet RAL package :

package eth_ral_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

// -------------------------------------------------------------------------

// Step 1: Individual register class files

// Each file defines one register class (extends uvm_reg)

// -------------------------------------------------------------------------

// Core operation mode register

`include "eth_moder_reg.sv"

// Interrupt registers (source W1C, mask RW)

`include "eth_int_regs.sv"

// Inter-packet gap and packet length registers

`include "eth_ipg_pkt_regs.sv"

// TX BD number and control mode register

`include "eth_tx_bd_ctrl_regs.sv"

// All six MIIM registers

`include "eth_miim_regs.sv"

// MAC address, hash table, and TX control

`include "eth_mac_hash_txctrl_regs.sv"

// -------------------------------------------------------------------------

// Step 2: Register block

// Instantiates all register objects, creates the memory,

// and builds the WISHBONE address map

// -------------------------------------------------------------------------

`include "eth_reg_block.sv"

// -------------------------------------------------------------------------

// Step 3: WISHBONE adapter, driver, monitor, and agent

// These bridge the RAL to the physical bus

// -------------------------------------------------------------------------

`include "eth_wb_adapter.sv"

// -------------------------------------------------------------------------

// Step 4: Environment and test classes

// Wires everything together and provides test entry points

// -------------------------------------------------------------------------

`include "eth_env_and_tests.sv"

endpackage : eth_ral_pkg

2- Ethernet Registers:

1-MODER (Mode Register)

//=============================================================================

// Register : MODER (Mode Register)

// Address  : 0x00

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_A000

// Bit map:

//   [31:17] Reserved

//   [16]    RECSMALL  - Receive Small Packets

//   [15]    PAD       - Padding Enabled

//   [14]    HUGEN     - Huge Packets Enable

//   [13]    CRCEN     - CRC Enable              (reset=1)

//   [12]    DLYCRCEN  - Delayed CRC Enabled

//   [11]    Reserved

//   [10]    FULLD     - Full Duplex

//   [9]     EXDFREN   - Excess Defer Enabled

//   [8]     NOBCKOF   - No Backoff              (reset=1)

//   [7]     LOOPBCK   - Loop Back

//   [6]     IFG       - Interframe Gap

//   [5]     PRO       - Promiscuous

//   [4]     IAM       - Individual Address Mode

//   [3]     BRO       - Broadcast Address

//   [2]     NOPRE     - No Preamble

//   [1]     TXEN      - Transmit Enable

//   [0]     RXEN      - Receive Enable

// =============================================================================

class eth_moder_reg extends uvm_reg;

`uvm_object_utils(eth_moder_reg)

// -------------------------------------------------------------------------

// Field declarations

// -------------------------------------------------------------------------

rand uvm_reg_field RECSMALL;

rand uvm_reg_field PAD;

rand uvm_reg_field HUGEN;

rand uvm_reg_field CRCEN;

rand uvm_reg_field DLYCRCEN;

rand uvm_reg_field FULLD;

rand uvm_reg_field EXDFREN;

rand uvm_reg_field NOBCKOF;

rand uvm_reg_field LOOPBCK;

rand uvm_reg_field IFG;

rand uvm_reg_field PRO;

rand uvm_reg_field IAM;

rand uvm_reg_field BRO;

rand uvm_reg_field NOPRE;

rand uvm_reg_field TXEN;

rand uvm_reg_field RXEN;

// -------------------------------------------------------------------------

// Constructor

// -------------------------------------------------------------------------

function new(string name = "eth_moder_reg");

// 32-bit register, 1 coverage model

super.new(name, 32, UVM_CVR_ALL);

endfunction

// -------------------------------------------------------------------------

// build() - configure every field

// configure(parent, size, lsb_pos, access, volatile,

//           reset_val, has_reset, is_rand, individually_accessible)

// -------------------------------------------------------------------------

virtual function void build();

RECSMALL = uvm_reg_field::type_id::create("RECSMALL");

PAD      = uvm_reg_field::type_id::create("PAD");

HUGEN    = uvm_reg_field::type_id::create("HUGEN");

CRCEN    = uvm_reg_field::type_id::create("CRCEN");

DLYCRCEN = uvm_reg_field::type_id::create("DLYCRCEN");

FULLD    = uvm_reg_field::type_id::create("FULLD");

EXDFREN  = uvm_reg_field::type_id::create("EXDFREN");

NOBCKOF  = uvm_reg_field::type_id::create("NOBCKOF");

LOOPBCK  = uvm_reg_field::type_id::create("LOOPBCK");

IFG      = uvm_reg_field::type_id::create("IFG");

PRO      = uvm_reg_field::type_id::create("PRO");

IAM      = uvm_reg_field::type_id::create("IAM");

BRO      = uvm_reg_field::type_id::create("BRO");

NOPRE    = uvm_reg_field::type_id::create("NOPRE");

TXEN     = uvm_reg_field::type_id::create("TXEN");

RXEN     = uvm_reg_field::type_id::create("RXEN");

//              parent  sz  lsb  access   vol  reset  has_rst rand  indv

RECSMALL.configure(this, 1, 16, "RW",     0,  1'b0,   1,      1,    0);

PAD     .configure(this, 1, 15, "RW",     0,  1'b0,   1,      1,    0);

HUGEN   .configure(this, 1, 14, "RW",     0,  1'b0,   1,      1,    0);

CRCEN   .configure(this, 1, 13, "RW",     0,  1'b1,   1,      1,    0); // reset=1

DLYCRCEN.configure(this, 1, 12, "RW",     0,  1'b0,   1,      1,    0);

// bit 11 reserved - not declared as a field

FULLD   .configure(this, 1, 10, "RW",     0,  1'b0,   1,      1,    0);

EXDFREN .configure(this, 1,  9, "RW",     0,  1'b0,   1,      1,    0);

NOBCKOF .configure(this, 1,  8, "RW",     0,  1'b1,   1,      1,    0); // reset=1

LOOPBCK .configure(this, 1,  7, "RW",     0,  1'b0,   1,      1,    0);

IFG     .configure(this, 1,  6, "RW",     0,  1'b0,   1,      1,    0);

PRO     .configure(this, 1,  5, "RW",     0,  1'b0,   1,      1,    0);

IAM     .configure(this, 1,  4, "RW",     0,  1'b0,   1,      1,    0);

BRO     .configure(this, 1,  3, "RW",     0,  1'b0,   1,      1,    0);

NOPRE   .configure(this, 1,  2, "RW",     0,  1'b0,   1,      1,    0);

TXEN    .configure(this, 1,  1, "RW",     0,  1'b0,   1,      1,    0);

RXEN    .configure(this, 1,  0, "RW",     0,  1'b0,   1,      1,    0);

endfunction

endclass : eth_moder_reg

2- INT_SOURCE (Interrupt Source Register) //=============================================================================

// Register : INT_SOURCE (Interrupt Source Register)

// Address  : 0x04

// Width    : 32 bits

// Access   : W1C (write 1 to clear each bit)

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:7]  Reserved

//   [6]     RXC   - Receive Control Frame

//   [5]     TXC   - Transmit Control Frame

//   [4]     BUSY  - Busy (buffer discarded)

//   [3]     RXE   - Receive Error

//   [2]     RXB   - Receive Frame

//   [1]     TXE   - Transmit Error

//   [0]     TXB   - Transmit Buffer

// =============================================================================

class eth_int_source_reg extends uvm_reg;

`uvm_object_utils(eth_int_source_reg)

rand uvm_reg_field RXC;

rand uvm_reg_field TXC;

rand uvm_reg_field BUSY;

rand uvm_reg_field RXE;

rand uvm_reg_field RXB;

rand uvm_reg_field TXE;

rand uvm_reg_field TXB;

function new(string name = "eth_int_source_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

RXC  = uvm_reg_field::type_id::create("RXC");

TXC  = uvm_reg_field::type_id::create("TXC");

BUSY = uvm_reg_field::type_id::create("BUSY");

RXE  = uvm_reg_field::type_id::create("RXE");

RXB  = uvm_reg_field::type_id::create("RXB");

TXE  = uvm_reg_field::type_id::create("TXE");

TXB  = uvm_reg_field::type_id::create("TXB");

// W1C: writing 1 clears the bit, writing 0 has no effect

// Hardware sets these bits when events occur

// volatile=1 because hardware can modify these bits

//           parent  sz  lsb  access  vol  reset  has_rst rand  indv

RXC .configure(this,  1,  6,  "W1C",   1,  1'b0,   1,      0,    0);

TXC .configure(this,  1,  5,  "W1C",   1,  1'b0,   1,      0,    0);

BUSY.configure(this,  1,  4,  "W1C",   1,  1'b0,   1,      0,    0);

RXE .configure(this,  1,  3,  "W1C",   1,  1'b0,   1,      0,    0);

RXB .configure(this,  1,  2,  "W1C",   1,  1'b0,   1,      0,    0);

TXE .configure(this,  1,  1,  "W1C",   1,  1'b0,   1,      0,    0);

TXB .configure(this,  1,  0,  "W1C",   1,  1'b0,   1,      0,    0);

endfunction

endclass : eth_int_source_reg

3- INT_MASK (Interrupt Mask Register)

//=============================================================================

// Register : INT_MASK (Interrupt Mask Register)

// Address  : 0x08

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:7]  Reserved

//   [6]     RXC_M  - Receive Control Frame Mask

//   [5]     TXC_M  - Transmit Control Frame Mask

//   [4]     BUSY_M - Busy Mask

//   [3]     RXE_M  - Receive Error Mask

//   [2]     RXF_M  - Receive Frame Mask

//   [1]     TXE_M  - Transmit Error Mask

//   [0]     TXB_M  - Transmit Buffer Mask

// =============================================================================

class eth_int_mask_reg extends uvm_reg;

`uvm_object_utils(eth_int_mask_reg)

rand uvm_reg_field RXC_M;

rand uvm_reg_field TXC_M;

rand uvm_reg_field BUSY_M;

rand uvm_reg_field RXE_M;

rand uvm_reg_field RXF_M;

rand uvm_reg_field TXE_M;

rand uvm_reg_field TXB_M;

function new(string name = "eth_int_mask_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

RXC_M  = uvm_reg_field::type_id::create("RXC_M");

TXC_M  = uvm_reg_field::type_id::create("TXC_M");

BUSY_M = uvm_reg_field::type_id::create("BUSY_M");

RXE_M  = uvm_reg_field::type_id::create("RXE_M");

RXF_M  = uvm_reg_field::type_id::create("RXF_M");

TXE_M  = uvm_reg_field::type_id::create("TXE_M");

TXB_M  = uvm_reg_field::type_id::create("TXB_M");

//            parent  sz  lsb  access  vol  reset  has_rst rand  indv

RXC_M .configure(this, 1,  6,  "RW",   0,  1'b0,   1,      1,    0);

TXC_M .configure(this, 1,  5,  "RW",   0,  1'b0,   1,      1,    0);

BUSY_M.configure(this, 1,  4,  "RW",   0,  1'b0,   1,      1,    0);

RXE_M .configure(this, 1,  3,  "RW",   0,  1'b0,   1,      1,    0);

RXF_M .configure(this, 1,  2,  "RW",   0,  1'b0,   1,      1,    0);

TXE_M .configure(this, 1,  1,  "RW",   0,  1'b0,   1,      1,    0);

TXB_M .configure(this, 1,  0,  "RW",   0,  1'b0,   1,      1,    0);

endfunction

endclass : eth_int_mask_reg

4- IPGT (Back-to-Back Inter Packet Gap Register)

//=============================================================================

// Register : IPGT (Back-to-Back Inter Packet Gap Register)

// Address  : 0x0C

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0012

//

// Bit map:

//   [31:7]  Reserved

//   [6:0]   IPGT - Back to Back Inter Packet Gap value

//           Full duplex recommended: 0x15

//           Half duplex recommended/default: 0x12

// =============================================================================

class eth_ipgt_reg extends uvm_reg;

`uvm_object_utils(eth_ipgt_reg)

rand uvm_reg_field IPGT;

function new(string name = "eth_ipgt_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

IPGT = uvm_reg_field::type_id::create("IPGT");

//        parent  sz  lsb  access  vol  reset       has_rst rand  indv

IPGT.configure(this, 7,  0,  "RW",   0,  7'h12,     1,      1,    0);

//                                         ↑

//                                    reset = 0x12

endfunction

endclass : eth_ipgt_reg

5- IPGR1 (Non Back-to-Back Inter Packet Gap Register 1)

//=============================================================================

// Register : IPGR1 (Non Back-to-Back Inter Packet Gap Register 1)

// Address  : 0x10

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_000C

//

// Bit map:

//   [31:7]  Reserved

//   [6:0]   IPGR1 - Non Back to Back IPG 1

//           Recommended/default: 0x0C

//           Must be within range [0, IPGR2]

// =============================================================================

class eth_ipgr1_reg extends uvm_reg;

`uvm_object_utils(eth_ipgr1_reg)

rand uvm_reg_field IPGR1;

function new(string name = "eth_ipgr1_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

IPGR1 = uvm_reg_field::type_id::create("IPGR1");

//         parent  sz  lsb  access  vol  reset    has_rst rand  indv

IPGR1.configure(this, 7,  0,  "RW",   0,  7'h0C,  1,      1,    0);

endfunction

endclass : eth_ipgr1_reg

6- IPGR2 (Non Back-to-Back Inter Packet Gap Register 2)

//=============================================================================

// Register : IPGR2 (Non Back-to-Back Inter Packet Gap Register 2)

// Address  : 0x14

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0012

//

// Bit map:

//   [31:7]  Reserved

//   [6:0]   IPGR2 - Non Back to Back IPG 2

//           Recommended/default: 0x12

// =============================================================================

class eth_ipgr2_reg extends uvm_reg;

`uvm_object_utils(eth_ipgr2_reg)

rand uvm_reg_field IPGR2;

function new(string name = "eth_ipgr2_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

IPGR2 = uvm_reg_field::type_id::create("IPGR2");

//         parent  sz  lsb  access  vol  reset    has_rst rand  indv

IPGR2.configure(this, 7,  0,  "RW",   0,  7'h12,  1,      1,    0);

endfunction

endclass : eth_ipgr2_reg

7- PACKETLEN (Packet Length Register)

//=============================================================================

// Register : PACKETLEN (Packet Length Register)

// Address  : 0x18

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0040_0600

//

// Bit map:

//   [31:16] MINFL - Minimum Frame Length  (reset = 0x0040 = 64)

//   [15:0]  MAXFL - Maximum Frame Length  (reset = 0x0600 = 1536)

// =============================================================================

class eth_packetlen_reg extends uvm_reg;

`uvm_object_utils(eth_packetlen_reg)

rand uvm_reg_field MINFL;

rand uvm_reg_field MAXFL;

function new(string name = "eth_packetlen_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

MINFL = uvm_reg_field::type_id::create("MINFL");

MAXFL = uvm_reg_field::type_id::create("MAXFL");

//        parent  sz   lsb  access  vol  reset       has_rst rand  indv

MINFL.configure(this, 16,  16, "RW",  0,  16'h0040,  1,      1,    0);

MAXFL.configure(this, 16,   0, "RW",  0,  16'h0600,  1,      1,    0);

endfunction

endclass : eth_packetlen_reg

8- COLLCONF (Collision and Retry Configuration Register)

//=============================================================================

// Register : COLLCONF (Collision and Retry Configuration Register)

// Address  : 0x1C

// Width    : 32 bits

// Access   : RW

// Reset    : 0x000F_003F

//

// Bit map:

//   [31:20] Reserved

//   [19:16] MAXRET   - Maximum Retry         (reset = 0xF = 15)

//   [15:6]  Reserved

//   [5:0]   COLLVALID - Collision Valid Window (reset = 0x3F = 63)

// =============================================================================

class eth_collconf_reg extends uvm_reg;

`uvm_object_utils(eth_collconf_reg)

rand uvm_reg_field MAXRET;

rand uvm_reg_field COLLVALID;

function new(string name = "eth_collconf_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

MAXRET    = uvm_reg_field::type_id::create("MAXRET");

COLLVALID = uvm_reg_field::type_id::create("COLLVALID");

//            parent  sz  lsb  access  vol  reset    has_rst rand  indv

MAXRET   .configure(this, 4, 16, "RW",  0,  4'hF,   1,      1,    0);

COLLVALID.configure(this, 6,  0, "RW",  0,  6'h3F,  1,      1,    0);

endfunction

endclass : eth_collconf_reg

9- TX_BD_NUM (Transmit Buffer Descriptor Number Register)

//=============================================================================

// Register : TX_BD_NUM (Transmit Buffer Descriptor Number Register)

// Address  : 0x20

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0040 (64 TX BDs by default)

//

// Bit map:

//   [31:8]  Reserved

//   [7:0]   TX_BD_NUM - Number of TX Buffer Descriptors

//           Range: 0x00 to 0x80

//           Values > 0x80 are ignored

//           0x80 means all 128 BDs are TX (no RX BDs)

//           0x00 means all 128 BDs are RX (no TX BDs)

//

// Note: Number of RX BDs = 0x80 - TX_BD_NUM

// =============================================================================

class eth_tx_bd_num_reg extends uvm_reg;

`uvm_object_utils(eth_tx_bd_num_reg)

rand uvm_reg_field TX_BD_NUM;

// Constraint: legal range is 0x00 to 0x80

constraint c_tx_bd_num_range {

TX_BD_NUM.value inside {[0:128]};

}

function new(string name = "eth_tx_bd_num_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

TX_BD_NUM = uvm_reg_field::type_id::create("TX_BD_NUM");

//            parent  sz  lsb  access  vol  reset    has_rst rand  indv

TX_BD_NUM.configure(this, 8,  0,  "RW",   0,  8'h40,  1,      1,    0);

//                                               ↑

//                                   default 64 TX BDs (0x40)

endfunction

endclass : eth_tx_bd_num_reg

10- CTRLMODER (Control Module Mode Register)

//=============================================================================

// Register : CTRLMODER (Control Module Mode Register)

// Address  : 0x24

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:3]  Reserved

//   [2]     TXFLOW  - Transmit Flow Control

//   [1]     RXFLOW  - Receive Flow Control

//   [0]     PASSALL - Pass All Receive Frames

// =============================================================================

class eth_ctrlmoder_reg extends uvm_reg;

`uvm_object_utils(eth_ctrlmoder_reg)

rand uvm_reg_field TXFLOW;

rand uvm_reg_field RXFLOW;

rand uvm_reg_field PASSALL;

function new(string name = "eth_ctrlmoder_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

TXFLOW  = uvm_reg_field::type_id::create("TXFLOW");

RXFLOW  = uvm_reg_field::type_id::create("RXFLOW");

PASSALL = uvm_reg_field::type_id::create("PASSALL");

//           parent  sz  lsb  access  vol  reset  has_rst rand  indv

TXFLOW .configure(this, 1,  2,  "RW",   0,  1'b0,  1,      1,    0);

RXFLOW .configure(this, 1,  1,  "RW",   0,  1'b0,  1,      1,    0);

PASSALL.configure(this, 1,  0,  "RW",   0,  1'b0,  1,      1,    0);

endfunction

endclass : eth_ctrlmoder_reg

11- MIIMODER (MII Mode Register)

//=============================================================================

// Register : MIIMODER (MII Mode Register)

// Address  : 0x28

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0064

//

// Bit map:

//   [31:9]  Reserved

//   [8]     MIINOPRE - No Preamble (0=32-bit preamble, 1=no preamble)

//   [7:0]   CLKDIV   - Clock Divider (reset=0x64=100)

//           MDC = Host_CLK / (2 * CLKDIV)

// =============================================================================

class eth_miimoder_reg extends uvm_reg;

`uvm_object_utils(eth_miimoder_reg)

rand uvm_reg_field MIINOPRE;

rand uvm_reg_field CLKDIV;

function new(string name = "eth_miimoder_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

MIINOPRE = uvm_reg_field::type_id::create("MIINOPRE");

CLKDIV   = uvm_reg_field::type_id::create("CLKDIV");

//            parent  sz  lsb  access  vol  reset    has_rst rand  indv

MIINOPRE.configure(this, 1,  8,  "RW",   0,  1'b0,   1,      1,    0);

CLKDIV  .configure(this, 8,  0,  "RW",   0,  8'h64,  1,      1,    0);

//                                              ↑

//                                   default divider = 100 (0x64)

endfunction

endclass : eth_miimoder_reg

12- MIICOMMAND (MII Command Register)

//=============================================================================

// Register : MIICOMMAND (MII Command Register)

// Address  : 0x2C

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:3]  Reserved

//   [2]     WCTRLDATA - Write Control Data to PHY

//   [1]     RSTAT     - Read Status from PHY

//   [0]     SCANSTAT  - Scan Status (continuous read)

//

// Note: Only one command bit should be set at a time.

//       BUSY in MIISTATUS is set while command is executing.

//       Next command must wait until BUSY=0.

// =============================================================================

class eth_miicommand_reg extends uvm_reg;

`uvm_object_utils(eth_miicommand_reg)

rand uvm_reg_field WCTRLDATA;

rand uvm_reg_field RSTAT;

rand uvm_reg_field SCANSTAT;

// Constraint: only one command bit at a time

constraint c_one_command {

$onehot0({WCTRLDATA.value, RSTAT.value, SCANSTAT.value});

}

function new(string name = "eth_miicommand_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

WCTRLDATA = uvm_reg_field::type_id::create("WCTRLDATA");

RSTAT     = uvm_reg_field::type_id::create("RSTAT");

SCANSTAT  = uvm_reg_field::type_id::create("SCANSTAT");

//             parent  sz  lsb  access  vol  reset  has_rst rand  indv

WCTRLDATA.configure(this, 1,  2,  "RW",   0,  1'b0,  1,      1,    0);

RSTAT    .configure(this, 1,  1,  "RW",   0,  1'b0,  1,      1,    0);

SCANSTAT .configure(this, 1,  0,  "RW",   0,  1'b0,  1,      1,    0);

endfunction

endclass : eth_miicommand_reg

13- MIIADDRESS (MII Address Register)

//=============================================================================

// Register : MIIADDRESS (MII Address Register)

// Address  : 0x30

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:13] Reserved

//   [12:8]  RGAD - Register Address within selected PHY  (5 bits)

//   [7:5]   Reserved

//   [4:0]   FIAD - PHY Address                           (5 bits)

// =============================================================================

class eth_miiaddress_reg extends uvm_reg;

`uvm_object_utils(eth_miiaddress_reg)

rand uvm_reg_field RGAD;

rand uvm_reg_field FIAD;

function new(string name = "eth_miiaddress_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

RGAD = uvm_reg_field::type_id::create("RGAD");

FIAD = uvm_reg_field::type_id::create("FIAD");

//       parent  sz  lsb  access  vol  reset  has_rst rand  indv

RGAD.configure(this, 5,  8,  "RW",   0,  5'h0,  1,      1,    0);

FIAD.configure(this, 5,  0,  "RW",   0,  5'h0,  1,      1,    0);

endfunction

endclass : eth_miiaddress_reg

14- MIITX_DATA (MII Transmit Data Register)

//=============================================================================

// Register : MIITX_DATA (MII Transmit Data Register)

// Address  : 0x34

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:16] Reserved

//   [15:0]  CTRLDATA - Control Data to write to PHY register

// =============================================================================

class eth_miitx_data_reg extends uvm_reg;

`uvm_object_utils(eth_miitx_data_reg)

rand uvm_reg_field CTRLDATA;

function new(string name = "eth_miitx_data_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

CTRLDATA = uvm_reg_field::type_id::create("CTRLDATA");

//           parent  sz   lsb  access  vol  reset     has_rst rand  indv

CTRLDATA.configure(this, 16,   0,  "RW",   0,  16'h0,   1,      1,    0);

endfunction

endclass : eth_miitx_data_reg

15- MIIRX_DATA (MII Receive Data Register)

//=============================================================================

// Register : MIIRX_DATA (MII Receive Data Register)

// Address  : 0x38

// Width    : 32 bits

// Access   : RO (read only - written by hardware after PHY read)

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:16] Reserved

//   [15:0]  PRSD - Received Data from PHY (read only)

// =============================================================================

class eth_miirx_data_reg extends uvm_reg;

`uvm_object_utils(eth_miirx_data_reg)

rand uvm_reg_field PRSD;

function new(string name = "eth_miirx_data_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

PRSD = uvm_reg_field::type_id::create("PRSD");

// RO: read only, hardware writes this after PHY read completes

// volatile=1: hardware updates this field

// rand=0: we never randomize read-only fields

//       parent  sz   lsb  access  vol  reset    has_rst rand  indv

PRSD.configure(this, 16,   0,  "RO",   1,  16'h0,  1,      0,    0);

endfunction

endclass : eth_miirx_data_reg

16- MIISTATUS (MII Status Register)

//=============================================================================

// Register : MIISTATUS (MII Status Register)

// Address  : 0x3C

// Width    : 32 bits

// Access   : RO (all bits read-only, set/cleared by hardware)

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:3]  Reserved

//   [2]     NVALID   - Invalid (1=data invalid during scan)

//   [1]     BUSY     - MII busy (1=operation in progress)

//   [0]     LINKFAIL - Link Failed

// =============================================================================

class eth_miistatus_reg extends uvm_reg;

`uvm_object_utils(eth_miistatus_reg)

rand uvm_reg_field NVALID;

rand uvm_reg_field BUSY;

rand uvm_reg_field LINKFAIL;

function new(string name = "eth_miistatus_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

NVALID   = uvm_reg_field::type_id::create("NVALID");

BUSY     = uvm_reg_field::type_id::create("BUSY");

LINKFAIL = uvm_reg_field::type_id::create("LINKFAIL");

// All RO: hardware sets/clears these

// volatile=1: hardware modifies these fields

// rand=0: never randomize read-only fields

//          parent  sz  lsb  access  vol  reset  has_rst rand  indv

NVALID  .configure(this, 1,  2,  "RO",   1,  1'b0,  1,      0,    0);

BUSY    .configure(this, 1,  1,  "RO",   1,  1'b0,  1,      0,    0);

LINKFAIL.configure(this, 1,  0,  "RO",   1,  1'b0,  1,      0,    0);

endfunction

endclass : eth_miistatus_reg

17- MAC_ADDR0 (MAC Address Register  0)

//=============================================================================

// Register : MAC_ADDR0 (MAC Address Register 0)

// Address  : 0x40

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:24] Byte 2 of MAC address

//   [23:16] Byte 3 of MAC address

//   [15:8]  Byte 4 of MAC address

//   [7:0]   Byte 5 of MAC address

//

// Note: Address transmitted byte 0 first (byte 0 is in MAC_ADDR1[15:8])

//       Full 48-bit MAC = {MAC_ADDR1[15:8], MAC_ADDR1[7:0],

//                          MAC_ADDR0[31:24], MAC_ADDR0[23:16],

//                          MAC_ADDR0[15:8],  MAC_ADDR0[7:0]}

// =============================================================================

class eth_mac_addr0_reg extends uvm_reg;

`uvm_object_utils(eth_mac_addr0_reg)

rand uvm_reg_field BYTE2;

rand uvm_reg_field BYTE3;

rand uvm_reg_field BYTE4;

rand uvm_reg_field BYTE5;

function new(string name = "eth_mac_addr0_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

BYTE2 = uvm_reg_field::type_id::create("BYTE2");

BYTE3 = uvm_reg_field::type_id::create("BYTE3");

BYTE4 = uvm_reg_field::type_id::create("BYTE4");

BYTE5 = uvm_reg_field::type_id::create("BYTE5");

//        parent  sz  lsb  access  vol  reset   has_rst rand  indv

BYTE2.configure(this, 8, 24, "RW",  0,  8'h0,   1,      1,    0);

BYTE3.configure(this, 8, 16, "RW",  0,  8'h0,   1,      1,    0);

BYTE4.configure(this, 8,  8, "RW",  0,  8'h0,   1,      1,    0);

BYTE5.configure(this, 8,  0, "RW",  0,  8'h0,   1,      1,    0);

endfunction

endclass : eth_mac_addr0_reg

18- MAC_ADDR1 (MAC Address Register 1)

// =============================================================================

// Register : MAC_ADDR1 (MAC Address Register 1)

// Address  : 0x44

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:16] Reserved

//   [15:8]  Byte 0 of MAC address (transmitted first)

//   [7:0]   Byte 1 of MAC address

// =============================================================================

class eth_mac_addr1_reg extends uvm_reg;

`uvm_object_utils(eth_mac_addr1_reg)

rand uvm_reg_field BYTE0;

rand uvm_reg_field BYTE1;

function new(string name = "eth_mac_addr1_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

BYTE0 = uvm_reg_field::type_id::create("BYTE0");

BYTE1 = uvm_reg_field::type_id::create("BYTE1");

//        parent  sz  lsb  access  vol  reset   has_rst rand  indv

BYTE0.configure(this, 8, 8,  "RW",  0,  8'h0,   1,      1,    0);

BYTE1.configure(this, 8, 0,  "RW",  0,  8'h0,   1,      1,    0);

endfunction

endclass : eth_mac_addr1_reg

19- HASH0 (Hash Register 0)

// =============================================================================

// Register : HASH0 (Hash Register 0)

// Address  : 0x48

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:0]  Hash0 - Lower 32 bits of 64-bit hash table

//

// Note: Together HASH0 and HASH1 form a 64-bit hash table.

//       Used when IAM=1 in MODER for multicast address filtering.

//       CRC of 48-bit address → 6-bit index → bit in HASH0/HASH1

// =============================================================================

class eth_hash0_reg extends uvm_reg;

`uvm_object_utils(eth_hash0_reg)

rand uvm_reg_field HASH0;

function new(string name = "eth_hash0_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

HASH0 = uvm_reg_field::type_id::create("HASH0");

//         parent  sz   lsb  access  vol  reset      has_rst rand  indv

HASH0.configure(this, 32,   0,  "RW",   0,  32'h0,    1,      1,    0);

endfunction

endclass : eth_hash0_reg

20- HASH1 (Hash Register 1)

// =============================================================================

// Register : HASH1 (Hash Register 1)

// Address  : 0x4C

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:0]  Hash1 - Upper 32 bits of 64-bit hash table

// =============================================================================

class eth_hash1_reg extends uvm_reg;

`uvm_object_utils(eth_hash1_reg)

rand uvm_reg_field HASH1;

function new(string name = "eth_hash1_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

HASH1 = uvm_reg_field::type_id::create("HASH1");

//         parent  sz   lsb  access  vol  reset      has_rst rand  indv

HASH1.configure(this, 32,   0,  "RW",   0,  32'h0,    1,      1,    0);

endfunction

endclass : eth_hash1_reg

21- TXCTRL (TX Control Register)

// =============================================================================

// Register : TXCTRL (TX Control Register)

// Address  : 0x50

// Width    : 32 bits

// Access   : RW

// Reset    : 0x0000_0000

//

// Bit map:

//   [31:17] Reserved

//   [16]    TXPAUSERQ - TX Pause Request

//           Writing 1 starts sending PAUSE control frame.

//           Bit is automatically cleared to 0 after frame sent.

//   [15:0]  TXPAUSETV - TX Pause Timer Value

//           Value sent in the PAUSE control frame timer field.

// ============================================================================

class eth_txctrl_reg extends uvm_reg;

`uvm_object_utils(eth_txctrl_reg)

rand uvm_reg_field TXPAUSERQ;

rand uvm_reg_field TXPAUSETV;

function new(string name = "eth_txctrl_reg");

super.new(name, 32, UVM_CVR_ALL);

endfunction

virtual function void build();

TXPAUSERQ = uvm_reg_field::type_id::create("TXPAUSERQ");

TXPAUSETV = uvm_reg_field::type_id::create("TXPAUSETV");

// TXPAUSERQ: writing 1 triggers PAUSE frame, auto-clears

The TPAUSERQ signal (request) is latched internally in the MAC Control Generator and reset after the PAUSE control frame has been transmitted.

// We model as RW because host writes 1 to request

// Hardware clears it automatically (volatile=1)

//             parent  sz   lsb  access  vol  reset     has_rst rand  indv

TXPAUSERQ.configure(this,  1,  16,  "RW",   1,  1'b0,   1,      1,    0);

TXPAUSETV.configure(this, 16,   0,  "RW",   0,  16'h0,  1,      1,    0);

endfunction

endclass : eth_txctrl_reg

3-Register block :

=============================================================================

// File      : eth_reg_block.sv

// Purpose   : Complete UVM Register Block for Ethernet MAC IP Core

//

// Contains:

//   - All 21 configuration/status registers

//   - BD RAM modeled as uvm_mem (256 x 32-bit words)

//   - Single WISHBONE register map

//   - Adapter connection point

//

// Register map (WISHBONE slave interface):

//   0x00  MODER

//   0x04  INT_SOURCE

//   0x08  INT_MASK

//   0x0C  IPGT

//   0x10  IPGR1

//   0x14  IPGR2

//   0x18  PACKETLEN

//   0x1C  COLLCONF

//   0x20  TX_BD_NUM

//   0x24  CTRLMODER

//   0x28  MIIMODER

//   0x2C  MIICOMMAND

//   0x30  MIIADDRESS

//   0x34  MIITX_DATA

//   0x38  MIIRX_DATA

//   0x3C  MIISTATUS

//   0x40  MAC_ADDR0

//   0x44  MAC_ADDR1

//   0x48  HASH0

//   0x4C  HASH1

//   0x50  TXCTRL

//   0x400-0x7FF  BD RAM (128 BDs x 8 bytes = 256 x 32-bit words)

// =============================================================================

// Include all register class files

`include "eth_moder_reg.sv"

`include "eth_int_regs.sv"

`include "eth_ipg_pkt_regs.sv"

`include "eth_tx_bd_ctrl_regs.sv"

`include "eth_miim_regs.sv"

`include "eth_mac_hash_txctrl_regs.sv"

class eth_reg_block extends uvm_reg_block;

`uvm_object_utils(eth_reg_block)

// =========================================================================

// Register handles — one per register in the DUT

// =========================================================================

// Core mode and control

rand eth_moder_reg        MODER;        // 0x00

rand eth_int_source_reg   INT_SOURCE;   // 0x04

rand eth_int_mask_reg     INT_MASK;     // 0x08

// Inter-packet gap

rand eth_ipgt_reg         IPGT;         // 0x0C

rand eth_ipgr1_reg        IPGR1;        // 0x10

rand eth_ipgr2_reg        IPGR2;        // 0x14

// Packet length and collision

rand eth_packetlen_reg    PACKETLEN;    // 0x18

rand eth_collconf_reg     COLLCONF;     // 0x1C

// Buffer descriptor and flow control

rand eth_tx_bd_num_reg    TX_BD_NUM;    // 0x20

rand eth_ctrlmoder_reg    CTRLMODER;    // 0x24

// MIIM interface

rand eth_miimoder_reg     MIIMODER;     // 0x28

rand eth_miicommand_reg   MIICOMMAND;   // 0x2C

rand eth_miiaddress_reg   MIIADDRESS;   // 0x30

rand eth_miitx_data_reg   MIITX_DATA;  // 0x34

rand eth_miirx_data_reg   MIIRX_DATA;  // 0x38  (RO)

rand eth_miistatus_reg    MIISTATUS;    // 0x3C  (RO)

// MAC addressing

rand eth_mac_addr0_reg    MAC_ADDR0;    // 0x40

rand eth_mac_addr1_reg    MAC_ADDR1;    // 0x44

// Hash table for multicast

rand eth_hash0_reg        HASH0;        // 0x48

rand eth_hash1_reg        HASH1;        // 0x4C

// TX flow control

rand eth_txctrl_reg       TXCTRL;       // 0x50

// =========================================================================

// BD RAM — modeled as uvm_mem

// 256 locations x 32 bits = 1024 bytes

// WISHBONE address range: 0x400 - 0x7FC

// =========================================================================

uvm_mem eth_bd_mem;

// =========================================================================

// Register map — single WISHBONE slave map

// =========================================================================

uvm_reg_map reg_map;

// =========================================================================

// Constructor

// =========================================================================

function new(string name = "eth_reg_block");

// UVM_NO_COVERAGE: use UVM_CVR_ALL to enable all coverage

super.new(name, UVM_NO_COVERAGE);

endfunction

// =========================================================================

// build() — create and configure all registers and the map

// =========================================================================

virtual function void build();

// ---------------------------------------------------------------------

// Step 1: Create all register objects

// ---------------------------------------------------------------------

MODER      = eth_moder_reg::type_id::create("MODER");

INT_SOURCE = eth_int_source_reg::type_id::create("INT_SOURCE");

INT_MASK   = eth_int_mask_reg::type_id::create("INT_MASK");

IPGT       = eth_ipgt_reg::type_id::create("IPGT");

IPGR1      = eth_ipgr1_reg::type_id::create("IPGR1");

IPGR2      = eth_ipgr2_reg::type_id::create("IPGR2");

PACKETLEN  = eth_packetlen_reg::type_id::create("PACKETLEN");

COLLCONF   = eth_collconf_reg::type_id::create("COLLCONF");

TX_BD_NUM  = eth_tx_bd_num_reg::type_id::create("TX_BD_NUM");

CTRLMODER  = eth_ctrlmoder_reg::type_id::create("CTRLMODER");

MIIMODER   = eth_miimoder_reg::type_id::create("MIIMODER");

MIICOMMAND = eth_miicommand_reg::type_id::create("MIICOMMAND");

MIIADDRESS = eth_miiaddress_reg::type_id::create("MIIADDRESS");

MIITX_DATA = eth_miitx_data_reg::type_id::create("MIITX_DATA");

MIIRX_DATA = eth_miirx_data_reg::type_id::create("MIIRX_DATA");

MIISTATUS  = eth_miistatus_reg::type_id::create("MIISTATUS");

MAC_ADDR0  = eth_mac_addr0_reg::type_id::create("MAC_ADDR0");

MAC_ADDR1  = eth_mac_addr1_reg::type_id::create("MAC_ADDR1");

HASH0      = eth_hash0_reg::type_id::create("HASH0");

HASH1      = eth_hash1_reg::type_id::create("HASH1");

TXCTRL     = eth_txctrl_reg::type_id::create("TXCTRL");

// -----------------------------------------------------

// Configure + build registers

// -----------------------------------------------------

MODER .configure(this);  MODER.build();

INT_SOURCE .configure(this); INT_SOURCE.build();

INT_MASK .configure(this); INT_MASK.build();

IPGT.configure(this); IPGT.build();

IPGR1 .configure(this); IPGR1.build();

IPGR2.configure(this); IPGR2.build();

PACKETLEN .configure(this); PACKETLEN.build();

COLLCONF.configure(this); COLLCONF.build();

TX_BD_NUM .configure(this); TX_BD_NUM.build();

CTRLMODER .configure(this); CTRLMODER.build();

MIIMODER .configure(this); MIIMODER.build();

MIICOMMAND.configure(this); MIICOMMAND.build();

MIIADDRESS .configure(this); MIIADDRESS.build();

MIITX_DATA .configure(this); MIITX_DATA.build();

MIIRX_DATA .configure(this); MIIRX_DATA.build();

MIISTATUS .configure(this); MIISTATUS.build();

MAC_ADDR0 .configure(this); MAC_ADDR0.build();

MAC_ADDR1 .configure(this); MAC_ADDR1.build();

HASH0 .configure(this); HASH0.build();

HASH1 .configure(this); HASH1.build();

TXCTRL  .configure(this); TXCTRL.build();

// -----------------------------------------------------

// Create register map

// -----------------------------------------------------

reg_map = create_map(

"reg_map",

32'h0000_0000,

4,

UVM_LITTLE_ENDIAN,

1

);

default_map = reg_map;

// -----------------------------------------------------

// Add registers to map

// -----------------------------------------------------

reg_map.add_reg(MODER,      32'h00, "RW");

reg_map.add_reg(INT_SOURCE, 32'h04, "RW");

reg_map.add_reg(INT_MASK,   32'h08, "RW");

reg_map.add_reg(IPGT,       32'h0C, "RW");

reg_map.add_reg(IPGR1,      32'h10, "RW");

reg_map.add_reg(IPGR2,      32'h14, "RW");

reg_map.add_reg(PACKETLEN,  32'h18, "RW");

reg_map.add_reg(COLLCONF,   32'h1C, "RW");

reg_map.add_reg(TX_BD_NUM,  32'h20, "RW");

reg_map.add_reg(CTRLMODER,  32'h24, "RW");

reg_map.add_reg(MIIMODER,   32'h28, "RW");

reg_map.add_reg(MIICOMMAND, 32'h2C, "RW");

reg_map.add_reg(MIIADDRESS, 32'h30, "RW");

reg_map.add_reg(MIITX_DATA, 32'h34, "RW");

reg_map.add_reg(MIIRX_DATA, 32'h38, "RO");

reg_map.add_reg(MIISTATUS,  32'h3C, "RO");

reg_map.add_reg(MAC_ADDR0,   32'h40, "RW");

reg_map.add_reg(MAC_ADDR1,   32'h44, "RW");

reg_map.add_reg(HASH0,       32'h48, "RW");

reg_map.add_reg(HASH1,       32'h4C, "RW");

reg_map.add_reg(TXCTRL,      32'h50, "RW");

// -----------------------------------------------------

// BD RAM

// offset = 0x400 — BD RAM starts here in WISHBONE address space

// Location 0 → WISHBONE addr 0x400 (first TX BD status word)

// Location 1 → WISHBONE addr 0x404 (first TX BD pointer word)

// Location 255 → WISHBONE addr 0x7FC (last BD pointer word)

// ---------------------------------------------------------------------

eth_bd_mem = new(

"eth_bd_mem",

256,              // number of words

32,               // word size

"RW",

UVM_NO_COVERAGE  );

eth_bd_mem.configure(this);

reg_map.add_mem(eth_bd_mem, 32'h400, "RW");

// Lock model

lock_model();

endfunction

// =========================================================

// Convenience functions

// =========================================================

function logic [47:0] get_mac_address();

logic [47:0] mac;

mac[47:40] = MAC_ADDR1.BYTE0.get();

mac[39:32] = MAC_ADDR1.BYTE1.get();

mac[31:24] = MAC_ADDR0.BYTE2.get();

mac[23:16] = MAC_ADDR0.BYTE3.get();

mac[15:8]  = MAC_ADDR0.BYTE4.get();

mac[7:0]   = MAC_ADDR0.BYTE5.get();

return mac;

endfunction

function void set_mac_address_mirror(logic [47:0] mac);

MAC_ADDR1.BYTE0.set(mac[47:40]);

MAC_ADDR1.BYTE1.set(mac[39:32]);

MAC_ADDR0.BYTE2.set(mac[31:24]);

MAC_ADDR0.BYTE3.set(mac[23:16]);

MAC_ADDR0.BYTE4.set(mac[15:8]);

MAC_ADDR0.BYTE5.set(mac[7:0]);

endfunction

function void get_bd_split(output int n_tx_bd, output int n_rx_bd);

n_tx_bd = TX_BD_NUM.TX_BD_NUM.get();

n_rx_bd = 128 - n_tx_bd;

endfunction

function logic [31:0] get_tx_bd_addr(int bd_index);

return 32'h400 + (bd_index * 8);

endfunction

function logic [31:0] get_rx_bd_addr(int bd_index);

int tx_count;

tx_count = TX_BD_NUM.TX_BD_NUM.get();

return 32'h400 + ((tx_count + bd_index) * 8);

endfunction

// -------------------------------------------------------------------------

// Check if MIIM module is busy (poll MIISTATUS.BUSY)

// Returns 1 if busy, 0 if ready

// -------------------------------------------------------------------------

task is_miim_busy(output bit busy);

uvm_status_e status;

uvm_reg_data_t rdata;

MIISTATUS.read(status, rdata);

busy = rdata[1];

endtask

// -------------------------------------------------------------------------

// Wait for MIIM operation to complete

// Polls MIISTATUS.BUSY until cleared

// -------------------------------------------------------------------------

task wait_miim_done(int timeout_cycles = 100000);

uvm_status_e status;

uvm_reg_data_t rdata;

int count = 0;

do begin

MIISTATUS.read(status, rdata);

count++;

if (count >= timeout_cycles)

`uvm_fatal("MIIM_TIMEOUT", "MIIM busy timeout")

end while (rdata[1]);

endtask

// -------------------------------------------------------------------------

// Check if any interrupt is pending in INT_SOURCE

// Returns the raw INT_SOURCE value

// -------------------------------------------------------------------------

task read_int_source(output uvm_reg_data_t  int_src);

uvm_status_e status;

INT_SOURCE.read(status, int_src);

endtask

// -------------------------------------------------------------------------

// Clear specific interrupt bits by writing 1 to them (W1C)

// mask: bitmask of interrupts to clear (1=clear, 0=leave)

// -------------------------------------------------------------------------

task clear_interrupts(uvm_reg_data_t  mask);

uvm_status_e status;

INT_SOURCE.write(status, mask);

// W1C: writing 1 to a bit clears it

// writing 0 to a bit has no effect

endtask

// -------------------------------------------------------------------------

// Clear ALL pending interrupts

// -------------------------------------------------------------------------

task clear_all_interrupts();

clear_interrupts(32'h0000_007F); // all 7 interrupt bits

endtask

endclass : eth_reg_block

4- Ethernet wishbone adapter:

// =============================================================================

// File      : eth_wb_adapter.sv

// Purpose   : UVM Register Adapter — translates between uvm_reg_bus_op

//             (abstract RAL operations) and wb_transaction

//             (concrete WISHBONE bus transactions)

//

// This is the bridge between the RAL model and the physical WISHBONE interface.

// Every reg.read() and reg.write() call flows through this adapter.

//

// reg2bus: converts abstract register operation → wb_transaction

// bus2reg: converts wb_transaction response → abstract register data

// =============================================================================

// =============================================================================

// WISHBONE transaction item

// Used by wb_slave_driver and wb_slave_monitor

// =============================================================================

class wb_transaction extends uvm_sequence_item;

`uvm_object_utils_begin(wb_transaction)

`uvm_field_int(addr,   UVM_ALL_ON)

`uvm_field_int(wdata,  UVM_ALL_ON)

`uvm_field_int(rdata,  UVM_ALL_ON)

`uvm_field_int(we,     UVM_ALL_ON)

`uvm_field_int(sel,    UVM_ALL_ON)

`uvm_field_int(ack,    UVM_ALL_ON)

`uvm_field_int(err,    UVM_ALL_ON)

`uvm_object_utils_end

// -------------------------------------------------------------------------

// WISHBONE signal fields

// -------------------------------------------------------------------------

rand logic [31:0] addr;    // ADDR_I (write) / ADDR_I (read)

rand logic [31:0] wdata;   // DATA_I (write data to DUT)

logic [31:0] rdata;   // DATA_O (read data from DUT)

rand logic        we;      // WE_I  (1=write, 0=read)

rand logic [3:0]  sel;     // SEL_I (byte enables, must be 4'hF)

logic        ack;     // ACK_O (DUT asserts on completion)

logic        err;     // ERR_O (DUT asserts on error)

// -------------------------------------------------------------------------

// Constraints

// -------------------------------------------------------------------------

// SEL must always be 4'hF for this MAC

// Any other value causes ERR_O per the spec

constraint c_sel_full { sel == 4'hF; }

function new(string name = "wb_transaction");

super.new(name);

endfunction

endclass : wb_transaction

// =============================================================================

// WISHBONE Register Adapter

// =============================================================================

class eth_wb_adapter extends uvm_reg_adapter;

`uvm_object_utils(eth_wb_adapter)

function new(string name = "eth_wb_adapter");

super.new(name);

// Tell RAL that this bus supports byte enables

supports_byte_enable = 0;

// Tell RAL that we provide a response item

provides_responses   = 1;

endfunction

// -------------------------------------------------------------------------

// reg2bus()

//

// Called by RAL when it wants to perform a register access.

// Converts the abstract uvm_reg_bus_op into a wb_transaction.

//

// rw.kind    = UVM_READ or UVM_WRITE

// rw.addr    = register offset (e.g. 0x00 for MODER, 0x0C for IPGT)

// rw.data    = data to write (ignored for reads)

// rw.n_bits  = number of bits (32 for this bus)

// -------------------------------------------------------------------------

virtual function uvm_sequence_item reg2bus(const ref uvm_reg_bus_op rw);

wb_transaction tr;

tr = wb_transaction::type_id::create("tr");

// Map abstract operation fields to WISHBONE signal fields

void'(tr.randomize() with {

tr.addr  = rw.addr;                              // register offset

tr.wdata = rw.data;                              // write data (0 for reads)

tr.we    = (rw.kind == UVM_WRITE) ? 1'b1 : 1'b0;

tr.sel   = 4'hF;                                 // always full word

tr.ack   = 0;                                    // will be set by driver

tr.err   = 0;                                    // will be set by driver

});

return tr;

endfunction : reg2bus

// -------------------------------------------------------------------------

// bus2reg()

//

// Called by RAL when the driver has completed a transaction.

// Converts the wb_transaction response back into uvm_reg_bus_op.

//

// After this function:

//   rw.data   = data read from hardware (for reads)

//   rw.status = UVM_IS_OK or UVM_NOT_OK

// -------------------------------------------------------------------------

virtual function void bus2reg(uvm_sequence_item bus_item ,  ref uvm_reg_bus_op rw);

wb_transaction tr;

// Cast the generic item back to wb_transaction

if (!$cast(tr, bus_item)) begin

`uvm_fatal("CAST_FAIL",   "bus2reg: failed to cast bus_item to wb_transaction")

end

// Extract data and status from the completed WISHBONE transaction

rw.data   = tr.rdata;                            // DATA_O from DUT

rw.status = (tr.ack && !tr.err) ? UVM_IS_OK  : UVM_NOT_OK; // ACK without error

// Byte enables — we always use full word

rw.byte_en = 4'hF;

endfunction : bus2reg

endclass : eth_wb_adapter

// =============================================================================

// WISHBONE Slave Driver

// Drives WISHBONE slave interface of the DUT (the configuration/register port)

// =============================================================================

class wb_slave_driver extends uvm_driver #(wb_transaction);

`uvm_component_utils(wb_slave_driver)

// Virtual interface handle — connected in connect_phase

virtual wb_if vif;

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

function void build_phase(uvm_phase phase);

super.build_phase(phase);

if (!uvm_config_db #(virtual wb_if)::get(

this, "", "wb_vif", vif))

`uvm_fatal("NO_VIF",

"wb_slave_driver: no virtual interface found")

endfunction

task run_phase(uvm_phase phase);

wb_transaction tr;

// Initialize bus to idle state

vif.CYC_I  <= 1'b0;

vif.STB_I  <= 1'b0;

vif.WE_I   <= 1'b0;

vif.SEL_I  <= 4'hF;

vif.ADDR_I <= 32'h0;

vif.DATA_I <= 32'h0;

// Wait for reset to deassert

@(negedge vif.RST_I);

repeat(2) @(posedge vif.CLK_I);

forever begin

// Get next transaction from sequencer

seq_item_port.get_next_item(tr);

// Drive it onto the bus

drive_transaction(tr);

// Signal completion to sequencer

seq_item_port.item_done();

end

endtask

// -------------------------------------------------------------------------

// Drive a single WISHBONE transaction

// Handles both reads and writes

// Waits for ACK_O or ERR_O from DUT

// -------------------------------------------------------------------------

task drive_transaction(wb_transaction tr);

// Synchronize to rising clock edge

@(posedge vif.CLK_I);

#1ps;   // small skew to avoid setup violations

// Drive address and control signals

vif.ADDR_I <= tr.addr;

vif.DATA_I <= tr.wdata;

vif.WE_I   <= tr.we;

vif.SEL_I  <= tr.sel;

vif.STB_I  <= 1'b1;

vif.CYC_I  <= 1'b1;

// Wait for DUT to respond

// DUT must assert ACK_O or ERR_O within a reasonable time

begin : wait_response

int timeout = 0;

@(posedge vif.CLK_I);

while (!vif.ACK_O && !vif.ERR_O) begin

timeout++;

if (timeout > 1000) begin

`uvm_error("WB_TIMEOUT",

$sformatf(

"WISHBONE timeout: no ACK/ERR after 1000 cycles.\n" +

"  ADDR=0x%08X WE=%0b",

tr.addr, tr.we))

break;

end

@(posedge vif.CLK_I);

end

end

// Capture response signals

tr.rdata = vif.DATA_O;    // read data (valid when ACK_O=1)

tr.ack   = vif.ACK_O;

tr.err   = vif.ERR_O;

// Deassert bus signals (return to idle)

// CYC must be cleared between transfers per spec

@(posedge vif.CLK_I);

#1ps;

vif.STB_I  <= 1'b0;

vif.CYC_I  <= 1'b0;

vif.WE_I   <= 1'b0;

// Log transaction at debug level

`uvm_info("WB_DRV",

$sformatf("%s: ADDR=0x%08X DATA=0x%08X ACK=%0b ERR=%0b",

tr.we ? "WRITE" : "READ ",

tr.addr,

tr.we ? tr.wdata : tr.rdata,

tr.ack, tr.err),

UVM_HIGH)

endtask : drive_transaction

// -------------------------------------------------------------------------

// Utility: assert RST_I for N cycles then deassert

// -------------------------------------------------------------------------

task assert_reset(int cycles = 5);

vif.RST_I <= 1'b1;

repeat(cycles) @(posedge vif.CLK_I);

vif.RST_I <= 1'b0;

repeat(2) @(posedge vif.CLK_I);

`uvm_info("WB_DRV",

$sformatf("Reset asserted for %0d cycles then deasserted",

cycles),

UVM_MEDIUM)

endtask

endclass : wb_slave_driver

// =============================================================================

// WISHBONE Slave Monitor

// Observes all transactions on the WISHBONE slave interface

// Sends observed transactions to the scoreboard via analysis port

// =============================================================================

class wb_slave_monitor extends uvm_monitor;

`uvm_component_utils(wb_slave_monitor)

virtual wb_if vif;

// Analysis port: sends observed transactions to scoreboard/coverage

uvm_analysis_port #(wb_transaction) ap;

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

function void build_phase(uvm_phase phase);

super.build_phase(phase);

ap = new("ap", this);

if (!uvm_config_db #(virtual wb_if)::get(

this, "", "wb_vif", vif))

`uvm_fatal("NO_VIF",

"wb_slave_monitor: no virtual interface found")

endfunction

task run_phase(uvm_phase phase);

wb_transaction tr;

forever begin

// Wait for start of a WISHBONE transfer

// Transfer starts when both CYC_I and STB_I are asserted

@(posedge vif.CLK_I);

if (vif.CYC_I && vif.STB_I) begin

tr = wb_transaction::type_id::create("tr");

// Capture the transaction inputs

tr.addr  = vif.ADDR_I;

tr.wdata = vif.DATA_I;

tr.we    = vif.WE_I;

tr.sel   = vif.SEL_I;

// Wait for response

while (!vif.ACK_O && !vif.ERR_O) begin

@(posedge vif.CLK_I);

end

// Capture response

tr.rdata = vif.DATA_O;

tr.ack   = vif.ACK_O;

tr.err   = vif.ERR_O;

// Broadcast to scoreboard and coverage collectors

ap.write(tr);

end

end

endtask

endclass : wb_slave_monitor

// =============================================================================

// WISHBONE Slave Agent

// Packages driver, monitor, and sequencer together

// =============================================================================

class wb_slave_agent extends uvm_agent;

`uvm_component_utils(wb_slave_agent)

wb_slave_driver    driver;

wb_slave_monitor   monitor;

uvm_sequencer      #(wb_transaction) sequencer;

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

function void build_phase(uvm_phase phase);

super.build_phase(phase);

driver    = wb_slave_driver::type_id::create("driver", this);

monitor   = wb_slave_monitor::type_id::create("monitor", this);

sequencer = uvm_sequencer #(wb_transaction)::type_id::create(

"sequencer", this);

endfunction

function void connect_phase(uvm_phase phase);

// Connect driver to sequencer

driver.seq_item_port.connect(sequencer.seq_item_export);

endfunction

endclass : wb_slave_agent

// =============================================================================

// File      : eth_env.sv

// Purpose   : UVM Environment — wires the register block, adapter,

//             and predictor together inside the verification environment

// =============================================================================

class eth_env extends uvm_env;

`uvm_component_utils(eth_env)

// -------------------------------------------------------------------------

// Component handles

// -------------------------------------------------------------------------

wb_slave_agent      wb_slave_agt;   // drives WISHBONE slave IF of DUT

eth_reg_block       reg_block;      // RAL model of all 21 registers + BD RAM

eth_wb_adapter      wb_adapter;     // translates RAL ops ↔ wb_transactions

uvm_reg_predictor   #(wb_transaction) predictor; // auto-updates RAL mirrors

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

// -------------------------------------------------------------------------

// build_phase: create all components

// -------------------------------------------------------------------------

function void build_phase(uvm_phase phase);

super.build_phase(phase);

// Create WISHBONE slave agent

wb_slave_agt = wb_slave_agent::type_id::create("wb_slave_agt", this);

// Create and build the register block

// This calls build() internally which creates all 21 registers

// and the BD memory and the register map

reg_block = eth_reg_block::type_id::create("reg_block");

reg_block.build();

// Create the adapter

wb_adapter = eth_wb_adapter::type_id::create("wb_adapter", this);

// Create the register predictor

// The predictor listens to the monitor's analysis port and

// automatically updates the RAL mirror values when it sees

// bus transactions complete — without needing explicit predict() calls

predictor = uvm_reg_predictor #(wb_transaction)::type_id::create(

"predictor", this);

endfunction : build_phase

// -------------------------------------------------------------------------

// connect_phase: wire everything together

// -------------------------------------------------------------------------

function void connect_phase(uvm_phase phase);

super.connect_phase(phase);

// ── Connect RAL map to the WISHBONE sequencer via adapter ──────────

// This tells the RAL:

//   "when you need to do a bus transaction,

//    use wb_adapter to convert it,

//    and send it to wb_slave_agt.sequencer"

reg_block.reg_map.set_sequencer(

wb_slave_agt.sequencer,

wb_adapter

);

// ── Connect monitor analysis port → predictor ──────────────────────

// Every time the monitor sees a completed WISHBONE transaction,

// it sends it to the predictor.

// The predictor uses the adapter to decode the transaction

// and updates the RAL mirror register values automatically.

predictor.bus_in.connect(wb_slave_agt.monitor.ap);

// Tell the predictor which adapter and map to use

predictor.adapter = wb_adapter;

predictor.map     = reg_block.reg_map;

endfunction : connect_phase

endclass : eth_env

// =============================================================================

// File      : eth_base_test.sv

// Purpose   : Base test class — all test cases extend this

//             Handles environment creation and common configuration

// =============================================================================

class eth_base_test extends uvm_test;

`uvm_component_utils(eth_base_test)

eth_env env;

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

function void build_phase(uvm_phase phase);

super.build_phase(phase);

// Create the environment

env = eth_env::type_id::create("env", this);

endfunction

// -------------------------------------------------------------------------

// end_of_elaboration_phase: print topology for debug

// -------------------------------------------------------------------------

function void end_of_elaboration_phase(uvm_phase phase);

uvm_top.print_topology();

endfunction

endclass : eth_base_test

// =============================================================================

// File      : tc_reg_reset_values.sv

// Purpose   : Test — verify all register reset values after RST_I

// =============================================================================

class tc_reg_reset_values extends eth_base_test;

`uvm_component_utils(tc_reg_reset_values)

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

task run_phase(uvm_phase phase);

uvm_reg_hw_reset_seq rst_seq;

phase.raise_objection(this);

`uvm_info("TEST", "Starting tc_reg_reset_values", UVM_LOW)

// ── Step 1: Assert hard reset ──────────────────────────────────────

`uvm_info("TEST", "Asserting RST_I for 10 cycles", UVM_MEDIUM)

env.wb_slave_agt.driver.assert_reset(10);

// ── Step 2: Wait for DUT to settle after reset ─────────────────────

#200ns;

// ── Step 3: Create and run the hardware reset check sequence ────────

// This reads every register via WISHBONE frontdoor and

// compares each value against the reset value declared in the RAL

rst_seq = uvm_reg_hw_reset_seq::type_id::create("rst_seq");

rst_seq.model = env.reg_block;   // tell it which block to check

rst_seq.start(env.wb_slave_agt.sequencer);

`uvm_info("TEST", "tc_reg_reset_values complete", UVM_LOW)

phase.drop_objection(this);

endtask

endclass : tc_reg_reset_values

// =============================================================================

// File      : tc_reg_walking_one.sv

// Purpose   : Test — walking-1 and walking-0 across all RW bits

// =============================================================================

class tc_reg_walking_one extends eth_base_test;

`uvm_component_utils(tc_reg_walking_one)

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

task run_phase(uvm_phase phase);

uvm_reg_bit_bash_seq bash_seq;

uvm_status_e         status;

phase.raise_objection(this);

`uvm_info("TEST", "Starting tc_reg_walking_one", UVM_LOW)

// ── Step 1: Reset first ────────────────────────────────────────────

env.wb_slave_agt.driver.assert_reset(10);

#200ns;

// ── Step 2: Disable TX and RX before touching registers ────────────

// Prevents the DUT from trying to use registers we are about to bash

env.reg_block.MODER.TXEN.write(status, 1'b0);

env.reg_block.MODER.RXEN.write(status, 1'b0);

// ── Step 3: Run bit bash sequence ──────────────────────────────────

// This automatically:

//   - walks 1 through every RW bit in every register

//   - walks 0 through every RW bit in every register

//   - skips reserved and read-only bits

//   - restores each register to reset value after each bit test

bash_seq = uvm_reg_bit_bash_seq::type_id::create("bash_seq");

bash_seq.model = env.reg_block;

bash_seq.start(env.wb_slave_agt.sequencer);

`uvm_info("TEST", "tc_reg_walking_one complete", UVM_LOW)

phase.drop_objection(this);

endtask

endclass : tc_reg_walking_one

// =============================================================================

// File      : tc_bd_walking_one.sv

// Purpose   : Test — walking patterns across BD RAM + reset preservation

// =============================================================================

class tc_bd_walking_one extends eth_base_test;

`uvm_component_utils(tc_bd_walking_one)

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

task run_phase(uvm_phase phase);

uvm_mem_walk_seq   mem_seq;

uvm_status_e       status;

uvm_reg_data_t     rdata;

phase.raise_objection(this);

`uvm_info("TEST", "Starting tc_bd_walking_one", UVM_LOW)

// ── Step 1: Reset and settle ───────────────────────────────────────

env.wb_slave_agt.driver.assert_reset(10);

#200ns;

// ── Step 2: Ensure MAC is not in reset and not running ─────────────

// BD RAM only accessible when MAC not in reset

env.reg_block.MODER.write(status, 32'h0000_A000);

// Reset value: TXEN=0, RXEN=0, CRCEN=1, NOBCKOF=1

// TX and RX disabled so no DUT-internal BD access conflicts

// ── Step 3: Write known pattern before reset (for preservation test)

`uvm_info("TEST", "Writing preservation pattern to BD RAM", UVM_MEDIUM)

for (int i = 0; i < 256; i++) begin

env.reg_block.eth_bd_mem.write(

status,

i,                           // offset

32'hA5A5_0000 | i,           // unique value per location

UVM_FRONTDOOR,

env.reg_block.reg_map

);

end

// ── Step 4: Assert and deassert reset ─────────────────────────────

`uvm_info("TEST", "Asserting reset to check BD RAM preservation",

UVM_MEDIUM)

env.wb_slave_agt.driver.assert_reset(10);

#200ns;

// Re-enable access after reset

env.reg_block.MODER.write(status, 32'h0000_A000);

// ── Step 5: Verify BD RAM preserved through reset ──────────────────

`uvm_info("TEST", "Verifying BD RAM preserved after reset", UVM_MEDIUM)

for (int i = 0; i < 256; i++) begin

env.reg_block.eth_bd_mem.read(

status,

i,

rdata,

UVM_FRONTDOOR,

env.reg_block.reg_map

);

if (rdata !== (32'hA5A5_0000 | i)) begin

`uvm_error("BD_PRESERVE",

$sformatf(

"BD RAM offset %0d corrupted by reset: " +

"expected 0x%08X got 0x%08X",

i, 32'hA5A5_0000 | i, rdata))

end

end

// ── Step 6: Run the full memory walk sequence ──────────────────────

// This runs walking-1, walking-0, and checkerboard patterns

// across all 256 BD RAM locations

`uvm_info("TEST", "Running uvm_mem_walk_seq on BD RAM", UVM_MEDIUM)

mem_seq = uvm_mem_walk_seq::type_id::create("mem_seq");

mem_seq.model = env.reg_block;

mem_seq.start(env.wb_slave_agt.sequencer);

`uvm_info("TEST", "tc_bd_walking_one complete", UVM_LOW)

phase.drop_objection(this);

endtask

endclass : tc_bd_walking_one

// =============================================================================

// File      : tc_reg_max_values.sv

// Purpose   : Test — write max values, read back, hard reset, verify

// =============================================================================

class tc_reg_max_values extends eth_base_test;

`uvm_component_utils(tc_reg_max_values)

function new(string name, uvm_component parent);

super.new(name, parent);

endfunction

task run_phase(uvm_phase phase);

uvm_reg          regs[$];

uvm_reg_data_t   max_val;

uvm_reg_data_t   rdata;

uvm_reg_data_t   inv_val;

uvm_status_e     status;

phase.raise_objection(this);

`uvm_info("TEST", "Starting tc_reg_max_values", UVM_LOW)

// ── Phase 1: Reset first ───────────────────────────────────────────

env.wb_slave_agt.driver.assert_reset(10);

#200ns;

// Collect all registers from the block

env.reg_block.get_registers(regs);

// ── Phase 2: Write maximum RW value to every register ─────────────

foreach (regs[i]) begin

// Skip read-only registers

if (regs[i].get_access() == "RO") continue;

// Get the mask of all RW-accessible bits

// This gives us the maximum writable value

max_val = get_rw_mask(regs[i]);

`uvm_info("TEST",

$sformatf("Writing max to %-15s = 0x%08X",

regs[i].get_name(), max_val),

UVM_HIGH)

regs[i].write(status, max_val);

if (status != UVM_IS_OK)

`uvm_error("MAX_WRITE_FAIL",

$sformatf("Failed to write max value to %s",

regs[i].get_name()))

end

// ── Phase 3: Read back and verify ─────────────────────────────────

foreach (regs[i]) begin

if (regs[i].get_access() == "RO") continue;

max_val = get_rw_mask(regs[i]);

regs[i].read(status, rdata);

// For W1C registers (INT_SOURCE), writing 1 clears bits

// so expected readback is 0, not max_val

if (has_w1c_field(regs[i])) begin

if (rdata !== 32'h0) begin

`uvm_error("MAX_READBACK",

$sformatf(

"%s (W1C): expected 0x00000000 after write, got 0x%08X",

regs[i].get_name(), rdata))

end

end

else begin

if ((rdata & max_val) !== max_val) begin

`uvm_error("MAX_READBACK",

$sformatf(

"%s: expected 0x%08X, got 0x%08X (diff=0x%08X)",

regs[i].get_name(),

max_val, rdata,

max_val ^ rdata))

end

end

end

// ── Phase 4: Write inverse of reset values ─────────────────────────

foreach (regs[i]) begin

if (regs[i].get_access() == "RO") continue;

inv_val = ~regs[i].get_reset() & get_rw_mask(regs[i]);

regs[i].write(status, inv_val);

end

// ── Phase 5: Hard reset ────────────────────────────────────────────

`uvm_info("TEST",

"Hard reset — all registers should return to reset values",

UVM_MEDIUM)

env.wb_slave_agt.driver.assert_reset(10);

#200ns;

// ── Phase 6: Verify reset values ──────────────────────────────────

foreach (regs[i]) begin

uvm_reg_data_t exp_reset;

exp_reset = regs[i].get_reset();

regs[i].read(status, rdata);

if (rdata !== exp_reset) begin

`uvm_error("RESET_VERIFY",

$sformatf(

"%s: expected reset=0x%08X, got=0x%08X after RST_I",

regs[i].get_name(), exp_reset, rdata))

end

end

`uvm_info("TEST", "tc_reg_max_values complete", UVM_LOW)

phase.drop_objection(this);

endtask

// -------------------------------------------------------------------------

// Helper: get the bitmask of all RW-accessible bits in a register

// Reserved and RO bits are masked out

// -------------------------------------------------------------------------

function uvm_reg_data_t get_rw_mask(uvm_reg rg);

uvm_reg_field fields[$];

uvm_reg_data_t mask = 0;

rg.get_fields(fields);

foreach (fields[i]) begin

if (fields[i].get_access() == "RW") begin

// Set all bits in this field's position

for (int b = 0; b < fields[i].get_n_bits(); b++) begin

mask[fields[i].get_lsb_pos() + b] = 1'b1;

end

end

end

return mask;

endfunction

// -------------------------------------------------------------------------

// Helper: check if a register has any W1C fields

// -------------------------------------------------------------------------

function bit has_w1c_field(uvm_reg rg);

uvm_reg_field fields[$];

rg.get_fields(fields);

foreach (fields[i]) begin

if (fields[i].get_access() == "W1C")

return 1;

end

return 0;

endfunction

endclass : tc_reg_max_values

// =============================================================================

// File      : eth_ral_pkg.sv

// Purpose   : Package that includes all RAL files in the correct

//             compilation order.

//

// Compilation order matters in SystemVerilog:

//   1. Base classes must come before derived classes

//   2. Register classes must come before the block

//   3. The block must come before the adapter

//   4. The adapter must come before the environment

//   5. The environment must come before the tests

//

// Usage in your simulator:

//   vlog eth_ral_pkg.sv

//   vlog tb_top.sv

//

// Or in a filelist:

//   +incdir+./eth_ral

//   ./eth_ral/eth_ral_pkg.sv

// =============================================================================

package eth_ral_pkg;

// Import UVM base package first

import uvm_pkg::*;

`include "uvm_macros.svh"

// -------------------------------------------------------------------------

// Step 1: Individual register class files

//         Each file defines one register class (extends uvm_reg)

// -------------------------------------------------------------------------

// Core operation mode register

`include "eth_moder_reg.sv"

// Interrupt registers (source W1C, mask RW)

`include "eth_int_regs.sv"

// Inter-packet gap and packet length registers

`include "eth_ipg_pkt_regs.sv"

// TX BD number and control mode register

`include "eth_tx_bd_ctrl_regs.sv"

// All six MIIM registers

`include "eth_miim_regs.sv"

// MAC address, hash table, and TX control

`include "eth_mac_hash_txctrl_regs.sv"

// -------------------------------------------------------------------------

// Step 2: Register block

//         Instantiates all register objects, creates the memory,

//         and builds the WISHBONE address map

// -------------------------------------------------------------------------

`include "eth_reg_block.sv"

// -------------------------------------------------------------------------

// Step 3: WISHBONE adapter, driver, monitor, and agent

//         These bridge the RAL to the physical bus

// -------------------------------------------------------------------------

`include "eth_wb_adapter.sv"

// -------------------------------------------------------------------------

// Step 4: Environment and test classes

//         Wires everything together and provides test entry points

// -------------------------------------------------------------------------

`include "eth_env_and_tests.sv"

endpackage : eth_ral_pkg