# Ethernet MAC Core Tests

## GROUP A: BD ADDRESS MANAGEMENT CORNER CASES

### tc_tx_bd_wrap_on_error

Gap: Existing plan tests WR bit for normal completion (tc_tx_wrap_bit) but not after an error aborts transmission.

When a Tx frame is aborted due to error (underrun, retry limit, late collision), the RTL writes status to the current BD and advances TxBDAddress via TempTxBDAddress. The WR bit still controls whether the address wraps or increments — but the next BD the core reads after an error must be the one immediately following the aborted BD, not a re-read of the same one.

- Setup: Two TX BDs. BD0 has WR=0. BD1 has WR=1.

- Fault: Force underrun on BD0 transmission (starve WISHBONE master)

- Expected: UR=1 written to BD0. Core advances to BD1. BD1 transmitted correctly. BD1 WR=1 wraps back to BD0.

- Verify: TxBDAddress advances after abort, does not re-read BD0, WR=1 correctly wraps back, no BD skipped

### tc_tx_retry_bd_address_hold

Gap: No test verifies that BD address is NOT advanced during collision retry. RTL uses TxRetryPacket_NotCleared to re-read the same BD, not TxStatusWrite which would advance. This is a critical distinction.

- Setup: Single TX BD with RD=1

- Fault: Inject 3 collisions then let frame succeed

- Expected: BD address stays at same value throughout all 3 retries. After successful transmission, address advances. RTRY=3 in BD status.

- Verify: TxBDAddress unchanged during backoff and retry cycles. Address only increments after TxStatusWrite (success or final abort). Same BD re-read on each retry via TxRetryPacket_NotCleared path.

### tc_rx_bd_address_init_on_rxen

Gap: No test verifies RxBDAddress is initialized to r_TxBDNum<<1 when RXEN is first enabled, not to zero. If TX_BD_NUM is changed before enabling RX, the RX BD base address must reflect the new split.

- Setup: Write TX_BD_NUM=0x20 (32 TX BDs). Then enable RXEN.

- Expected: First RX BD address = 0x20 × 8 = 0x100 offset from BD RAM base = 0x500. RX BDs start at 0x500, not 0x400.

- Verify: First received frame written to BD at 0x500. BDs at 0x400-0x4FC (TX region) not used for RX.

### tc_tx_bd_address_init_on_txen

Gap: No test verifies TxBDAddress resets to 0 specifically when TXEN is toggled (not just on RST_I). If TXEN is cleared and re-asserted, TxBDAddress must restart from BD0, not continue from where it left off.

- Setup: Transmit 3 frames. Disable TXEN. Re-enable TXEN.

- Expected: After TXEN re-enable, core re-reads BD0 (TxBDAddress=0), not BD3.

- Verify: First transmission after TXEN toggle uses BD at offset 0x400, not wherever the pointer was before disable.

### tc_tx_bd_num_change_mid_operation

Gap: TX_BD_NUM register controls the TX/RX BD split. No test verifies what happens if TX_BD_NUM is changed while TXEN and RXEN are both active (which the spec warns against but behavior is undefined).

- Setup: TX_BD_NUM=0x40 (default). Active TX and RX in progress.

- Fault: Write TX_BD_NUM=0x10 while both TX and RX enabled

- Expected: Per spec, registers should not change after TXEN/RXEN set. This is a corner case to document actual behavior.

- Verify: Does TX continue using old BD split? Does RX base address change immediately? Is data corrupted? Does system hang?

### tc_rx_bd_wrap_boundary_at_zero_tx

Gap: When TX_BD_NUM=0x00 (all BDs are RX), the RX BD base address is 0 (r_TxBDNum[6:0] = 0). The last RX BD has WR=1 and wraps back to address 0. No test verifies this specific wrap case where TX region is zero.

- Setup: TX_BD_NUM=0x00. All 128 BDs as RX. Mark last BD (0x7FC) with WR=1.

- Expected: After last BD used, wrap back to BD at 0x400 (address 0 in BD RAM = r_TxBDNum<<1 = 0)

- Verify: RX BD pointer wraps to 0x400 correctly with zero TX BDs

## GROUP B: PAUSE TIMER CORNER CASES

### tc_fc_pause_timer_zero_value

Gap: No test sends a PAUSE frame with timer value = 0. This is a valid PAUSE frame but should cause immediate unpausing (or never pausing).

- Setup: RXFLOW=1, TXFLOW=0. Active TX in progress.

- Inject: Valid PAUSE frame with PauseTimerValue = 0x0000

- Expected: SetPauseTimer fires but PauseTimer loaded with 0. PauseTimerEq0 immediately true. Pause signal never asserts (or asserts for zero cycles). TX never paused.

- Verify: TX transmission not interrupted. RXC interrupt still set (frame was valid PAUSE). Timer value = 0 does not crash.

### tc_fc_pause_timer_max_value

Gap: No test sends PAUSE with maximum timer value 0xFFFF (65535 slot times). At 100Mbps one slot = 512 bit times = 5.12µs. 65535 slots ≈ 335ms of pausing.

- Setup: RXFLOW=1. TX actively transmitting back-to-back frames.

- Inject: PAUSE frame with PauseTimerValue = 0xFFFF

- Expected: TX paused for 65535 slot times. SlotTimer counts to 64 (512 bits at 8 bits/clock), generates pulse to PauseTimer. PauseTimer decrements 65535 times. TX resumes only after timer exhausted.

- Verify: TX paused for correct duration. Subsequent TX frames all deferred. Timer decrement visible in each slot. TX resumes exactly at PauseTimerEq0.

### tc_fc_pause_overwrite_during_pause

Gap: No test sends a second PAUSE frame while the first PAUSE timer is still counting. RTL latches new timer value via SetPauseTimer which fires on ReceiveEnd & RxFlow. The new value overwrites the old one.

- Setup: RXFLOW=1. Send PAUSE with value=0x00FF (255 slots). While timer counting, send second PAUSE with value=0x8000 (32768 slots).

- Expected: PauseTimer reloaded with 0x8000 mid-count. TX remains paused for 32768 slots from the second PAUSE, not 255.

- Verify: Timer value changes mid-count. TX pause duration extends. No spurious unpausing between the two PAUSE frames.

### tc_fc_pause_timer_rxflow_cleared_mid_pause

Gap: No test clears RXFLOW while PAUSE timer is counting. RTL: Pause = RxFlow & ~PauseTimerEq0_sync2. If RxFlow cleared, Pause immediately deasserts.

- Setup: RXFLOW=1. Send PAUSE frame with value=0x00FF. While timer counting with ~100 slots remaining, clear RXFLOW bit.

- Expected: Pause signal deasserts immediately when RXFLOW cleared (Pause = 0 & ~PauseTimerEq0 = 0). TX resumes despite timer not reaching zero. Timer continues counting in background but Pause stays deasserted.

- Verify: TX resumes immediately on RXFLOW clear. Timer keeps decrementing. If RXFLOW re-asserted before timer expires, Pause re-asserts.

### tc_fc_pause_during_own_pause_transmission

Gap: No test verifies what happens when the core is sending a PAUSE frame (TPauseRq) while simultaneously receiving a PAUSE frame that would pause it. This creates a logical conflict — should the outgoing PAUSE be blocked?

- Setup: TXFLOW=1, RXFLOW=1. Write TXPAUSERQ=1 to start outgoing PAUSE. Simultaneously inject incoming PAUSE frame from PHY.

- Expected: Outgoing PAUSE frame completes transmission (CtrlMux active). Incoming PAUSE sets timer. TX paused after outgoing PAUSE frame finishes.

- Verify: Outgoing PAUSE not truncated by incoming Pause timer. TXC interrupt set (outgoing). RXC interrupt set (incoming). Both frames handled independently.

### tc_fc_pause_timer_slot_divider

Gap: SlotTimer uses Divider2 (a clock divider signal). The slot timer counts 64 increments (6-bit counter, all ones = SlotFinished). No test verifies slot timing accuracy at 10 Mbps vs 100 Mbps.

- Setup at 10 Mbps (2.5MHz MRxClk): Send PAUSE with timer=1. Measure cycles until TX resumes.

- Setup at 100 Mbps (25MHz MRxClk): Same PAUSE timer=1. Measure cycles.

- Expected: Slot time = 512 bit times = 64 bytes × 8 bits. At 100Mbps: 512 / 100M = 5.12µs. At 10Mbps: 512 / 10M = 51.2µs. Ratio = 10:1.

- Verify: Pause duration differs by exactly 10x between speeds. SlotFinished pulse interval matches specification.

## GROUP C: INTER-PACKET GAP CORNER CASES

### tc_tx_ipgr1_equals_ipgr2

Gap: Spec says IPGR1 must be within [0, IPGR2]. No test verifies degenerate case where IPGR1 = IPGR2. RTL StartDefer condition: NibCnt <= IPGR1 AND NibCnt != IPGR2. When IPGR1=IPGR2, the NibCnt != IPGR2 prevents deferral exactly at the boundary.

- Setup: IPGR1=18, IPGR2=18. Rule1=0 (non back-to-back). Inject MCrS at NibCnt=18.

- Expected: StartDefer condition: NibCnt<=18 (true) AND NibCnt!=18 (false) → StartDefer = 0. StartIdle condition: NibCnt>=18 (true) → StartIdle = 1. Core proceeds to idle, not defer.

- Verify: No infinite deferral. Core transitions to Idle at NibCnt=18 despite carrier sense present. The NibCnt != IPGR2 guard prevents lockout.

### tc_tx_ipgr2_less_than_ipgr1

Gap: No test verifies behavior when IPGR2 < IPGR1 (invalid configuration per spec). RTL has no hardware protection against this. Both StartDefer (NibCnt<=IPGR1) and StartIdle (NibCnt>=IPGR2) may be simultaneously true.

- Setup: IPGR1=15, IPGR2=12. Rule1=0. No carrier sense.

- Expected: At NibCnt=12: StartIdle true (12>=12). At NibCnt=13: StartDefer condition becomes evaluable (13<=15 but if StartIdle already fired, StateIPG already exited). Priority of StartIdle vs StartDefer determines outcome.

- Verify: Observe which transition fires first. Document whether StartIdle or StartDefer takes priority at same NibCnt value. Check RTL for if(StartDefer | StartIdle) StateIPG <= 0 — both clear StateIPG but different next states.

### tc_tx_ipgt_zero

Gap: No test sets IPGT=0. RTL: StartIdle = StateIPG & Rule1 & NibCnt[6:0] >= IPGT. When IPGT=0, NibCnt>=0 is always true from the first clock in StateIPG. Core would transition to Idle in the first cycle of IPG.

- Setup: IPGT=0. FullD=1 (or previous frame just sent to set Rule1=1). Transmit two back-to-back frames.

- Expected: Zero gap between frames. MTxEn never deasserts between frame N FCS and frame N+1 preamble. Violates Ethernet standard (0.96µs minimum) but core should not hang.

- Verify: Back-to-back frames with zero gap. No state machine stall. Frame content correct.

### tc_tx_carrier_during_ipg_both_windows

Gap: tc_hd_carrier_sense_ifg tests carrier within IPGR1 but does not test the specific boundary between IPGR1 and IPGR2 windows. The RTL uses NibCnt[6:0] <= IPGR1 which includes the boundary.

- Scenario A: Inject MCrS at NibCnt = IPGR1 exactly. Expected: StartDefer fires (NibCnt <= IPGR1 is true at boundary). Counter resets.

- Scenario B: Inject MCrS at NibCnt = IPGR1+1. Expected: StartDefer does NOT fire. Counter continues to IPGR2.

- Verify: The <= vs < boundary determines defer behavior. Both scenarios must produce different results to confirm RTL uses <= not <.

## GROUP D: INVALID SYMBOL AND MRXERR CORNER CASES

### tc_rx_invalid_symbol_at_sfd

Gap: tc_rx_invalid_symbol only tests invalid symbol during data payload. No test injects MRxErr with MRxD=0xE during the SFD byte specifically (RxStateSFD active).

- Setup: Normal preamble, then during SFD reception inject MRxErr=1 with MRxD=4'hE

- Expected: SetInvalidSymbol fires during SFD. InvalidSymbol flag set. Frame continues (no abort). But SFD byte corrupted — question: does core still accept frame start or does it fail SFD recognition?

- Verify: InvalidSymbol set, IS=1 in Rx BD. Whether frame was started (RxStartFrm asserted) or SFD recognition missed. Document behavior.

### tc_rx_invalid_symbol_at_preamble

Gap: No test injects MRxErr with MRxD=0xE during preamble state. During preamble, RxStatePreamble is active, not RxStateSFD or RxStateData. The LatchedMRxErr condition checks RxStatePreamble | RxStateSFD | (|RxStateData) | RxStateIdle & ~Transmitting.

- Setup: Inject MRxErr=1 with MRxD=4'hE during preamble (before SFD)

- Expected: LatchedMRxErr fires (condition covers RxStatePreamble). SetInvalidSymbol also fires (MRxDV=1 & MRxErr=1 & MRxD==0xE). InvalidSymbol=1. LatchedMRxErr & ~InvalidSymbol = 0. No abort.

- Verify: Preamble error with 0xE code does not abort frame. IS=1 in BD. Compare: preamble error with non-0xE code should abort.

### tc_rx_mrxerr_then_invalid_symbol_same_frame

Gap: No test injects two different MRxErr types in the same frame. From RTL analysis: if MRxErr with MRxD≠0xE fires first (LatchedMRxErr=1, InvalidSymbol=0), RxAbort fires immediately. But what if invalid symbol (0xE) fires first, then unknown error fires?

- Setup: Inject MRxErr with MRxD=0xE at byte 10. Then inject MRxErr with MRxD=0xA at byte 20.

- Expected: First error: InvalidSymbol=1, frame continues. Second error: LatchedMRxErr=1, InvalidSymbol=1 (already set), LatchedMRxErr & ~InvalidSymbol = 0. Second error does NOT abort because InvalidSymbol was already set.

- Verify: Frame completes despite second error. IS=1 in BD. This reveals the InvalidSymbol flag protects against abort even for subsequent unknown errors within same frame.

### tc_rx_mrxerr_outside_frame

Gap: tc_rx_phy_error tests MRxErr during active frame. No test injects MRxErr when MRxDV=0 (outside frame). RTL LatchedMRxErr requires MRxDV=1, so inter-frame errors should be silently ignored.

- Setup: Between two valid frames, deassert MRxDV, assert MRxErr for several cycles.

- Expected: LatchedMRxErr stays 0 (MRxDV=0 prevents latching). No RxAbort. Both surrounding frames received without error. IS=0 in both BDs.

- Verify: Inter-frame PHY errors do not corrupt next frame reception. No spurious BD status bits set.

## GROUP E: DELAYED CRC CORNER CASES

### tc_rx_dlycrcen_with_short_frame

Gap: DLYCRCEN shifts CRC calculation 4 bytes after SFD. On receive side this means the first 4 bytes of payload are NOT included in CRC checking. No test combines DLYCRCEN with short frames near the minimum.

- Setup: DLYCRCEN=1, RECSMALL=1. Send frame where payload = 5 bytes total.

- Expected: CRC calculation starts 4 bytes after SFD. Only 1 byte of payload included in CRC. The 4 delayed bytes treated as tag or proprietary header. Frame may be accepted with unusual CRC behavior.

- Verify: DlyCrcCnt counts to 9 (from RTL: counts 1 to 9 cycling). CRC computed over different window. Document whether frame passes or fails CRC with DLYCRCEN.

### tc_tx_dlycrcen_crc_window

Gap: tc_tx_delayed_crc only verifies that DLYCRCEN=1 starts CRC 4 bytes after SFD. No test verifies the exact 4-byte window by corrupting bytes 1-4 vs bytes 5+ and observing CRC impact.

- Setup: DLYCRCEN=1. Transmit frame.

- Scenario A: Corrupt bytes 1-4 of payload on the wire (within the delayed window). Expected: CRC not affected (these bytes excluded from CRC calculation). Receiver may or may not detect error depending on FCS.

- Scenario B: Corrupt byte 5 onwards. Expected: CRC detects error. Receiver CRC error.

- Verify: DLYCRCEN window boundaries are exactly 4 bytes. DlyCrcCnt behavior in RTL (counts 1-9, Initialize_Crc active during early count).

## GROUP F: CONTROL FRAME CORNER CASES

### tc_fc_pause_frame_received_while_sending_pause

Gap: No test handles the race condition where the core is actively transmitting its own PAUSE frame (CtrlMux=1) when an incoming PAUSE frame arrives from the PHY.

- Setup: TXFLOW=1, RXFLOW=1. Assert TPauseRq. While CtrlMux=1 (outgoing PAUSE transmitting), inject incoming PAUSE from PHY.

- Expected: Incoming PAUSE detected during outgoing PAUSE transmission. ReceivedPauseFrm set. SetPauseTimer fires. PauseTimer loaded. After outgoing PAUSE completes, TX is immediately paused by received timer value.

- Verify: Outgoing PAUSE frame not corrupted. Incoming PAUSE correctly processed despite CtrlMux=1. Both TXC and RXC interrupts set.

### tc_fc_tpauserq_during_pause

Gap: TPauseRq is latched internally (WillSendControlFrame). No test asserts TPauseRq while TX is already paused by a received PAUSE frame (Pause=1).

- Setup: RXFLOW=1, TXFLOW=1. Receive PAUSE frame, TX paused. While paused, write TXPAUSERQ=1 to TXCTRL.

- Expected: WillSendControlFrame set (TPauseRq & TxFlow). But TxStartFrmOut = ~Pause for data frames. For PAUSE transmission: CtrlMux waits for WillSendControlFrame & ~TxUsedDataOut. Can the core send outgoing PAUSE while its TX is paused by incoming PAUSE?

- Verify: Whether PAUSE-to-PAUSE scenario is handled correctly. Outgoing PAUSE may be deferred until incoming pause timer expires, or may take priority. Document RTL behavior.

### tc_fc_control_frame_with_promiscuous

Gap: No test combines promiscuous mode (PRO=1) with an incoming PAUSE frame that has an invalid destination address. Discussed theoretically earlier but not in existing test plan.

- Setup: PRO=1, RXFLOW=1, PASSALL=0. Send PAUSE frame with Dst=00:11:22:33:44:55 (not multicast, not own MAC). EtherType=0x8808, Opcode=0x0001.

- Expected: Address recognition: PRO=1 accepts frame (M=1). Control frame detection: address fails (not 01:80:c2:.. and not own MAC). PAUSE NOT processed. Timer NOT updated. RXC NOT set.

- Verify: Frame accepted by address recognition (no RxAbort). But control frame path ignores it. Frame stored or not depends on PASSALL. CF=0 in Rx BD (not a valid control frame).

### tc_fc_pause_address_boundary

Gap: No test verifies that ONLY the two valid PAUSE destinations are accepted by the control frame detector, and that addresses adjacent to the reserved multicast (e.g., 01:80:c2:00:00:00 or 01:80:c2:00:00:02) are rejected.

- Scenario A: Dst = 01:80:c2:00:00:00 (one below reserved multicast). Expected: NOT a valid PAUSE destination. Timer not updated.

- Scenario B: Dst = 01:80:c2:00:00:01 (exact reserved multicast). Expected: Valid PAUSE. Timer updated.

- Scenario C: Dst = 01:80:c2:00:00:02 (one above). Expected: NOT valid. Timer not updated.

- Verify: Exact address comparison in control frame detector. No off-by-one in multicast address check.

## GROUP G: RESET CORNER CASES

### tc_rst_txen_rxen_auto_disable

Gap: No test verifies the auto-disable behavior documented in the spec: if TX_BD_NUM=0x80, RXEN is auto-disabled regardless of the RXEN bit. If TX_BD_NUM=0x00, TXEN is auto-disabled. tc_tx_bd_num_split checks this but not in a reset context.

- Scenario A: Write TX_BD_NUM=0x80 with RXEN=1. Assert RST_I. Deassert. Write TX_BD_NUM=0x80 again. Write MODER with RXEN=1.

- Expected: RXEN bit is written but receiver disabled because no RX BDs exist. No frames received.

- Verify: RXEN bit value in register vs actual receiver behavior. Auto-disable is functional even after reset cycle.

### tc_rst_pause_timer_on_reset

Gap: No test verifies that the PAUSE timer is cleared on reset. If TX is paused due to a received PAUSE frame, asserting RST_I should clear the pause state and allow TX to resume.

- Setup: RXFLOW=1. Receive PAUSE frame with large timer value. Verify TX is paused.

- Fault: Assert RST_I while TX is paused.

- Expected: PauseTimer cleared to 0 on reset. Pause signal deasserts. TX can transmit immediately after reset completes and TXEN re-enabled.

- Verify: No residual pause state after reset. TX operates normally immediately after re-initialization.

### tc_rst_mid_pause_transmission

Gap: No test resets during outgoing PAUSE frame transmission (CtrlMux=1, SendingCtrlFrm=1).

- Setup: TXFLOW=1. Assert TPauseRq. Wait for CtrlMux=1 (PAUSE frame started). Assert RST_I mid-transmission.

- Expected: Partial PAUSE frame on MTx interface (may have incomplete CRC). WillSendControlFrame cleared by reset. CtrlMux cleared. After reset, core returns to normal operation. TPAUSERQ auto-cleared (it is already cleared after PAUSE sent per spec, but if reset mid-send, the latch clears).

- Verify: No state machine hang. Core operable after reset. Partial frame on wire does not corrupt receiver state.

### tc_rst_clears_interrupt_source

Gap: No test verifies INT_SOURCE and INT_MASK are both cleared after RST_I. If interrupts were pending before reset, they should not remain set after reset.

- Setup: Generate TXB and RXB interrupts. Read INT_SOURCE — both bits set.

- Fault: Assert RST_I.

- Expected: INT_SOURCE reset to 0x0. INTA_O deasserts. INT_MASK reset to 0x0. All interrupt conditions cleared.

- Verify: INT_SOURCE reads 0x0 after reset. INTA_O deasserted. Matches tc_reg_reset_values but specifically targets interrupt clearing during active interrupt condition.

## GROUP H: WISHBONE INTERFACE CORNER CASES

### tc_wb_back_to_back_register_writes

Gap: No test performs back-to-back WISHBONE register writes with no idle cycles between them. The core uses WbEn and WbEn_q to detect access cycles. Two immediate consecutive writes may cause the second to be missed if ACK handling has pipeline behavior.

- Setup: Write MODER, then immediately write PACKETLEN with CYC_I held high between transactions (pipelined WISHBONE).

- Expected: Both writes accepted. ACK_O for first, then ACK_O for second. No ERR_O.

- Verify: Both register values updated correctly. No write dropped in back-to-back scenario.

### tc_wb_inta_deassert_timing

Gap: No test verifies INTA_O deasserts within one clock cycle of writing 1 to the relevant INT_SOURCE bit. INTA_O is driven by INT_SOURCE & INT_MASK. The clear write and INTA_O deassert timing depends on when the register update propagates.

- Setup: Enable TXB_M in INT_MASK. Trigger TXB interrupt. Observe INTA_O asserted.

- Action: Write 1 to TXB bit in INT_SOURCE.

- Expected: INTA_O deasserts within 1-2 clock cycles of the write being accepted (ACK_O asserted).

- Verify: Exact cycle count between ACK_O for INT_SOURCE write and INTA_O deassert. No prolonged spurious interrupt assertion.

### tc_wb_master_err_response

Gap: No test drives M_ERR_I=1 in response to a DMA read (TX path) or DMA write (RX path). The master interface includes M_ERR_I for error response from memory.

- Setup: Configure TX BD. Start frame transmission (core begins DMA reads). Drive M_ERR_I=1 in response to a DMA read.

- Expected: DMA access aborted. Frame transmission may abort (underrun likely). UR bit set in Tx BD if FIFO starved.

- Verify: Core handles M_ERR_I gracefully. No WISHBONE deadlock (M_CYC_O deasserts). System recovers for next frame. M_ERR_I on RX DMA write similarly tested.

### tc_wb_bd_access_during_dma

Gap: No test attempts host (WISHBONE slave) BD access while DMA (WISHBONE master) is also accessing the BD RAM. The internal SPRAM is shared between both paths with priority logic.

- Setup: Start frame transmission (DMA active reading BD). Simultaneously attempt host write to same BD region via WISHBONE slave.

- Expected: One of the two accesses is stalled (arbitration). The BD RAM cannot be accessed by both simultaneously. Host access either stalls (wait states) or gets ERR_O. DMA takes priority or host takes priority (RTL-dependent).

- Verify: No data corruption. Only one access proceeds at a time. Host and DMA accesses are serialized. Document priority.

## GROUP I: LOOPBACK CORNER CASES

### tc_tx_loopback_collision_behavior

Gap: tc_tx_loopback only verifies data appears on RX path. No test verifies collision behavior in loopback mode. In loopback, MColl and MCrS are typically irrelevant since data never hits the wire.

- Setup: LOOPBCK=1, FULLD=0 (half duplex). Transmit a frame. Inject MColl from PHY during transmission.

- Expected: In loopback mode, the transmitted data is internally looped. MColl behavior in loopback is implementation-specific — the RTL may or may not respond to collisions in loopback.

- Verify: Does collision abort loopback transmission? Is RTRY incremented? Is the looped-back frame corrupted? Document whether CSMA/CD applies in loopback mode.

### tc_tx_loopback_pause_frame

Gap: No test verifies PAUSE frame behavior in loopback. A PAUSE frame transmitted in loopback mode should loop back to the RX path. If RXFLOW=1, the core would then pause its own TX because it received its own PAUSE frame.

- Setup: LOOPBCK=1, TXFLOW=1, RXFLOW=1. Write TXPAUSERQ=1 with timer=0x0100.

- Expected: Outgoing PAUSE frame transmitted. Loopback routes it to RX input. Control frame detector validates it (destination must match own MAC or reserved multicast). If valid: PAUSE timer loaded. TX pauses itself.

- Verify: Self-pause behavior in loopback. TXC interrupt (outgoing PAUSE). Possible RXC interrupt (if looped-back PAUSE is valid). TX pauses after sending its own PAUSE frame.

## GROUP J: MULTIPLE INTERRUPT CORNER CASES

### tc_int_txb_and_txe_same_bd

Gap: A single BD cannot generate both TXB and TXE simultaneously — it is either success (TXB) or error (TXE). But consecutive BDs processed quickly could generate both almost simultaneously. No test verifies this case.

- Setup: Two TX BDs with IRQ=1. BD0 transmits successfully (TXB). BD1 immediately underruns (TXE). Both complete within a few clock cycles.

- Expected: Both TXB and TXE bits set in INT_SOURCE. Single INTA_O assertion. No bit lost.

- Verify: INT_SOURCE shows 0x03 (TXB | TXE). Host reads both conditions. Clearing one does not affect the other.

### tc_int_rxb_while_clearing_txb

Gap: No test verifies that a new RXB interrupt arriving while the host is writing to clear TXB does not lose the RXB event. This is a write-1-to-clear race condition.

- Setup: TXB=1 pending. RXB arrives simultaneously with host write-1-to-clear TXB.

- Expected: Host writes 0x01 to INT_SOURCE (clear TXB only). Simultaneously RXB fires. Result: TXB cleared, RXB set.

- Verify: After the write: TXB=0, RXB=1. No lost interrupts. RTL must latch RXB independently of the write path.

### tc_int_busy_with_rxb_pending

Gap: No test generates BUSY and RXB simultaneously. BUSY fires when a buffer is received but discarded due to no empty BD. RXB fires when a buffer is received and stored. These can co-exist if some BDs were empty (stored) and then BD pool exhausted (discarded) in rapid succession.

- Setup: Provide 2 Rx BDs (E=1). Send 3 frames rapidly. First two fill the BDs (RXB×2). Third arrives with no BD (BUSY).

- Expected: INT_SOURCE = BUSY | RXB. All three bits potentially set. INTA_O asserts. Host must service both conditions.

- Verify: BUSY and RXB both visible in INT_SOURCE simultaneously. Host can distinguish stored frames from discarded ones.

## GROUP K: HASH TABLE CORNER CASES

### tc_addr_hash_all_zeros

Gap: No test verifies hash table behavior when HASH0=HASH1=0x00000000. With all hash bits zero and IAM=1, no multicast address should be accepted. PRO=0, BRO=0.

- Setup: IAM=1, HASH0=0, HASH1=0. Send multicast frame with any address.

- Expected: Hash function produces 6-bit index. HASH0/HASH1 bit at that index = 0. Frame rejected.

- Verify: All multicast frames rejected. Only own unicast address accepted (via direct comparison path). Zero hash table blocks all multicast.

### tc_addr_hash_all_ones

Gap: HASH0=HASH1=0xFFFFFFFF with IAM=1 means all 64 possible hash values match. Every multicast address should be accepted.

- Setup: IAM=1, HASH0=0xFFFFFFFF, HASH1=0xFFFFFFFF. Send 10 different multicast frames.

- Expected: All 10 multicast frames accepted (hash bit always 1 regardless of address). M=0 in Rx BD (hit, not promiscuous).

- Verify: All multicast addresses accepted. M bit=0 (address hit via hash, not promiscuous accept). Unicast to different address still filtered (IAM=1 only covers individual/multicast hash, unicast still compared directly).

### tc_addr_hash_collision

Gap: No test verifies that two different multicast addresses that hash to the same 6-bit index are both accepted when IAM=1 (this is expected hash table behavior, not an error).

- Setup: IAM=1. Calculate two multicast addresses that produce the same CRC-based hash index. Set only that one bit in HASH0 or HASH1.

- Expected: Both multicast addresses accepted (same hash bit). Other multicast addresses rejected (different hash bits, not set).

- Verify: Hash collision is intentional and correct behavior. Both addresses accepted, others filtered.

## GROUP L: SYSTEM-LEVEL CORNER CASES

### tc_system_tx_rx_mode_switch

Gap: No test switches from half-duplex to full-duplex mid-operation by toggling FULLD in MODER. The spec warns against changing MODER after TXEN/RXEN, but this corner tests actual behavior.

- Setup: FULLD=0 (half duplex). Active TX and RX. Toggle FULLD=1.

- Expected: Rule1 immediately forced to 1 via FullD signal. Collision signals (MColl, MCrS) masked. IPGT path used for next IPG. Any ongoing collision handling aborted or ignored.

- Verify: No state machine deadlock. Rule1 updates within one MTxClk. Collision masking takes effect. Document whether in-progress frame is aborted.

### tc_system_all_bd_tx_rx_toggle

Gap: No test alternates between all-TX (TX_BD_NUM=0x80) and all-RX (TX_BD_NUM=0x00) configurations rapidly.

- Setup: Write TX_BD_NUM=0x80. Transmit burst. Write TX_BD_NUM=0x00. Receive burst. Repeat 5 times.

- Expected: RxBDAddress re-initialized to r_TxBDNum[6:0] when RXEN re-enabled after each change. TxBDAddress re-initialized to 0 when TXEN re-enabled. No BD address corruption between modes.

- Verify: First frame in each mode uses correct BD base address. No leftover state from previous mode. System correctly switches roles.

### tc_system_miim_during_heavy_tx

Gap: No test verifies MIIM operation while the core is simultaneously performing heavy DMA activity (TX burst). The MIIM module uses the same system clock but different interface (MDC/MDIO). Resource contention on the WISHBONE bus should not affect MIIM since it is a separate module.

- Setup: Start 10-frame TX burst. Simultaneously initiate MIIM PHY register read (RSTAT=1).

- Expected: MIIM read completes correctly despite DMA activity. BUSY deasserts. PRSD contains correct PHY data. TX frames all transmitted correctly. No cross-contamination.

- Verify: Both operations complete successfully and independently. MIIM BUSY duration unaffected by DMA. TX frame data unaffected by MIIM serial traffic.

### tc_system_crc_then_ipg_then_pause

Gap: No test creates the full sequence: frame with CRC error received, immediately followed by PAUSE frame, immediately followed by TX frame. Tests all three path interactions in sequence.

- Setup: RXFLOW=1, IRQ=1 in Rx BD.

- Sequence:

  - PHY sends frame with corrupted CRC

  - Immediately PHY sends valid PAUSE frame (minimal gap)

  - Host transmits TX frame while pause timer active

- Expected:

  - Rx BD written with CRC=1, RXE interrupt

  - PAUSE timer loaded, RXC interrupt set

  - TX frame deferred until PAUSE timer expires

- Verify: All three events captured in correct order. No missed interrupts. CRC error does not prevent PAUSE from being processed. TX waits for PAUSE timer before transmitting.

Let me read the uploaded file carefully and cross-reference with what I previously wrote.Good. Now let me carefully compare against what I wrote previously and find what's truly unique to my list.Here are the unique corner cases from my previous list that are not covered in the verification plan document:

## 1. Simultaneous TX/RX Race Conditions

- TX and RX hitting the BD table boundary simultaneously — TX_BD_NUM=0x40, both TX and RX wrap their last BD (WR=1) at the exact same clock cycle

- RX frame arrives exactly while TX is in FCS state — the precise overlap where TX is transmitting its last 4 CRC bytes and a new RX frame starts arriving simultaneously

- TX underrun triggered specifically by RX DMA write stealing the WISHBONE bus — not just generic wait states, but the exact contention where an RX DMA write causes the TX DMA read to stall long enough to drain the TX FIFO

- TX and RX BDs wrap simultaneously — both last BDs have WR=1 and both complete at the same time, wrap-around race on both sides

## 2. Flow Control Race Conditions

- PAUSE frame received at the exact cycle TX is asserting TxStartFrm — does TX stop before or after the first nibble goes out?

- PAUSE frame with timer value = 0x0000 — zero pause, TX should resume immediately without any actual pause

- PAUSE frame with timer value = 0xFFFF — maximum pause duration, verify RX path continues functioning during the entire maximum pause window

- Two consecutive PAUSE frames arriving before the first timer expires — second PAUSE value must overwrite the first in the pause timer, not accumulate

- Host asserts TXPAUSERQ exactly when an incoming PAUSE frame is mid-reception — both TXFLOW and RXFLOW active, TX wants to send PAUSE while simultaneously being paused by incoming PAUSE

- PAUSE frame arrives during TX backoff state — MAC is already in binary exponential backoff after a collision, PAUSE arrives; which takes priority?

- PAUSE timer reaches zero at the exact cycle a new TxStartFrm arrives from host — race between timer expiry releasing TX and host requesting a new transmission

## 3. WISHBONE Bus Contention

- Configuration register write via WISHBONE slave exactly while WISHBONE master is performing DMA — slave and master interfaces active simultaneously on the same cycle

- WISHBONE M_ERR_I asserted during active DMA frame transfer — verify error is reflected correctly in BD status and correct interrupt fires

- Back-to-back WISHBONE DMA transfers with zero idle cycles — TX DMA read completes and RX DMA write starts on the very next cycle with no gap

## 4. Collision Boundary Cases

- Collision at exactly NibCnt = COLLVALID (last nibble of collision window) — the precise boundary between normal collision and late collision

- Collision at exactly NibCnt = COLLVALID+1 — one nibble past the window, must be treated as late collision not normal collision

- Collision during PAD insertion — frame is short and being padded, collision occurs mid-padding before FCS

- Collision during FCS transmission — collision in the very last 4 bytes of the frame, verify full retransmission from preamble

- Back-to-back collisions on consecutive different frames — verify RTRY counter resets to zero between frames and does not carry over

## 5. MII Management vs MAC Simultaneous Operation

- PHY scan status continuously running while simultaneously transmitting a maximum-size frame — MDIO activity and TX MII activity at the same time

- PHY LINKFAIL asserted mid-transmission — link goes down exactly while a frame is being clocked out on MTxD

- MIIM write to PHY exactly when RX frame is arriving on MRxD — WISHBONE bus conflict between MIIM register access and RX DMA write

- CLKDIV changed while MIIM operation is actively in progress — not between operations like tc_miim_clkdiv_all does, but during an active MDC clock cycle

- Second MIIM command issued while BUSY=1 — host does not wait for BUSY to clear, verify second command is held off and first completes correctly

## 6. Address Recognition Edge Cases

- Frame with destination address = 0x00:00:00:00:00:00 (all zeros) — not broadcast, not multicast, not matching any real MAC, verify dropped cleanly

- Multicast frame whose CRC hash collides with a valid HASH table entry but is not the intended address — false positive scenario, frame passes hardware filter but should be rejected by software

- HASH0/HASH1 written while a multicast frame is mid-reception — hash lookup reads a partially updated table during address check

- PRO=1 and BRO=1 simultaneously — broadcast frame arrives, PRO should override BRO and frame must be accepted

- Frame destination matches own MAC address while PRO=1 — accepted by both UnicastOK and r_Pro paths simultaneously, verify no double-processing or BD corruption

- c_sys_hash_collision_during_flow_control: Two addresses map to same hash bit — one is valid multicast, other is PAUSE frame multicast address — verify PAUSE correctly identified and not confused with data multicas

## 7. IPG Timing Boundaries

- CarrierSense appears at exactly NibCnt = IPGR1 — the precise boundary: should defer and reset counter

- CarrierSense appears at exactly NibCnt = IPGR1+1 — one count past the window: counter must continue, not reset

- Back-to-back frames with IPGT = 0x00 — zero inter-packet gap, verify frames don't merge into one on the MII

- Switch from full duplex to half duplex without updating IPGR1/IPGR2 — IPGT was used in full duplex, now stale IPGR values govern half duplex timing

## 8. Interrupt Race Conditions

- INT_SOURCE bit cleared (write-1-to-clear) at the exact same cycle the same event fires again — the clear and the new event arrive simultaneously, bit must remain set

- TXB and TXE both eligible on the same BD — frame transmitted but also had an error condition, verify correct interrupt priority

## 9. Reset Edge Cases

- RST_I asserted while MIIM operation is in progress — per spec MIIM is NOT reset by RST_I, verify MIIM completes its current operation cleanly while rest of MAC resets

- RST_I deasserted and TXEN set immediately on the very next clock — no BD configuration done, transmitter enabled with uninitialized descriptors

## Additional System-Level Test Cases

Looking at your document, here are test cases that cover gaps not addressed:

### Group 11: Cross-Feature Interaction Tests

These test multiple features working simultaneously — something your document doesn't cover:

| Test | Description |
| --- | --- |
| tc_tx_crc_with_padding | PAD=1 and CRCEN=1 together — verify padding applied first THEN CRC calculated on padded frame |
| tc_rx_hash_with_promiscuous | IAM=1 and PRO=1 simultaneously — verify M bit behavior when frame matches hash vs when it doesn't |
| tc_tx_hugen_with_crc | HUGEN=1 sending >1518B frame — verify CRC correctly calculated on oversized frame |
| tc_rx_recsmall_with_crc | RECSMALL=1 receiving short frame — verify CRC error handling on sub-64B frames |
| tc_fulld_with_flow_control | FULLD=1 with RXFLOW=1 — verify PAUSE frame correctly blocks TX in full duplex while RX continues |
| tc_loopback_with_flow_control | LOOPBCK=1 with TXFLOW=1 — verify PAUSE frame sent by TX is received by RX in loopback |
| tc_iam_with_broadcast | IAM=1 and BRO=0 — verify broadcast frames still accepted even when hash table filtering is active |
| tc_nopre_with_dlycrcen | NOPRE=1 and DLYCRCEN=1 — verify CRC reference point when preamble is suppressed |
| tc_sys_bro_pro_pause_together | BRO=1, PRO=1, RXFLOW=1 — broadcast PAUSE arrives (FF:FF:FF:FF:FF:FF) — verify PAUSE not processed because dest is broadcast not multicast reserved address |

### Group 12: BD State Machine Corner Cases

Your document tests basic BD operations but misses these state transitions:

| Test | Description |
| --- | --- |
| tc_bd_ready_bit_race | Set RD bit in TX BD then immediately try to modify BD — verify hardware ignores host write while RD=1 |
| tc_rx_bd_empty_cleared_mid_rx | Clear Empty bit in RX BD while frame is being received — verify hardware behavior |
| tc_tx_bd_length_zero | Set TX BD with LEN=0 — verify frame not transmitted and BD behavior |
| tc_tx_bd_length_four | Set TX BD with LEN=4 exactly — verify not transmitted per spec note |
| tc_tx_bd_length_five | Set TX BD with LEN=5 — verify this is minimum that actually transmits |
| tc_rx_bd_wrap_mid_burst | WR bit set on middle BD during active reception burst — verify circular wrap happens correctly |
| tc_tx_multiple_errors_same_bd | Force underrun AND late collision simultaneously — verify which error bit takes priority |
| tc_bd_irq_bit_cleared_before_tx | Set IRQ=1 in BD then clear it before transmission starts — verify no interrupt generated |

### Group 13: Clock Domain & Timing Edge Cases

| Test | Description |
| --- | --- |
| tc_mdc_during_miim_operation | Change CLKDIV value while MIIM operation in progress — verify no MDIO frame corruption |
| tc_rxclk_faster_than_txclk | MRxClk at 25MHz and MTxClk at 2.5MHz simultaneously — verify independent clock domains |
| tc_mii_clk_glitch | Inject single cycle glitch on MTxClk — verify TX frame not corrupted |
| tc_wishbone_zero_wait_states | All WISHBONE transactions with ACK_O same cycle as STB_I — verify minimum latency operation |
| tc_wishbone_max_wait_states | Maximum wait states (255 cycles) on WISHBONE — verify no timeout or deadlock |

### Group 14: Interrupt System Edge Cases

Not covered in your Group 8:

| Test | Description |
| --- | --- |
| tc_int_source_w1c_during_event | Write 1 to clear INT_SOURCE bit exactly when same event fires again — verify bit stays set |
| tc_int_mask_change_during_active | Change INT_MASK while INTA_O is asserted — verify INTA_O responds immediately |
| tc_int_all_sources_simultaneously | Force all 7 interrupt sources at same time — verify all bits set in single INT_SOURCE read |
| tc_int_busy_with_rxe | BUSY and RXE both triggered — verify both bits set independently |
| tc_int_txe_without_irq_bit | Force TX error with IRQ=0 in TX BD — verify TXE does NOT assert per spec |
| tc_int_rxc_overrides_rxb | Control frame received with IRQ=1 in RX BD — verify RXC set instead of RXB per spec |

### Group 15: Address Recognition Edge Cases

Extends your Group 5 with missing scenarios:

| Test | Description |
| --- | --- |
| tc_addr_all_zeros_dest | Frame with destination MAC = 00:00:00:00:00:00 — verify behavior |
| tc_addr_multicast_bit_boundary | Multicast address hashing to bit 0 of HASH0 and bit 63 of HASH1 — verify boundary bits |
| tc_addr_hash_false_positive | Two different addresses mapping to same hash bit — verify upper layer sees both but M bit distinguishes |
| tc_addr_iam_zero_hash | IAM=1 but HASH0=HASH1=0x00000000 — verify all individual frames rejected |
| tc_addr_iam_full_hash | IAM=1 with HASH0=HASH1=0xFFFFFFFF — verify all individual frames accepted |
| tc_addr_mac_change_during_rx | Change MAC_ADDR0/1 while frame reception in progress — verify which address used |
| tc_addr_bro_with_pro | BRO=1 and PRO=1 — verify broadcast accepted because PRO overrides BRO per spec |

### Group 16: PAUSE Frame Edge Cases

Extends your Group 6 with missing scenarios:

| Test | Description |
| --- | --- |
| tc_fc_pause_timer_zero | Receive PAUSE frame with timer value = 0 — verify TX not paused at all |
| tc_fc_pause_timer_max | Receive PAUSE frame with timer value = 0xFFFF — verify TX paused for maximum duration |
| tc_fc_pause_during_pause | Receive second PAUSE frame while first timer still running — verify timer reloaded with new value |
| tc_fc_tx_pause_then_rx_pause | Send PAUSE frame then immediately receive PAUSE frame — verify both TXC and RXC interrupts |
| tc_fc_invalid_pause_dest | PAUSE frame with wrong destination address — verify not treated as control frame |
| tc_fc_pause_opcode_wrong | Frame with correct type=8808 but opcode≠0001 — verify not treated as PAUSE |
| tc_fc_txpauserq_auto_clear | Write TXPAUSERQ=1 and verify it auto-clears after PAUSE frame sent per spec |
| tc_fc_second_pause_request | Write TXPAUSERQ=1 while first PAUSE still being transmitted — verify second request held until first completes |

### Group 17: DMA Path Specific Tests

Your document covers DMA conceptually but these specific scenarios are missing:

| Test | Description |
| --- | --- |
| tc_dma_tx_read_error | M_ERR_I asserted during TX DMA read — verify UR bit set in TX BD and transmission aborted |
| tc_dma_rx_write_error | M_ERR_I asserted during RX DMA write — verify OR bit set in RX BD |
| tc_dma_non_word_aligned | TX buffer pointer at address 0x00000001 — verify correct byte selection on WISHBONE master |
| tc_dma_tx_burst_across_page | TX buffer spanning two memory pages — verify DMA correctly crosses address boundary |
| tc_dma_simultaneous_bd_access | Host accessing BD RAM via slave interface while DUT performing DMA — verify no corruption |
| tc_dma_wishbone_sel_not_full | M_SEL_O not 1111 during DMA — verify M_ERR_I response behavior |

### Group 18: Security & Robustness Tests

| Test | Description |
| --- | --- |
| tc_rob_invalid_register_address | Write to undefined register address (e.g. 0x54, 0x58) — verify ERR_O asserted |
| tc_rob_sel_not_1111 | Access valid register with SEL_I=0xE — verify ERR_O asserted per spec |
| tc_rob_write_to_ro_register | Write to MIIRX_DATA or MIISTATUS — verify register unchanged |
| tc_rob_txbd_num_above_max | Write 0x81 to TX_BD_NUM — verify register retains previous value per spec |
| tc_rob_cyc_without_stb | Assert CYC_I without STB_I — verify no transaction initiated |
| tc_rob_stb_without_cyc | Assert STB_I without CYC_I — verify ERR_O or no response |
| tc_rob_rapid_enable_disable | Toggle TXEN/RXEN rapidly every clock — verify no BD corruption or stuck state |