## Section 1 — Wishbone Slave Interface Assertions

WBS_RESET_NO_X_Z_PROPAGATION: This assertion is repeated for every port to ensure it doesn’t take x or z values after reset.

WBS_RST_ACK_DEASSERTED ACK_O must be deasserted while RST_I is high.

WBS_RST_ERR_DEASSERTED ERR_O must be deasserted while RST_I is high.

WBS_RST_INTA_DEASSERTED INTA_O must be deasserted during reset.

WBS_STB_REQUIRES_CYC STB_I must never be asserted without CYC_I also being asserted in the same cycle.

WBS_NO_ACK_WITHOUT_CYC ACK_O must never assert when CYC_I is deasserted.

WBS_NO_ERR_WITHOUT_CYC ERR_O must never assert when CYC_I is deasserted.

WBS_ACK_REQUIRES_STB_CYC ACK_O may only assert when both CYC_I and STB_I are simultaneously high..

WBS_ERR_REQUIRES_STB_CYC ERR_O may only assert when both CYC_I and STB_I are high.

WBS_ACK_ERR_MUTEX ACK_O and ERR_O must never be asserted simultaneously.

WBS_ERR_ON_PARTIAL_SEL When SEL_I is not 4'hF during a valid bus cycle ERR_O must assert.

WBS_NO_ACK_ON_PARTIAL_SEL ACK_O must not assert when SEL_I is not 4'hF.

WBS_ERR_ON_INVALID_ADDR When a valid bus cycle targets an out-of-range address ERR_O must be asserted.

WBS_ADDR_STABLE ADDR_I must not change before ACK_O arrives during a write or read cycle wait state.

WBS_WRITE_DATA_STABLE During a write cycle wait state DATA_I must remain stable until ACK_O arrives.

WBS_WRITE_SEL_STABLE SEL_I must not change during any wait state before ACK_O.

WBS_WE_STABLE_DURING_TRANSFER WE_I must not change during a DMA bus phase while waiting for ACK_O.

WBS_ACK_TIMEOUT For every valid bus cycle either ACK_O or ERR_O must arrive within N clock cycles. Without this check a slave deadlock would only be visible as a simulation timeout rather than a pointed failure.

## Section 2 — Wishbone Master Interface Assertions

WBM_RESET_NO_X_Z_PROPAGATION: This assertion is repeated for every port to ensure it doesn’t take x or z values after reset.

WBM_RST_STB_DEASSERTED M_STB_O must be deasserted during reset. MAC is initialized.

WBM_RST_CYC_DEASSERTED M_CYC_O must be deasserted during reset.

WBM_STB_REQUIRES_CYC M_STB_O must never assert without M_CYC_O also being asserted.

WBM_NO_ACK_WITHOUT_CYC M_ACK_I must never assert when M_CYC_O is deasserted.

WBM_NO_ERR_WITHOUT_CYC M_ERR_I must never assert when M_CYC_O is deasserted.

WBM_ACK_REQUIRES_STB_CYC M_ACK_I may only assert when both M_CYC_O and M_STB_O are simultaneously high.

WBM_ERR_REQUIRES_STB_CYC M_ERR_I may only assert when both M_CYC_O and M_STB_O are high.

WBM_ACK_ERR_MUTEX M_ACK_I and M_ERR_I must never be asserted simultaneously.

WBM_ADDR_STABLE_DURING_TRANSFER M_ADDR_O must not change while waiting for M_ACK_I during a DMA phase.

WBM_SEL_STABLE_DURING_TRANSFER M_SEL_O must not change while a DMA phase is active and awaiting M_ACK_I.

WBM_WE_STABLE_DURING_TRANSFER M_WE_O must not change during a DMA bus phase while waiting for M_ACK_I.

WBM_WDATA_STABLE_DURING_WRITE During an RX DMA write M_DATA_O must not change while waiting for M_ACK_I.

WBM_ERR_ON_PARTIAL_SEL When M_SEL_O is not 4'hF during a valid bus cycle M_ERR_I must be asserted.

WBM_NO_ACK_ON_PARTIAL_SEL M_ACK_I must not assert when M_SEL_O is not 4'hF.

WBM_ACK_TIMEOUT For every DMA bus phase M_ACK_I or M_ERR_I must arrive within N clock cycles. Without this a memory model timeout would appear as a silent hang rather than a pointed assertion failure.

## Section 3 — MII TX Interface Assertions

TX_RESET_NO_X_Z_PROPAGATION: This assertion is repeated for every port to ensure it doesn’t take x or z values after reset.

TX_CLK_FREQ: Check that  MTxClk frequency is 25 MHz or 2.5 MHz with half duty cycle.

TX_ERR_REQUIRES_TXEN MTxErr must only assert when MTxEn is also asserted.

TX_PREAMBLE_NIBBLE_PATTERN The first nibble after MTxEn rises must be 0x5 which is the correct preamble nibble per IEEE 802.3. A wrong first nibble means the remote receiver cannot synchronize its clock recovery PLL and will discard the frame.

TX_MIN_FRAME_NIBBLES MTxDV must be asserted for at least 10 cycles (min. packet length >4 + 1 SFD byte).

## Section 4 — MII RX Interface Assertions

RX_RESET_NO_X_Z_PROPAGATION: This assertion is repeated for every port to ensure it doesn’t take x or z values after reset.

RX_CLK_FREQ: Check that  MRxClk frequency is 25 MHz or 2.5 MHz with half duty cycle.

RX_ERR_DURING_ACTIVE_FRAME MRxErr must only assert when MRxDV is also asserted. An MRxErr outside a frame has no target BD and could corrupt the status of an unrelated BD.

RX_MIN_FRAME_NIBBLES MRxDV must be asserted for at least 10 cycles (min. packet length >4 + 1 SFD byte)

## Section 5 — MII Management Interface Assertions

MDIO_RESET_NO_X_Z_PROPAGATION: This assertion is repeated for every port to ensure it doesn’t take x or z values after reset.

MDIO_MDC_STABLE_DURING_RST MDC must be low and not toggling during RST_I.