4. Complete Test Plan

Group 1: Register & BD Access Tests

| Test | Description | RAL Method |
| --- | --- | --- |
| tc_reg_reset_values | Verify all 21 registers match spec reset values after RST_I | uvm_reg_hw_reset_seq |
| tc_reg_walking_one | Walking-1 across all RW bits, single WISHBONE cycles | uvm_reg_bit_bash_seq |
| tc_reg_walking_zero | Walking-0 across all RW bits | uvm_reg_bit_bash_seq |
| tc_reg_max_values | Write max value, read back, hard reset, verify | Custom seq |
| tc_reg_read_only_fields | Verify R-only fields (MIIRX_DATA, MIISTATUS bits) reject writes | Custom seq |
| tc_bd_walking_one | Walking-1 across BD RAM (0x400–0x7FF) | uvm_mem_walk_seq |
| tc_bd_reset_preservation | BD RAM holds values after soft reset (RST bit), clears after hard reset | Custom seq |
| tc_bd_boundary | Access first BD (0x400), last BD (0x7FC), and out-of-range | uvm_mem_access_seq |
| tc_tx_bd_num_split | Write TX_BD_NUM=0x32 → verify 50 TX BDs and 78 RX BDs are accessible | Custom seq |
| tc_bd_access_during_reset | Attempt BD access while RST bit set → expect no access | Custom seq |

Group 2: MIIM (MII Management) Tests

| Test | Description |
| --- | --- |
| tc_miim_clkdiv_all | Test all 255 clock divider values in MIIMODER.CLKDIV; verify MDC frequency = CLK/(2×CLKDIV) |
| tc_miim_write_phy_reg | Write to PHY control register (addr 0x0) via MIICOMMAND.WCTRLDATA; verify MDIO frame format |
| tc_miim_read_phy_reg | Read from PHY status register (addr 0x1); verify PRSD in MIIRX_DATA |
| tc_miim_read_nonwritable | Attempt write to read-only PHY register; verify PHY ignores it |
| tc_miim_reset_phy | Assert PHY reset through MIIM write; verify PHY responds |
| tc_miim_walk_phy_addr | Walking-1 across FIAD[4:0] with and without preamble |
| tc_miim_walk_reg_addr | Walking-1 across RGAD[4:0] with and without preamble |
| tc_miim_walk_data | Walking-1 across 16-bit write data with and without preamble |
| tc_miim_wrong_phy_addr | Read from non-existent PHY address → MDIO high-Z, verify MIIRX_DATA undefined |
| tc_miim_wrong_write_correct_read | Write to wrong PHY addr, read from correct addr; verify no corruption |
| tc_miim_busy_nvalid_write | Verify BUSY asserts during write, deasserts at end; NVALID behavior |
| tc_miim_busy_nvalid_read | Verify BUSY/NVALID durations during read operation |
| tc_miim_busy_nvalid_scan | Verify BUSY stays asserted during continuous scan; NVALID clears after first scan |
| tc_miim_scan_linkfail | Scan PHY status; detect LINKFAIL asserted/deasserted; verify MIISTATUS.LINKFAIL |
| tc_miim_scan_linkfail_sliding | Sliding LINKFAIL bit during scan — verify edge detection |
| tc_miim_stop_scan_after_read | Assert SCANSTAT then immediately clear; sliding stop with/without preamble |
| tc_miim_stop_scan_after_write | Stop scan immediately after write request; sliding |
| tc_miim_stop_scan_after_scan | Stop scan after 1st and 2nd scan iterations; sliding |
| tc_miim_no_preamble | Set MIIMODER.MIINOPRE=1; verify 32-bit preamble absent on MDIO |

Group 3: Frame Transmission Tests

| Test | Description |
| --- | --- |
| tc_tx_basic_frame | Transmit single minimum-length frame (64B); verify preamble + SFD + data + CRC on MII |
| tc_tx_max_frame | Transmit maximum-length frame (1518B); verify no truncation |
| tc_tx_short_frame_pad_on | Frame < MINFL with PAD=1; verify padded to 64B with zeros + CRC |
| tc_tx_short_frame_pad_off | Frame < MINFL with PAD=0; verify transmitted as-is (no padding) |
| tc_tx_crc_enable | CRCEN=1; verify 4-byte CRC appended to frame |
| tc_tx_crc_disable | CRCEN=0; verify no CRC appended |
| tc_tx_delayed_crc | DLYCRCEN=1; CRC calculation starts 4 bytes after SFD |
| tc_tx_huge_frame | HUGEN=1; transmit frame > 1518B; verify transmission completes |
| tc_tx_huge_disabled | HUGEN=0; transmit frame > MAXFL; verify abort at max |
| tc_tx_no_preamble | NOPRE=1 in MODER; verify 7-byte preamble suppressed |
| tc_tx_ipg_back_to_back | Transmit back-to-back frames; verify IPG ≥ 0.96µs (IPGT register value) |
| tc_tx_ipg_non_back_to_back | Verify IPGR1 and IPGR2 windows respected |
| tc_tx_multi_bd_chain | Chain 8 TX BDs (WR=0 until last); verify continuous transmission |
| tc_tx_wrap_bit | Last BD has WR=1; verify circular wrap to first BD |
| tc_tx_underrun | Throttle WISHBONE master to starve TX FIFO; verify TxAbort + UR bit in TX BD |
| tc_tx_done_interrupt | IRQ=1 in TX BD; verify TXB interrupt in INT_SOURCE after transmission |
| tc_tx_error_interrupt | Force underrun with IRQ=1; verify TXE interrupt |
| tc_tx_retry_count | In half-duplex: inject collision; verify RTRY field incremented in TX BD |
| tc_tx_max_retry | Inject MAX_RET+1 collisions; verify RL bit set, transmission aborted |
| tc_tx_late_collision | Inject collision after COLLVALID window; verify LC bit + abort |
| tc_tx_excessive_deferral | EXDFREN=0; keep carrier busy > defer limit; verify abort |
| tc_tx_excessive_deferral_wait | EXDFREN=1; verify TX waits indefinitely for carrier |
| tc_tx_loopback | LOOPBCK=1; verify transmitted data appears on RX path |
| tc_tx_carrier_sense_lost | Assert MCrS then deassert mid-frame; verify CS bit in TX BD |
| tc_tx_only_4byte_min | TX BD with LEN ≤ 4; verify frame not transmitted (spec note) |
| tc_tx_unaligned_ptr | TX buffer pointer at non-word-aligned address; verify correct byte extraction |

Group 4: Frame Reception Tests

| Test | Description |
| --- | --- |
| tc_rx_basic_frame | PHY sends standard frame; verify assembly and storage in memory via DMA |
| tc_rx_crc_check_pass | Valid CRC frame; verify no CRC error in RX BD |
| tc_rx_crc_check_fail | Corrupt last byte of CRC; verify CRC bit set in RX BD |
| tc_rx_short_frame_reject | Frame < MINFL; RECSMALL=0; verify frame dropped |
| tc_rx_short_frame_accept | Frame < MINFL; RECSMALL=1; verify SF bit set in RX BD |
| tc_rx_too_long_frame | Frame > MAXFL; HUGEN=0; verify reception stops at MAXFL; TL bit set |
| tc_rx_huge_frame_enable | Frame > MAXFL; HUGEN=1; verify full frame received |
| tc_rx_preamble_removal | Verify 7-byte preamble + SFD stripped; only payload in memory |
| tc_rx_short_preamble | PHY sends frame with <7-byte preamble (only SFD); verify accepted |
| tc_rx_delayed_crc | DLYCRCEN=1; verify CRC checking starts 4 bytes after SFD |
| tc_rx_dribble_nibble | PHY sends odd-nibble-count frame; verify DN and CRC bits in RX BD |
| tc_rx_invalid_symbol | PHY sets MRxD=0xE (100Mbps error code); verify IS bit in RX BD |
| tc_rx_phy_error | MRxErr asserted mid-frame; verify frame aborted, no error in BD |
| tc_rx_overrun | Throttle WISHBONE master; RX FIFO overrun; verify OR bit in RX BD |
| tc_rx_ifg_minimum | Two back-to-back frames with IFG < 960ns; verify second frame dropped |
| tc_rx_ifg_disable | r_IFG=1 in MODER; verify frames accepted regardless of IFG |
| tc_rx_multi_bd | Receive frames filling multiple RX BDs; verify wrap using WR bit |
| tc_rx_no_empty_bd | All RX BDs marked non-empty; verify frame discarded, BUSY interrupt |
| tc_rx_interrupt_rxb | IRQ=1 in RX BD; verify RXB interrupt after frame received |
| tc_rx_interrupt_rxe | Receive with error, IRQ=1; verify RXE interrupt |
| tc_rx_late_collision | Late collision during reception; verify LC bit in RX BD |
| tc_rx_unaligned_ptr | RX buffer pointer at non-word-aligned address; verify correct byte select |
| tc_rx_only_4byte_min | Frame ≤ 4 bytes; verify received with CRC error (per spec note) |

Group 5: Address Recognition Tests

| Test | Description |
| --- | --- |
| tc_addr_unicast_match | Frame dest = MAC_ADDR0/1; verify accepted |
| tc_addr_unicast_mismatch | Frame dest ≠ MAC address; PRO=0; verify dropped (RxAbort) |
| tc_addr_promiscuous | PRO=1; send frame with foreign dest; verify accepted + M bit set in RX BD |
| tc_addr_broadcast_accept | BRO=0; send broadcast (FF:FF:FF:FF:FF:FF); verify accepted |
| tc_addr_broadcast_reject | BRO=1; send broadcast; PRO=0; verify rejected |
| tc_addr_multicast_hash | IAM=1; load HASH0/HASH1; send multicast matching hash; verify accepted |
| tc_addr_multicast_hash_miss | IAM=1; multicast not in hash table; verify dropped |
| tc_addr_miss_bit | PRO=1; frame doesn't match MAC; verify M=1 in RX BD |

Group 6: Flow Control Tests (Full Duplex)

| Test | Description |
| --- | --- |
| tc_fc_rx_pause_frame | PHY sends valid PAUSE frame (type=8808, op=0001); RXFLOW=1; verify TX paused for timer value |
| tc_fc_rx_pause_timer_expire | PAUSE timer counts down to 0; verify TX resumes |
| tc_fc_rx_pause_passall_0 | PASSALL=0, RXFLOW=1; PAUSE frame not stored to memory; RXC interrupt set |
| tc_fc_rx_pause_passall_1 | PASSALL=1, RXFLOW=1; PAUSE frame stored to memory + CF bit set + RXC interrupt |
| tc_fc_rx_pause_passall_1_rxflow_0 | PASSALL=1, RXFLOW=0; stored as normal frame, RXB set, no pause |
| tc_fc_rx_pause_both_0 | PASSALL=0, RXFLOW=0; PAUSE frame silently dropped |
| tc_fc_tx_pause_request | Write TXPAUSERQ=1 to TXCTRL with TXPAUSETV; verify PAUSE frame transmitted with correct timer value |
| tc_fc_tx_pause_frame_format | Verify PAUSE frame structure: dest=01-80-c2-00-00-01, type=8808, op=0001, 42B reserved, CRC |
| tc_fc_tx_pause_blocked | TXFLOW=0; write TXPAUSERQ=1; verify no PAUSE frame sent |
| tc_fc_rx_invalid_pause | Receive frame with wrong type/opcode; verify not treated as PAUSE |
| tc_fc_slot_timer | Verify slot timer generates correct pulse intervals (64-byte slot time at 10/100 Mbps) |
| tc_fc_txc_interrupt | TXFLOW=1; send PAUSE frame; verify TXC interrupt in INT_SOURCE |
| tc_fc_rxc_interrupt | RXFLOW=1; receive PAUSE frame; verify RXC interrupt in INT_SOURCE |

Group 7: Half-Duplex / CSMA-CD Tests

| Test | Description |
| --- | --- |
| tc_hd_collision_backoff | FULLD=0; inject collision; verify jam pattern (0x99999999) + binary exponential backoff |
| tc_hd_collision_retry | Inject up to 15 collisions; verify each retry uses wider backoff window; TxRetry signal |
| tc_hd_collision_window | Verify ColWindow = COLLVALID bytes from preamble |
| tc_hd_nobckof | NOBCKOF=1; verify immediate retransmission after collision (no backoff) |
| tc_hd_carrier_sense_defer | MCrS asserted; verify TX defers; DF bit set after eventual success |
| tc_hd_carrier_sense_ifg | Carrier appears within IPGR1 window; verify counter reset |

Group 8: Interrupt System Tests

| Test | Description |
| --- | --- |
| tc_int_mask_all | INT_MASK=0x00; trigger all events; verify INTA_O never asserts |
| tc_int_unmask_txb | INT_MASK.TXB_M=1; transmit frame; verify INTA_O asserts |
| tc_int_unmask_rxb | INT_MASK.RXF_M=1; receive frame; verify INTA_O asserts |
| tc_int_clear_by_write1 | Write 1 to INT_SOURCE bits to clear; verify bits clear and INTA_O deasserts |
| tc_int_busy | Fill all RX BDs; receive new frame; verify BUSY bit in INT_SOURCE |
| tc_int_multiple_sources | Trigger TX + RX simultaneously; verify all bits set, single INTA_O |

Group 9: Reset & Clock Domain Tests

| Test | Description |
| --- | --- |
| tc_rst_hard_reset | Assert RST_I; verify all registers return to reset values; TX/RX halt |
| tc_rst_mid_tx | Assert RST_I during frame transmission; verify graceful abort |
| tc_rst_mid_rx | Assert RST_I during frame reception; verify BD not corrupted |
| tc_rst_bd_preservation | BD RAM holds values after RST_I (per design doc section 3.4.1) |
| tc_clk_10mbps | Set PHY clocks to 2.5MHz (10Mbps); full TX/RX test |
| tc_clk_100mbps | Set PHY clocks to 25MHz (100Mbps); full TX/RX test |
| tc_clk_no_link | MRxClk random 2-40MHz (no link); verify MAC does not lock up |

Group 10: Stress & Corner Case Tests

| Test | Description |
| --- | --- |
| tc_stress_all_bd_tx | Fill all 128 TDs as TX BDs (TX_BD_NUM=0x80); transmit burst |
| tc_stress_all_bd_rx | TX_BD_NUM=0x00; all BDs as RX; receive burst |
| tc_stress_simultaneous_txrx | Full duplex: simultaneous TX burst and RX burst |
| tc_stress_wb_wait_states | Add random wait states to WISHBONE slave; verify no deadlock |
| tc_stress_bd_boundary_addr | Access TX BDs at 0x400, RX BDs ending at 0x7FC; boundary check |
| tc_stress_random_frame_size | Randomize frame sizes from 5 to 1600 bytes; run 1000 frames |
| tc_stress_fifo_depth | Fill TX FIFO to capacity (16 words) then drain; verify no stall |

1- tc_reg_reset_values

After asserting RST_I, every one of the 21 configuration registers must read back its documented reset value as per the specification. Uses the UVM RAL built-in reset sequence so no manual register reads are needed.

Components used

wb_slave_agenteth_reg_block (RAL)uvm_reg_hw_reset_seqeth_coverage

Test flow

1-TestAssert RST_I=1 for 5 clock cycles via wb_slave_agent

2-TestDeassert RST_I=0, wait 2 clock cycles

3-uvm_reg_hw_reset_seq Iterates every register in eth_reg_block; calls reg.read() via frontdoor WISHBONE

4-wb_slave_driver Drives ADDR_I, WE_I=0, STB_I, CYC_I; waits for ACK_O

5-RAL predictor Compares read value against reg.get_reset() for every field

6-eth_coverageSamples reg_reset_cg covergroup — all register names

Pass criteria

MODER reset = 0x0000_A000 (NOPRE, NOBCKOF, FULLD bits check)

IPGT reset = 0x12, IPGR1 = 0x0C, IPGR2 = 0x12

PACKETLEN reset = 0x0040_0600

COLLCONF reset = 0x000F_003F

TX_BD_NUM reset = 0x40

MIIMODER reset = 0x64

All status/interrupt regs (INT_SOURCE, MIISTATUS) reset = 0x0

uvm_reg_hw_reset_seq reports zero mismatches

### uvm_reg_hw_reset_seq

It is a pre-built UVM sequence

Its entire job is one thing:

Read every register in your RAL model via the actual bus (frontdoor), compare each value against the reset value your RAL model declares, and report any mismatch as a UVM error.

### The Actual Source Code (Simplified)

Here is what the sequence actually does internally, stripped to its essence:

systemverilog

class uvm_reg_hw_reset_seq extends uvm_reg_sequence;

uvm_reg_block model;   // you set this — points to your eth_reg_block

task body();

uvm_reg regs[$];          // queue to hold all register handles

uvm_reg_data_t rdata;     // value read from hardware

uvm_reg_data_t exp;       // expected reset value from RAL model

uvm_status_e   status;

// Step 1: collect every register from the block

model.get_registers(regs);

// Step 2: for each register

foreach (regs[i]) begin

// Step 3: skip registers with no reset value defined

if (!regs[i].has_reset()) continue;

// Step 4: read the register from actual hardware

//         this goes through your adapter → WISHBONE bus

regs[i].read(status, rdata, UVM_FRONTDOOR);

// Step 5: get what the reset value SHOULD be

exp = regs[i].get_reset();

// Step 6: compare — only check bits that have reset values

//         (reserved bits are masked out automatically)

if ((rdata & regs[i].get_reset("HARD"))

!== regs[i].get_reset("HARD")) begin

`uvm_error("REG_HW_RESET",

$sformatf("Register '%s' expected 0x%0h after reset, got 0x%0h",

regs[i].get_full_name(), exp, rdata))

end

end

endtask

endclass

For MODER (offset 0x00): CYC_I = 1 STB_I = 1 WE_I = 0 ← read SEL_I = 4'hF ADDR_I = 0x00

DUT responds: ACK_O=1, DATA_O = current register value

adapter captures DATA_O, returns to RAL

RAL compares against 0x0000_A000

2-tc_reg_walking_one

Shifts a single 1-bit through every RW-accessible bit position in every register. For each bit position: write 1 to that bit only, read back, verify; then restore to reset value. Uses uvm_reg_bit_bash_seq.

Components used

wb_slave_agenteth_reg_block (RAL)uvm_reg_bit_bash_seq

Test flow

1uvm_reg_bit_bash_seqFor each RW register: writes all zeros (frontdoor), reads back, checks

2uvm_reg_bit_bash_seqFor each RW bit position N: writes 1 << N, reads back

3wb_slave_driverEach write: drives ADDR_I=reg_offset, DATA_I=value, WE_I=1, STB_I, CYC_I; waits ACK_O

4RAL predictorVerifies read data matches written value; flags W1C or read-only bits that differ

5uvm_reg_bit_bash_seqWrites all ones, reads back; checks RO bits unchanged, RW bits set

Pass criteria

Every RW bit in all 21 registers accepts 0→1 and 1→0 transitions

Read-only bits (MIIRX_DATA[15:0], MIISTATUS[2:0]) never change on write

No ERR_O asserted for valid register addresses with SEL_I=4'hF

ERR_O asserted when SEL_I ≠ 4'hF on any register access

### What uvm_reg_bit_bash_seq Actually Does Internally

The UVM library sequence does more than just walking 1. It runs four sub-patterns on every register. Here is its actual internal logic:

systemverilog

// Inside uvm_reg_bit_bash_seq (UVM library)

task body();

uvm_reg regs[$];

model.get_registers(regs);    // collect all registers

foreach (regs[i]) begin

bash_kth_reg(regs[i]);    // test this register completely

end

endtask

task bash_kth_reg(uvm_reg rg);

int n_bits;

uvm_reg_field fields[$];

uvm_reg_data_t  dc_mask;     // don't-care mask (reserved + RO bits)

uvm_reg_data_t  reset_val;

n_bits    = rg.get_n_bytes() * 8;   // 32 for your registers

reset_val = rg.get_reset();

rg.get_fields(fields);

// Build a mask of bits we should NOT touch or check

// (reserved bits, read-only bits, write-only bits)

dc_mask = 0;

foreach (fields[i]) begin

if (fields[i].get_access() != "RW")

dc_mask |= fields[i].get_mask();

//            ^ set those bit positions in the don't-care mask

end

// ── Pattern 1: Walking 1 ──────────────────────────────

for (int k = 0; k < n_bits; k++) begin

if (dc_mask[k]) continue;   // skip reserved/RO bits

bash_bit(rg, k, 1, reset_val, dc_mask);

//            ↑  ↑

//         bit  value to write

end

// ── Pattern 2: Walking 0 ──────────────────────────────

// First write all 1s to every RW bit

rg.write(status, ~dc_mask);

for (int k = 0; k < n_bits; k++) begin

if (dc_mask[k]) continue;

bash_bit(rg, k, 0, reset_val, dc_mask);

//            ↑  ↑

//         bit  value to write (0 this time)

end

endtask

task bash_bit(uvm_reg    rg,

int         k,          // which bit position

bit         val,        // 1 for walking-1, 0 for walking-0

uvm_reg_data_t reset_v,

uvm_reg_data_t dc_mask);

uvm_reg_data_t  wr_val;

uvm_reg_data_t  rd_val;

uvm_reg_data_t  exp_val;

uvm_status_e    status;

// ── Step A: Build the write value ──

if (val == 1)

wr_val = (1 << k);           // only bit k is 1, all others 0

else

wr_val = ~(1 << k) & ~dc_mask; // only bit k is 0, all RW bits 1

// ── Step B: Write to hardware (frontdoor WISHBONE) ──

rg.write(status, wr_val);

// ── Step C: Read back from hardware (frontdoor WISHBONE) ──

rg.read(status, rd_val);

// ── Step D: Build expected value ──

// For RW bits: expect exactly what you wrote

// For RO/reserved bits: expect their current value (unchanged)

exp_val = (wr_val & ~dc_mask) | (reset_v & dc_mask);

// ── Step E: Compare ──

if ((rd_val & ~dc_mask) !== (exp_val & ~dc_mask)) begin

`uvm_error("REG_BIT_BASH",

$sformatf("Reg '%s' bit %0d: wrote 0x%0h, expected 0x%0h, got 0x%0h",

rg.get_full_name(), k, wr_val, exp_val, rd_val))

end

// ── Step F: Restore register to reset value ──

rg.write(status, reset_v);

endtask

### Walking Through a Concrete Example — MODER Register

MODER is 32 bits wide. Reset value = 0x0000_A000.

Let us map out which bits are RW vs reserved:

Bit 31–17:  Reserved   → dc_mask bits 31–17 = 1  (skip these)

Bit 16:     RECSMALL   → RW  → test this

Bit 15:     PAD        → RW  → test this

Bit 14:     HUGEN      → RW  → test this

Bit 13:     CRCEN      → RW  → test this  (reset=1)

Bit 12:     DLYCRCEN   → RW  → test this

Bit 11:     Reserved   → dc_mask bit 11 = 1  (skip)

Bit 10:     FULLD      → RW  → test this

Bit 9:      EXDFREN    → RW  → test this

Bit 8:      NOBCKOF    → RW  → test this  (reset=1)

Bit 7:      LOOPBCK    → RW  → test this

Bit 6:      IFG        → RW  → test this

Bit 5:      PRO        → RW  → test this

Bit 4:      IAM        → RW  → test this

Bit 3:      BRO        → RW  → test this

Bit 2:      NOPRE      → RW  → test this

Bit 1:      TXEN       → RW  → test this

Bit 0:      RXEN       → RW  → test this

dc_mask = 0xFFFF_0800   (bits 31–17 and bit 11)

Now the sequence walks through 32 bit positions for MODER. Let us trace every interesting one:

#### Bit 0 — RXEN

k = 0

Step A: wr_val = (1 << 0) = 0x0000_0001

Step B: WISHBONE write

ADDR_I = 0x00   (MODER offset)

DATA_I = 0x0000_0001

WE_I   = 1

SEL_I  = 0xF

STB_I  = 1

CYC_I  = 1

→ wait for ACK_O

Step C: WISHBONE read

ADDR_I = 0x00

WE_I   = 0

→ DUT returns DATA_O

Step D: exp_val = (0x0000_0001 & ~0xFFFF0800)

| (0x0000A000 &  0xFFFF0800)

= 0x0000_0001 & 0x0000_F7FF

| 0x0000_A000 & 0xFFFF_0800

= 0x0000_0001

| 0x0000_A000  ← bits 13 and 8 are reset=1, in dc_mask? NO

wait — bits 13,8 are RW, not reserved

dc_mask only covers reserved bits

= 0x0000_0001

(reserved bits stay 0, RW bits we only wrote RXEN=1)

Step E: DUT read returns 0x0000_0001 → PASS ✓

Step F: restore: write 0x0000_A000 (MODER reset value)

Wait — step D needs clarification. The expected value for RW bits is exactly what you wrote. You wrote 0x0000_0001. Only RXEN=1. All other RW bits including CRCEN and NOBCKOF are 0. So you expect to read back 0x0000_0001. The restore in Step F puts MODER back to 0x0000_A000 before the next iteration.

#### Bit 11 — Reserved

k = 11

dc_mask[11] = 1   → continue (skip this bit entirely)

No write. No read. No compare.

The sequence just moves to k=12.

#### Bit 13 — CRCEN (reset = 1)

k = 13

Step A: wr_val = (1 << 13) = 0x0000_2000

Step B: WISHBONE write

ADDR_I = 0x00

DATA_I = 0x0000_2000   ← only CRCEN=1, everything else 0

WE_I   = 1

Step C: WISHBONE read

DUT returns DATA_O = 0x0000_2000

Step D: exp_val = 0x0000_2000   (just CRCEN bit set)

Step E: 0x0000_2000 === 0x0000_2000 → PASS ✓

Step F: restore: write 0x0000_A000

### The Walking-0 Pattern (Part 2 of the Same Sequence)

After all 32 walking-1 iterations for MODER, the sequence runs walking-0. First it writes all 1s to every RW bit:

Write all-RW-ones:

DATA_I = ~dc_mask = ~0xFFFF_0800 = 0x0000_F7FF

MODER now = 0x0000_F7FF

(every RW bit is 1: RECSMALL, PAD, HUGEN, CRCEN, DLYCRCEN,

FULLD, EXDFREN, NOBCKOF, LOOPBCK, IFG, PRO, IAM, BRO,

NOPRE, TXEN, RXEN all = 1)

Then for each RW bit position, it clears that one bit to 0:

Walking-0 for bit 0 (RXEN):

k = 0, val = 0

Step A: wr_val = ~(1 << 0) & ~dc_mask

= 0xFFFF_FFFE & 0x0000_F7FF

= 0x0000_F7FE

(every RW bit = 1 EXCEPT RXEN which is 0)

Step B: write 0x0000_F7FE to MODER

Step C: read back from DUT

Step D: exp_val = 0x0000_F7FE

Step E: compare → PASS ✓

Step F: restore to 0x0000_A000

### Now Let Us See Every WISHBONE Transaction For Just IPGT

IPGT is simpler — only 7 bits, all RW, reset = 0x12 = 0001_0010.

dc_mask = 0xFFFF_FF80   (bits 31–7 are reserved or don't exist)

Walking-1 iterations: 7 total (bits 0–6)

Walking-0 iterations: 7 total (bits 0–6)

Total WISHBONE cycles: 7×(write+read+restore) + setup + 7×(write+read+restore)

= 7×3 + 1 + 7×3 = 43 WISHBONE cycles just for IPGT

Here is every transaction:

WALKING-1 FOR IPGT:

─── Bit 0 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000001  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000001  ACK

compare 0x01 vs 0x01 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 1 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000002  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000002  ACK

compare 0x02 vs 0x02 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 2 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000004  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000004  ACK

compare 0x04 vs 0x04 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 3 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000008  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000008  ACK

compare 0x08 vs 0x08 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 4 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000010  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000010  ACK

compare 0x10 vs 0x10 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 5 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000020  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000020  ACK

compare 0x20 vs 0x20 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 6 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000040  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000040  ACK

compare 0x40 vs 0x40 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

Bits 7–31: all in dc_mask → skipped entirely

WALKING-0 SETUP FOR IPGT:

Write: ADDR=0x0C  DATA=0x0000007F  WE=1  SEL=0xF → ACK

(all 7 RW bits = 1: 0111_1111)

─── Bit 0 (RXEN walking-0) ──────────────────────────────

Write: ADDR=0x0C  DATA=0x0000007E  WE=1  SEL=0xF → ACK

(0111_1111 with bit 0 cleared = 0111_1110)

Read:  ADDR=0x0C  WE=0             → DATA=0x0000007E  ACK

compare 0x7E vs 0x7E → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

... (repeat for bits 1–6)

### What Bugs This Test Catches

Each failure points to a very specific RTL problem:

Failure type                    What it means in RTL

────────────────────────────    ──────────────────────────────────────────

Read back all zeros always      Register not connected to DATA_O path

Wrong address decode in eth_registers.v

Read back all ones always       DATA_O stuck high

Missing output enable

Bit N always reads 0            That specific flip-flop not written

Bit N of DATA_I not connected to FF input

Bit N always reads 1              FF reset value wrong, or bit stuck

Walking-1 PASS but              Register has bit coupling — writing one

walking-0 FAIL on same bit      bit affects another (logic bug in DUT)

Reserved bits change            Reserved bit positions not properly

when RW bits written            masked in eth_registers.v

Value does not restore          Restore write not working

after Step F                     (same as write failure but only on

the reset value pattern)

ERR_O asserts on valid          SEL_I checking too strict, or address

register address                decode bug in eth_wishbone.v

### Total Bus Transactions Across All 21 Registers

Per register:

Walking-1: (number of RW bits) × 3 transactions (write + read + restore)

Walking-0: 1 setup write + (number of RW bits) × 3 transactions

For your 21 registers (approximate RW bit counts):

MODER       15 RW bits → 15×3 + 1 + 15×3 = 91

INT_SOURCE   7 RW bits →  7×3 + 1 +  7×3 = 43

INT_MASK     7 RW bits →  43

IPGT         7 RW bits →  43

IPGR1        7 RW bits →  43

IPGR2        7 RW bits →  43

PACKETLEN   32 RW bits → 32×3 + 1 + 32×3 = 193

COLLCONF    14 RW bits →  85

TX_BD_NUM    8 RW bits →  49

CTRLMODER    3 RW bits →  19

MIIMODER     9 RW bits →  55

MIICOMMAND   3 RW bits →  19

MIIADDRESS  10 RW bits →  61

MIITX_DATA  16 RW bits →  97

MAC_ADDR0   32 RW bits → 193

MAC_ADDR1   16 RW bits →  97

HASH0       32 RW bits → 193

HASH1       32 RW bits → 193

TXCTRL      17 RW bits → 103

──────────────────────────────

Total ≈ 1,663 WISHBONE transactions

At ~3 clock cycles each with no wait states ≈ 5,000 clock cycles

At 100MHz system clock = 50 microseconds of simulation time

This is why uvm_reg_bit_bash_seq is always run early in regression — it is fast, completely automatic, and catches a huge class of RTL connectivity bugs before you even attempt functional tests.

3- tc_bd_walking_one

Exercises all 1024 bytes of the internal BD RAM using uvm_mem_walk_seq. Verifies that the RAM correctly stores and returns data at every address without corruption. BD RAM is only accessible when the MAC is not in reset.

Components used

wb_slave_agenteth_bd_mem (uvm_mem)uvm_mem_walk_seq

Test flow

1TestEnsure MAC is not in reset (RST bit in MODER cleared)

2uvm_mem_walk_seqWrites walking-1 pattern to every 32-bit word from offset 0x400 to 0x7FC

3wb_slave_driverEach write: ADDR_I = 0x400+offset, DATA_I = pattern, WE_I=1, SEL_I=0xF

4uvm_mem_walk_seqReads back every location and compares

5TestAssert RST_I, then read BD RAM — verify values preserved

Pass criteria

All 256 32-bit words (0x400–0x7FC) store and return correct values

No ACK timeout for any BD address within 0x400–0x7FF range

Addresses outside 0x400–0x7FF do not alias into BD RAM

RAM values preserved across soft reset (RST_I) as per spec

READY bit (RD/E) prevents host access once set — verify ERR_O or no effect when written

4- tc_tx_bd_num_split

Writes 0x32 (50) to TX_BD_NUM register and verifies that exactly 50 TX BDs are accessible at 0x400–0x58C and 78 RX BDs are accessible at 0x590–0x7FC. Tests the boundary address for each region.

Components used

wb_slave_agenteth_reg_block (RAL)eth_bd_mem (uvm_mem)eth_tx_bd_num_seq

Test flow

1eth_tx_bd_num_seqWrite TX_BD_NUM = 0x32 via RAL frontdoor

2eth_tx_bd_num_seqWrite all 50 TX BDs: address range 0x400 to 0x58C (8 bytes each × 50 = 0x190)

3eth_tx_bd_num_seqWrite all 78 RX BDs: address range 0x590 to 0x7FC

4eth_tx_bd_num_seqRead back all 128 BDs and verify no overlap or corruption

5TestWrite TX_BD_NUM = 0x80 (all TX): verify RXEN auto-disabled in MODER

6TestWrite TX_BD_NUM = 0x00 (all RX): verify TXEN auto-disabled in MODER

Pass criteria

TX BD at 0x400 (first) and 0x58C (last) correctly stored and read

RX BD at 0x590 (first) and 0x7FC (last) correctly stored and read

RXEN auto-disables when TX_BD_NUM = 0x80

TXEN auto-disables when TX_BD_NUM = 0x00

Values > 0x80 are ignored (register retains previous value)

5- tc_miim_write_phy_reg

Programs a PHY configuration register by setting up MIIADDRESS, MIITX_DATA, then asserting WCTRLDATA in MIICOMMAND. Verifies the full 64-bit MIIM frame appears on the MDC/MDIO interface with correct preamble, address, and data fields.

Components used

wb_slave_agent mdio_agenteth_reg_block (RAL)eth_miim_write_seqmdio_monitoreth_scoreboard

Test flow

1eth_miim_write_seqRAL write: MIIMODER.CLKDIV = 0x28 (MDC = CLK/40)

2eth_miim_write_seqRAL write: MIIADDRESS.FIAD = 0x01 (PHY addr), RGAD = 0x00 (control reg)

3eth_miim_write_seqRAL write: MIITX_DATA.CTRLDATA = 0x2100

4eth_miim_write_seqRAL write: MIICOMMAND.WCTRLDATA = 1

5wb_slave_driverIssues all four WISHBONE write cycles with ACK_O handshake(The driver waits for the DUT to generate ACK_O before starting the next write.)

6mdio_monitorCaptures MDC edges and samples MDIO bit stream

7mdio_monitorDecodes: 32-bit preamble(1s), ST=01, OP=01(write), FIAD, RGAD, TA=10, DATA

8eth_miim_write_seq poll MIISTATUS.BUSY via RAL read until cleared (reg_model.miistatus.read(status, value) performs:Physical bus read ,Receives value from DUT ,Updates mirror value automatically)

9mdio_agentSimulates PHY acknowledging write (drives MDIO during TA with value 2'b10 for write)

10eth_scoreboardVerifies decoded frame fields match programmed values

Pass criteria

MDIO preamble = 32 ones (unless MIINOPRE set)

ST field = 2'b01

OP field = 2'b01 for write

FIAD[4:0] matches MIIADDRESS.FIAD written

RGAD[4:0] matches MIIADDRESS.RGAD written

TA = 2'b10 for write

DATA[15:0] matches MIITX_DATA.CTRLDATA

MIISTATUS.BUSY deasserts at end of operation

6- tc_miim_clkdiv_all

Iterates through all valid CLKDIV values (1–255) and for each value triggers a MIIM read operation, measures the MDC frequency, and verifies it equals CLK/(2×CLKDIV).

Components used

wb_slave_agent mdio_agenteth_reg_block (RAL)eth_miim_clkdiv_seq

Test flow

1eth_miim_clkdiv_seq For CLKDIV = 1 to 255: RAL write to MIIMODER.CLKDIV

2eth_miim_clkdiv_seq Trigger MIICOMMAND.RSTAT = 1 to start a read operation

3mdio_monitor Measures MDC period: records rising edge timestamps, computes frequency

4mdio_monitor Verifies MDC_freq = SYS_CLK_freq / (2 × CLKDIV) ± 1 cycle tolerance

5eth_miim_clkdiv_seqWait for BUSY = 0 before next iteration

Pass criteria

MDC frequency matches formula for all 255 values

No MDC glitches (spurious edges) between operations

BUSY correctly asserted for full duration of each read

CLKDIV=0 treated as 1 or ignored (per implementation)

We need read operation as MDC does not run continuously. It only toggles when the MIIM module is actively performing an operation.

MDC is a divided version of CLK. The counter counts CLKDIV system clock cycles, then toggles MDC. One full MDC period = 2 × CLKDIV system clock cycles.

7- tc_miim_scan_linkfail

Asserts SCANSTAT in MIICOMMAND to start continuous PHY status polling. The mdio_agent (PHY model) first returns link-OK, then drives LINKFAIL=1 in the status register. Verifies MIISTATUS.LINKFAIL reflects the PHY state and NVALID clears after first scan completes.

Components used

wb_slave_agent mdio_agenteth_reg_block (RAL)eth_miim_scan_seq

Test flow

1eth_miim_scan_seqWrite MIICOMMAND.SCANSTAT = 1; MIIADDRESS.FIAD = PHY_ADDR

2mdio_agentFirst scan response: bit[2]=1 (link OK) in PHY status register

3eth_miim_scan_seqRAL read MIISTATUS: verify NVALID=1 (invalid until first scan done)

4mdio_monitorDetects end of first scan frame; NVALID should clear

5eth_miim_scan_seqRAL read MIISTATUS: verify NVALID=0, LINKFAIL=0

6mdio_agentSecond scan response: bit[2]=0 (link fail) in PHY status register

7eth_miim_scan_seqRAL read MIISTATUS: verify LINKFAIL=1

8eth_miim_scan_seqWrite MIICOMMAND = 0x00 to stop scan; verify BUSY deasserts

Pass criteria

NVALID=1 until first complete scan frame received

NVALID=0 after first scan completes

LINKFAIL tracks PHY bit[2] of status register

BUSY stays asserted during entire scan operation

BUSY deasserts only after scan explicitly stopped

8- tc_miim_stop_scan_after_read

Asserts SCANSTAT in MIICOMMAND to start continuous PHY status polling, then performs a sliding stop by clearing SCANSTAT immediately after scan begins. The mdio_agent (PHY model) responds to scan reads with changing link status (LINK OK → LINK FAIL). Verifies that scanning stops cleanly without initiating further MII transactions after SCANSTAT is cleared, for both MIINOPRE = 0 and MIINOPRE = 1 modes.

COMPONENTS USED

wb_slave_agent mdio_agenteth_reg_block (RAL) eth_miim_stop_scan_seq  mdio_monitoreth_scoreboard

TEST FLOW

| 1 | eth_miim_stop_scan_seq | RAL write: MIIMODER.CLKDIV = 0x28 (set MDC to measurable frequency), MIIMODER.MIINOPRE = 0 (preamble enabled for first pass) |
| --- | --- | --- |
| 2 | eth_miim_stop_scan_seq | RAL write: MIIADDRESS = 0x0101 (FIAD=1, RGAD=1 — PHY status register) |
| 3 | eth_miim_stop_scan_seq | RAL write: MIICOMMAND = 0x00000001 (SCANSTAT=1 — start continuous scan) |
| 4 | mdio_monitor | Detects first MDC rising edge — records scan start timestamp; confirms MDC frequency matches CLKDIV formula |
| 5 | mdio_agent | Responds to scan frame: drives TA acknowledge (MDIO=0 on second TA bit), drives 16-bit PHY status = 0x0024 (link OK, bit[2]=1) |
| 6 | eth_miim_stop_scan_seq | Waits for first MDC rising edge after SCANSTAT was written using @(posedge mdio_vif.MDC) — this marks MDC cycle 0 of frame 1 |
| 7 | eth_miim_stop_scan_seq | Outer loop: preamble ∈ {enabled, disabled} — for disabled preamble, repeat steps 1–6 with MIINOPRE=1 and frame_len=32 |
| 8 | eth_miim_stop_scan_seq | Inner loop: stop_at_cycle from 0 to frame_len (0 to 64 with preamble, 0 to 32 without) — each iteration: (a) write MIICOMMAND=0x01 to restart scan, (b) wait exactly stop_at_cycle MDC rising edges using repeat(stop_at_cycle) @(posedge mdio_vif.MDC), (c) write MIICOMMAND=0x00 (clear SCANSTAT) |
| 9 | mdio_monitor | For each iteration: records timestamp of last MDC edge before stop; verifies no further MDC edges appear within 10 CLK cycles after stop command |
| 10 | eth_miim_stop_scan_seq | RAL read MIISTATUS in a polling loop: waits for BUSY=0 with timeout of 50,000 CLK cycles; if timeout fires, records the stop_at_cycle value that caused the hang |
| 11 | mdio_monitor | Verifies MDIO line returns to high-Z state within 2 MDC cycles of SCANSTAT being cleared; no spurious edges on MDIO after stop |
| 12 | eth_miim_stop_scan_seq | For stop positions where stop_at_cycle < frame_len: verifies exactly 0 complete scan frames were delivered to PHY (partial frame was in progress, aborted) |
| 13 | eth_miim_stop_scan_seq | For stop_at_cycle = frame_len (exactly at end): verifies exactly 1 complete scan frame was delivered; DUT completed current frame before stopping |
| 14 | eth_miim_stop_scan_seq | Recovery check: after BUSY=0, write MIICOMMAND.RSTAT=1 (single read), verify it completes normally — confirms MIIM state machine returned to idle cleanly |
| 15 | eth_scoreboard | After all iterations: verifies zero cases where BUSY failed to clear; zero cases where MDC continued after stop; zero cases where MDIO was not released to high-Z |

Pass criteria

BUSY deasserts within 128 × CLKDIV CLK cycles after SCANSTAT cleared for every stop position

MDC stops toggling within 2 CLK cycles of SCANSTAT being cleared at every stop position

MDIO returns to high-Z within 2 MDC cycles after scan stops at every stop position

No MDC edges appear after stop for at least 100 CLK cycles (confirms MDC truly stopped, not just paused)

Recovery single-read operation (RSTAT) completes correctly after every stop position — confirms state machine returns to idle cleanly

With preamble disabled (MIINOPRE=1): all above hold with frame_len=32 instead of 64

Zero simulation timeouts across all 65 + 33 = 98 iterations total (64+1 with preamble, 32+1 without)

9- tc_miim_stop_scan_after_write

Stop scan immediately after a write request is issued; sliding stop position across the write frame while both WCTRLDATA and SCANSTAT are simultaneously set.

COMPONENTS USED

wb_slave_agent mdio_agenteth_reg_block (RAL) eth_miim_stop_scan_seq  mdio_monitoreth_scoreboard

TEST FLOW

| 1 | eth_miim_stop_scan_seq | RAL write: MIIMODER.CLKDIV = 0x28, MIIMODER.MIINOPRE = 0 (preamble enabled for first pass) |
| --- | --- | --- |
| 2 | eth_miim_stop_scan_seq | RAL write: MIIADDRESS = 0x0101 (FIAD=1, RGAD=1) |
| 3 | eth_miim_stop_scan_seq | RAL write: MIITX_DATA = 0x00002100 (PHY control register value: reset + loopback) — this is the data the write frame will carry |
| 4 | eth_miim_stop_scan_seq | RAL write: MIICOMMAND = 0x00000001 (SCANSTAT=1 only — start scan) |
| 5 | mdio_monitor | Confirms scan is running: detects MDC toggling, records scan_active = 1 |
| 6 | mdio_agent | Responds to first scan frame with PHY status = 0x0024 (link OK) to confirm scan is operational before the test begins |
| 7 | eth_miim_stop_scan_seq | Waits for exactly one complete scan frame to confirm scan is stably running: repeat(frame_len) @(posedge mdio_vif.MDC) |
| 8 | eth_miim_stop_scan_seq | RAL write: MIICOMMAND = 0x00000005 (WCTRLDATA=1 AND SCANSTAT=1 simultaneously — both bits set at the same time) — this is the critical moment: a write command is injected while scan is running |
| 9 | mdio_monitor | Captures the MIICOMMAND write timestamp; begins monitoring MDIO to detect whether DUT starts a write frame or continues scanning |
| 10 | eth_miim_stop_scan_seq | Outer loop: preamble {enabled, disabled} |
| 11 | eth_miim_stop_scan_seq | Inner loop: stop_at_cycle from 0 to frame_len — for each iteration: (a) restore to step 8 state (scan running + WCTRLDATA pending), (b) wait stop_at_cycle MDC edges, (c) write MIICOMMAND = 0x00000004 (clear SCANSTAT, keep WCTRLDATA=1) |
| 12 | mdio_monitor | At each iteration: decodes the MDIO bit stream; determines whether a WRITE frame or a SCAN frame is currently in progress; records frame type and completion status |
| 13 | mdio_agent | For any write frame detected: drives TA field correctly (MDIO=high-Z then MDIO=0); does NOT drive data response (write frames have host driving the data, not PHY) |
| 14 | eth_miim_stop_scan_seq | After clearing SCANSTAT: polls MIISTATUS.BUSY until cleared with 50,000-cycle timeout |
| 15 | eth_scoreboard | Verifies write frame integrity: decodes ST=01, OP=01, FIAD=00001, RGAD=00001, TA=10, DATA=0x2100 from the captured MDIO bit stream; any bit mismatch flagged as write frame corruption |
| 16 | eth_scoreboard | Verifies exactly one complete write frame appears on MDIO regardless of when SCANSTAT was cleared; the write must not be silently dropped |
| 17 | eth_miim_stop_scan_seq | Verifies scan does not restart after write completes: monitors MDC for 200 CLK cycles after BUSY clears — no new MDC edges should appear since SCANSTAT was cleared |
| 18 | eth_miim_stop_scan_seq | Recovery check: write MIICOMMAND.RSTAT=1, verify single read completes normally — confirms state machine is in idle, not stuck in write or scan state |

Pass criteria

Write frame appears on MDIO and completes correctly for every value of stop_at_cycle (0 to 64 with preamble, 0 to 32 without) — write is never silently dropped regardless of when SCANSTAT is cleared

Write frame content is bit-perfect: ST=01, OP=01 (write), FIAD matches MIIADDRESS.FIAD, RGAD matches MIIADDRESS.RGAD, TA=10, DATA=0x2100 for all sliding positions

Scan does not restart after write frame completes (SCANSTAT was cleared)

BUSY deasserts after write frame completes, not prematurely after scan stops

No overlapping scan and write frames on MDIO (no frame interleaving or corruption)

With preamble disabled: write frame = 32 MDC cycles; all above still hold

Recovery read operation completes correctly after every stop position

10-tc_miim_stop_scan_after_scan

Stop scan at the exact boundary between the 1st and 2nd completed scan frames, and between the 2nd and 3rd, using a narrow sliding window centered on each frame boundary.

COMPONENTS USED

wb_slave_agent mdio_agenteth_reg_block (RAL) eth_miim_stop_scan_seq  mdio_monitoreth_scoreboard

Test flow

| 1 | eth_miim_stop_scan_seq | RAL write: MIIMODER.CLKDIV = 0x28, MIIMODER.MIINOPRE = 0 |
| --- | --- | --- |
| 2 | eth_miim_stop_scan_seq | RAL write: MIIADDRESS = 0x0101 (FIAD=1, RGAD=1 — PHY status register) |
| 3 | eth_miim_stop_scan_seq | RAL write: MIICOMMAND = 0x00000001 (SCANSTAT=1 — start continuous scan) |
| 4 | mdio_monitor | Detects first MDC rising edge; begins counting MDC edges from this point as the absolute frame counter reference total_mdc_count |
| 5 | mdio_agent | For every scan frame that arrives: drives TA acknowledge then drives PHY status = 0x0024 (link OK, LINKFAIL=0) for all frames; this ensures scan runs cleanly during the sliding window measurement |
| 6 | eth_miim_stop_scan_seq | Outer loop: preamble ∈ {enabled, disabled} — frame_len = 64 with preamble, frame_len = 32 without |
| 7 | eth_miim_stop_scan_seq | Sub-test A — boundary between frame 1 and frame 2: inner loop with stop_offset from -5 to +5 MDC cycles relative to frame_len (absolute stop cycle = frame_len + stop_offset, range 59–69 with preamble, 27–37 without) |
| 8 | eth_miim_stop_scan_seq | For each stop_offset iteration of Sub-test A: (a) write MIICOMMAND=0x01 to restart fresh scan, (b) @(posedge mdio_vif.MDC) to sync to first edge, (c) repeat(frame_len + stop_offset) @(posedge mdio_vif.MDC) to reach stop position, (d) write MIICOMMAND=0x00 to clear SCANSTAT |
| 9 | mdio_monitor | For each Sub-test A iteration: counts total complete scan frames that appeared on MDIO before MDC stopped; records n_complete_frames |
| 10 | eth_miim_stop_scan_seq | For Sub-test A with stop_offset ≤ 0 (stop at or before boundary): verifies n_complete_frames == 1; DUT stopped after frame 1, frame 2 never started |
| 11 | eth_miim_stop_scan_seq | For Sub-test A with stop_offset > 0 (stop after boundary): verifies n_complete_frames == 2; DUT committed to frame 2 before seeing the stop, must complete it |
| 12 | eth_miim_stop_scan_seq | For all Sub-test A iterations: polls MIISTATUS.BUSY until cleared; verifies BUSY clears correctly regardless of stop_offset; no timeout |
| 13 | mdio_monitor | For all Sub-test A iterations: verifies no partial frames appear — every frame that starts must complete fully; if frame 2 started it must be 64 complete MDC cycles (not 60 or 70) |
| 14 | eth_miim_stop_scan_seq | Sub-test B — boundary between frame 2 and frame 3: inner loop with stop_offset from -5 to +5, absolute stop cycle = (2 × frame_len) + stop_offset (range 123–133 with preamble, 59–69 without) |
| 15 | eth_miim_stop_scan_seq | For each Sub-test B iteration: same procedure as steps 8–13 but waiting 2×frame_len + stop_offset MDC edges before clearing SCANSTAT |
| 16 | mdio_monitor | For Sub-test B with stop_offset ≤ 0: verifies n_complete_frames == 2; for stop_offset > 0: verifies n_complete_frames == 3 |
| 17 | eth_miim_stop_scan_seq | After Sub-test B completes: verifies NVALID=0 in MIISTATUS (at least one full scan completed so data is valid) and LINKFAIL=0 (PHY returned link OK on all frames during test) |
| 18 | eth_miim_stop_scan_seq | Recovery check for both sub-tests: after each BUSY=0, write MIICOMMAND.RSTAT=1, verify single read completes; confirms state machine returned to idle and did not get stuck at frame boundary |
| 19 | eth_scoreboard | Final check: total frames observed = sum of all n_complete_frames across both sub-tests and both preamble modes; verifies count matches expected value of (1+2)×11 + (2+3)×11 = 33+55 = 88 complete frames across all iterations |

Pass criteria

- Sub-test A with stop_offset ≤ 0: exactly 1 complete scan frame transmitted in all 6 cases (offsets -5,-4,-3,-2,-1,0)

- Sub-test A with stop_offset > 0: exactly 2 complete scan frames transmitted in all 5 cases (offsets +1,+2,+3,+4,+5)

- Sub-test B with stop_offset ≤ 0: exactly 2 complete scan frames transmitted in all 6 cases

- Sub-test B with stop_offset > 0: exactly 3 complete scan frames transmitted in all 5 cases

- No partial frames on MDIO at any stop position — every frame that starts must be exactly frame_len MDC cycles long

- BUSY deasserts after last complete frame finishes for all 44 iterations across both sub-tests

- NVALID=0 after test (at least 1 complete scan confirmed valid data)

- LINKFAIL=0 throughout (mdio_agent drove link-OK on every frame)

- Recovery single read completes correctly after every iteration

- With preamble disabled (frame_len=32): all frame count checks hold with boundary at cycle 32 and 64 instead of 64 and 128

- Zero simulation timeouts across all 44 iterations × 2 preamble modes = 88 total iterations

11- tc_tx_basic_frame

Transmits a single 64-byte Ethernet frame. The wb_master_agent holds the frame bytes in its SRAM (associative array). A TX BD is programmed pointing to that SRAM address. After TXEN=1, the DUT reads the frame via WISHBONE master and transmits nibbles on MTxD. The mii_agent captures and reassembles the full frame.

Components used

wb_slave_agent wb_master_agent mii_agenteth_reg_block (RAL)eth_bd_mem (uvm_mem)eth_tx_frame_seqeth_scoreboardeth_coverage

Test flow

1eth_tx_frame_seqWrite 64 bytes of frame data into wb_master_agent SRAM at address 0x0000_1000 (backdoor write to SRAM model)

2eth_tx_frame_seqRAL write MODER: TXEN=0, CRCEN=1, PAD=1, FULLD=0 (configure before enable)

3eth_tx_frame_seqProgram TX BD at 0x400: LEN=64, CRC=1, PAD=1, IRQ=1, WR=1, RD=0

4eth_tx_frame_seqWrite TXPNT at 0x404 = 0x0000_1000

5eth_tx_frame_seqSet TX BD RD=1 (ready bit) — this arms the transmitter

6eth_tx_frame_seqRAL write MODER.TXEN = 1

7DUTSees RD=1, issues WISHBONE master reads: M_ADDR_O=0x1000, M_WE_O=0

8wb_master_agentResponds to each read burst with frame bytes from SRAM, asserts M_ACK_I

9DUTFills TX FIFO (16 words), begins transmitting: MTxEn=1, MTxD = preamble nibbles

10mii_monitorCaptures MTxD stream: 14 nibbles preamble (0x5), 2 nibbles SFD (0xD), then data nibbles

11DUTTransmits data nibbles (LSB first), then 4-byte CRC nibbles; deasserts MTxEn

12DUTWrites TX BD: RD=0, RTRY=0, DF=0, CS=0; asserts INTA_O (IRQ=1)

13eth_scoreboardReconstructs frame from captured MII nibbles; compares against original SRAM bytes

14eth_scoreboardRecalculates expected CRC-32; verifies appended CRC matches

Pass criteria

Preamble = 7 bytes of 0x55, SFD = 0xD5

Frame payload matches exactly what was written to SRAM

CRC-32 appended and mathematically correct

MTxEn deasserts on same cycle as last CRC nibble

TX BD RD bit cleared to 0 after transmission

INTA_O asserts and INT_SOURCE.TXB=1

M_WE_O=0 for all DMA reads (read-only on TX path)

12-tc_tx_short_frame_pad on and off

Transmits a 20-byte payload (below 64-byte MINFL). Runs twice: once with PAD=1 (should pad to 64 bytes then append CRC), once with PAD=0 (transmits as-is with or without CRC based on CRCEN).

Components used

wb_slave_agent wb_master_agent mii_agenteth_reg_block (RAL)eth_tx_frame_seqeth_scoreboard

Test flow

1eth_tx_frame_seqWrite 20-byte payload to SRAM at 0x0000_2000

2eth_tx_frame_seqRun A: MODER.PAD=1, CRCEN=1; program TX BD LEN=20, RD=1

3DUTTransmits 20 data bytes, then zero-pads to 60 bytes (MINFL=64 minus 4-byte CRC), then appends 4-byte CRC

4mii_monitorCaptures total frame length = 64 bytes (preamble/SFD excluded)

5eth_tx_frame_seqRun B: MODER.PAD=0, CRCEN=1; same TX BD

6DUTTransmits 20 data bytes, then 4-byte CRC (no padding) — total payload 24 bytes

7eth_scoreboardVerifies frame lengths match expected for each run

Pass criteria

Run A: transmitted frame = 64 bytes total (20 data + 40 zeros pad + 4 CRC)

Run A: CRC computed over padded frame (64 bytes)

Run B: transmitted frame = 24 bytes (20 data + 4 CRC, no pad)

TX BD status bits correct (no error flags) in both runs

13- tc_tx_underrun

Forces the wb_master_agent to insert random WAIT states (withhold M_ACK_I) so the TX FIFO drains empty while transmission is in progress. Verifies the DUT aborts transmission and sets the UR bit in the TX BD.

Components used

wb_slave_agentwb_master_agentmii_agenteth_reg_block (RAL)eth_tx_underrun_seqeth_scoreboard

Test flow

1eth_tx_underrun_seqProgram 512-byte frame in SRAM; set TX BD RD=1, IRQ=1

2eth_tx_underrun_seqConfigure wb_master_agent to hold M_ACK_I low for 500+ cycles after 4th read

3DUTStarts TX, reads first 4 words into FIFO, begins transmitting

4DUTFIFO empties while waiting for M_ACK_I; TxAbort asserted internally

5mii_monitorRecords MTxEn deasserting mid-frame (incomplete transmission)

6DUTWrites TX BD: RD=0, UR=1; asserts INTA_O

7eth_tx_underrun_seqRAL read INT_SOURCE: verify TXE=1

8eth_tx_underrun_seqRAL read TX BD status word: verify UR bit=1

Pass criteria

TX BD UR bit set to 1

INT_SOURCE.TXE = 1 (transmit error interrupt)

MTxEn deasserts cleanly (no partial nibble)

TX BD RD cleared to 0

No subsequent frame corruption — next TX BD works correctly after recovery

14- tc_tx_collision_retry

With MODER.FULLD=0, the mii_agent asserts MColl asynchronously during frame transmission. Verifies the DUT sends a jam pattern, performs binary exponential backoff, and retries. Repeats up to MAXRET times.

Components used

wb_slave_agent wb_master_agent mii_agenteth_reg_block (RAL)eth_collision_seqeth_scoreboard

Test flow

1eth_collision_seqMODER.FULLD=0, COLLCONF.MAXRET=15, COLLCONF.COLLVALID=0x3F

2eth_collision_seqProgram TX BD with 128-byte frame, RD=1

3eth_collision_seqEnable TXEN, wait until MTxEn=1 and 10 nibbles transmitted

4mii_agentAssert MColl asynchronously (simulates collision on wire)

5mii_monitorCaptures jam pattern: 4 bytes 0x99999999 on MTxD

6DUTStops TX, waits random backoff (eth_random.v: binary exponential)

7mii_monitorMeasures backoff time = random × slot_time

8DUTRetransmits from beginning of frame (preamble again)

9eth_collision_seqLet retry succeed (no MColl); verify TxDone and RTRY count in TX BD

Pass criteria

Jam pattern = 0x99999999 (4 bytes) immediately after collision

Backoff time is random within [0, 2^N - 1] slot times for Nth collision

RTRY field in TX BD = number of collisions before success

TX BD RL=1 and abort if MAXRET+1 collisions injected

WillTransmit signal deasserts during backoff (RX can proceed)