Ethernet IP Core verification environment

1.Top-Level UVM Architecture

### 2. Interface Definitions

We need four SystemVerilog interfaces:

| Interface | Signals |
| --- | --- |
| wb_if (slave) | CLK_I, RST_I, ADDR_I, DATA_I/O, SEL_I, WE_I, STB_I, CYC_I, ACK_O, ERR_O, INTA_O |
| wb_if (master) | M_ADDR_O, M_DATA_I/O, M_SEL_O, M_WE_O, M_STB_O, M_CYC_O, M_ACK_I, M_ERR_I |
| mii_if | MTxClk, MTxD[3:0], MTxEn, MTxErr, MRxClk, MRxD[3:0], MRxDV, MRxErr, MColl, MCrS |
| mdio_if | MDC, MDIO (bidirectional), Mdi, Mdo, MdoEn |

#### The Slave Interface — Configuration Port

This is how a processor (or testbench) configures the MAC. The DUT is the slave here

Used for:

- Writing configuration registers (MODER, PACKETLEN, COLLCONF, etc.)

- Reading status registers (INT_SOURCE, MIISTATUS, etc.)

- Reading/writing Buffer Descriptors in internal RAM (0x400–0x7FF)

The DUT receives commands on this interface. It never initiates anything here.

Signals on this interface (from spec Table 1):

INPUT into DUT (driven by testbench):          OUTPUT from DUT:

─────────────────────────────────────

CLK_I    — clock                                           ACK_O  — "I'm done, transaction OK"

RST_I    — reset                                            ERR_O  — "bad access (e.g. SEL≠1111)"

ADDR_I   — register/BD address                 INTA_O — interrupt to processor

DATA_I   — data to write                              DATA_O   — data read back

SEL_I    — byte enables (must be 1111)

WE_I     — 1=write, 0=read

STB_I    — "this transfer is valid"

CYC_I    — "a bus cycle is in progress"

#### The Master Interface — DMA Port

This is how the MAC autonomously moves frame data to/from system memory. The DUT is the master here — it initiates transactions without being asked.

Used for:

- TX path: DUT reads frame bytes from memory to fill its TX FIFO

- RX path: DUT writes received frame bytes into memory from its RX FIFO

The DUT initiates all transactions on this interface. testbench must respond as a slave.

Signals on this interface (from spec Table 1):

OUTPUT from DUT (DUT initiates):               INPUT into DUT (testbench responds):

────────────────────────────────────

M_ADDR_O  — memory address to access          M_ACK_I  — "memory access done"

M_DATA_O  — data to write to memory           M_ERR_I  — "memory access error"

M_SEL_O   — byte enables                                 M_DATA_I  — data read from memory

M_WE_O    — 1=write(RX), 0=read(TX)

M_STB_O   — "this transfer is valid"

M_CYC_O   — "a bus cycle is in progress"

────────────────────────────────────

wb_slave_agent

└── Connects to: WISHBONE SLAVE interface of DUT

└── Driven by:   testbench (acts as the RISC processor)

└── Purpose:     Write registers, set up BDs, read status, clear interrupts

└── Signals:     CLK_I, RST_I, ADDR_I, DATA_I, DATA_O, SEL_I,

WE_I, STB_I, CYC_I, ACK_O, ERR_O, INTA_O

wb_master_agent

└── Connects to: WISHBONE MASTER interface of DUT

└── Driven by:  testbench (acts as the memory/SRAM)

└── Purpose:     Respond to DMA reads (provide TX frame data) and

DMA writes (accept RX frame data into its SRAM model)

└── Signals:     M_ADDR_O, M_DATA_O, M_DATA_I, M_SEL_O, M_WE_O,

M_STB_O, M_CYC_O, M_ACK_I, M_ERR_I

mii_agent

└── Connects to: MII interface of DUT

└── Driven by:   Your testbench (acts as the Ethernet PHY chip)

└── Purpose:     Inject RX frames (drive MRxD nibbles + MRxDV),

capture TX frames (sample MTxD + MTxEn),

inject collisions (MColl) and carrier sense (MCrS)

└── Signals:     MTxClk, MTxD[3:0], MTxEn, MTxErr,

MRxClk, MRxD[3:0], MRxDV, MRxErr, MColl, MCrS

mdio_agent

└── Connects to: MDC/MDIO interface of DUT

└── Driven by:  testbench (acts as the PHY's management registers)

└── Purpose:     Respond to PHY register reads/writes initiated by DUT,

verify correct MIIM frame format on MDIO

└── Signals:     MDC (input, DUT drives), MDIO (bidirectional)

3. RAL Model — eth_reg_block

class eth_reg_block extends uvm_reg_block;

// All 21 registers

rand eth_moder_reg        MODER;        // 0x00

rand eth_int_source_reg   INT_SOURCE;   // 0x04

rand eth_int_mask_reg     INT_MASK;     // 0x08

rand eth_ipgt_reg         IPGT;         // 0x0C

rand eth_ipgr1_reg        IPGR1;        // 0x10

rand eth_ipgr2_reg        IPGR2;        // 0x14

rand eth_packetlen_reg    PACKETLEN;    // 0x18

rand eth_collconf_reg     COLLCONF;     // 0x1C

rand eth_tx_bd_num_reg    TX_BD_NUM;    // 0x20

rand eth_ctrlmoder_reg    CTRLMODER;    // 0x24

rand eth_miimoder_reg     MIIMODER;     // 0x28

rand eth_miicommand_reg   MIICOMMAND;   // 0x2C

rand eth_miiaddress_reg   MIIADDRESS;   // 0x30

rand eth_miitx_data_reg   MIITX_DATA;  // 0x34

rand eth_miirx_data_reg   MIIRX_DATA;  // 0x38  (R only)

rand eth_miistatus_reg    MIISTATUS;   // 0x3C  (R only)

rand eth_mac_addr0_reg    MAC_ADDR0;   // 0x40

rand eth_mac_addr1_reg    MAC_ADDR1;   // 0x44

rand eth_hash0_reg        HASH0;        // 0x48

rand eth_hash1_reg        HASH1;        // 0x4C

rand eth_txctrl_reg       TXCTRL;       // 0x50

// BD memory: 0x400–0x7FF

uvm_mem eth_bd_mem;  // 256 × 32-bit words

uvm_reg_map reg_map;

endclass

Each register class models its named fields. For example, MODER has fields: RECSMALL, PAD, HUGEN, CRCEN, DLYCRCEN, FULLD, EXDFREN, NOBCKOF, LOOPBCK, IFG, PRO, IAM, BRO, NOPRE, TXEN, RXEN — each with correct access type and reset value from the spec.

4. Complete Test Plan

Group 1: Register & BD Access Tests

| Test | Description | RAL Method |
| --- | --- | --- |
| tc_reg_reset_values | Verify all 21 registers match spec reset values after RST_I | uvm_reg_hw_reset_seq |
| tc_reg_walking_one | Walking-1 across all RW bits, single WISHBONE cycles | uvm_reg_bit_bash_seq |
| tc_reg_walking_zero | Walking-0 across all RW bits | uvm_reg_bit_bash_seq |
| tc_reg_max_values | Write max value, read back, hard reset, verify | Custom seq |
| tc_reg_ro_fields | Verify R-only fields (MIIRX_DATA, MIISTATUS bits) reject writes | Custom seq |
| tc_bd_walking_one | Walking-1 across BD RAM (0x400–0x7FF) | uvm_mem_walk_seq |
| tc_bd_reset_preservation | BD RAM holds values after soft reset (RST bit), clears after hard reset | Custom seq |
| tc_bd_boundary | Access first BD (0x400), last BD (0x7FC), and out-of-range | uvm_mem_access_seq |
| tc_tx_bd_num_split | Write TX_BD_NUM=0x32 → verify 50 TX BDs and 78 RX BDs are accessible | Custom seq |
| tc_bd_access_during_reset | Attempt BD access while RST bit set → expect no access | Custom seq |

Group 2: MIIM (MII Management) Tests

| Test | Description |
| --- | --- |
| tc_miim_clkdiv_all | Test all 255 clock divider values in MIIMODER.CLKDIV; verify MDC frequency = CLK/(2×CLKDIV) |
| tc_miim_write_phy_reg | Write to PHY control register (addr 0x0) via MIICOMMAND.WCTRLDATA; verify MDIO frame format |
| tc_miim_read_phy_reg | Read from PHY status register (addr 0x1); verify PRSD in MIIRX_DATA |
| tc_miim_read_nonwritable | Attempt write to read-only PHY register; verify PHY ignores it |
| tc_miim_reset_phy | Assert PHY reset through MIIM write; verify PHY responds |
| tc_miim_walk_phy_addr | Walking-1 across FIAD[4:0] with and without preamble |
| tc_miim_walk_reg_addr | Walking-1 across RGAD[4:0] with and without preamble |
| tc_miim_walk_data | Walking-1 across 16-bit write data with and without preamble |
| tc_miim_wrong_phy_addr | Read from non-existent PHY address → MDIO high-Z, verify MIIRX_DATA undefined |
| tc_miim_wrong_write_correct_read | Write to wrong PHY addr, read from correct addr; verify no corruption |
| tc_miim_busy_nvalid_write | Verify BUSY asserts during write, deasserts at end; NVALID behavior |
| tc_miim_busy_nvalid_read | Verify BUSY/NVALID durations during read operation |
| tc_miim_busy_nvalid_scan | Verify BUSY stays asserted during continuous scan; NVALID clears after first scan |
| tc_miim_scan_linkfail | Scan PHY status; detect LINKFAIL asserted/deasserted; verify MIISTATUS.LINKFAIL |
| tc_miim_scan_linkfail_sliding | Sliding LINKFAIL bit during scan — verify edge detection |
| tc_miim_stop_scan_after_read | Assert SCANSTAT then immediately clear; sliding stop with/without preamble |
| tc_miim_stop_scan_after_write | Stop scan immediately after write request; sliding |
| tc_miim_stop_scan_after_scan | Stop scan after 1st and 2nd scan iterations; sliding |
| tc_miim_no_preamble | Set MIIMODER.MIINOPRE=1; verify 32-bit preamble absent on MDIO |

Group 3: Frame Transmission Tests

| Test | Description |
| --- | --- |
| tc_tx_basic_frame | Transmit single minimum-length frame (64B); verify preamble + SFD + data + CRC on MII |
| tc_tx_max_frame | Transmit maximum-length frame (1518B); verify no truncation |
| tc_tx_short_frame_pad_on | Frame < MINFL with PAD=1; verify padded to 64B with zeros + CRC |
| tc_tx_short_frame_pad_off | Frame < MINFL with PAD=0; verify transmitted as-is (no padding) |
| tc_tx_crc_enable | CRCEN=1; verify 4-byte CRC appended to frame |
| tc_tx_crc_disable | CRCEN=0; verify no CRC appended |
| tc_tx_delayed_crc | DLYCRCEN=1; CRC calculation starts 4 bytes after SFD |
| tc_tx_huge_frame | HUGEN=1; transmit frame > 1518B; verify transmission completes |
| tc_tx_huge_disabled | HUGEN=0; transmit frame > MAXFL; verify abort at max |
| tc_tx_no_preamble | NOPRE=1 in MODER; verify 7-byte preamble suppressed |
| tc_tx_ipg_back_to_back | Transmit back-to-back frames; verify IPG ≥ 0.96µs (IPGT register value) |
| tc_tx_ipg_non_back_to_back | Verify IPGR1 and IPGR2 windows respected |
| tc_tx_multi_bd_chain | Chain 8 TX BDs (WR=0 until last); verify continuous transmission |
| tc_tx_wrap_bit | Last BD has WR=1; verify circular wrap to first BD |
| tc_tx_underrun | Throttle WISHBONE master to starve TX FIFO; verify TxAbort + UR bit in TX BD |
| tc_tx_done_interrupt | IRQ=1 in TX BD; verify TXB interrupt in INT_SOURCE after transmission |
| tc_tx_error_interrupt | Force underrun with IRQ=1; verify TXE interrupt |
| tc_tx_retry_count | In half-duplex: inject collision; verify RTRY field incremented in TX BD |
| tc_tx_max_retry | Inject MAX_RET+1 collisions; verify RL bit set, transmission aborted |
| tc_tx_late_collision | Inject collision after COLLVALID window; verify LC bit + abort |
| tc_tx_excessive_deferral | EXDFREN=0; keep carrier busy > defer limit; verify abort |
| tc_tx_excessive_deferral_wait | EXDFREN=1; verify TX waits indefinitely for carrier |
| tc_tx_loopback | LOOPBCK=1; verify transmitted data appears on RX path |
| tc_tx_carrier_sense_lost | Assert MCrS then deassert mid-frame; verify CS bit in TX BD |
| tc_tx_only_4byte_min | TX BD with LEN ≤ 4; verify frame not transmitted (spec note) |
| tc_tx_unaligned_ptr | TX buffer pointer at non-word-aligned address; verify correct byte extraction |

Group 4: Frame Reception Tests

| Test | Description |
| --- | --- |
| tc_rx_basic_frame | PHY sends standard frame; verify assembly and storage in memory via DMA |
| tc_rx_crc_check_pass | Valid CRC frame; verify no CRC error in RX BD |
| tc_rx_crc_check_fail | Corrupt last byte of CRC; verify CRC bit set in RX BD |
| tc_rx_short_frame_reject | Frame < MINFL; RECSMALL=0; verify frame dropped |
| tc_rx_short_frame_accept | Frame < MINFL; RECSMALL=1; verify SF bit set in RX BD |
| tc_rx_too_long_frame | Frame > MAXFL; HUGEN=0; verify reception stops at MAXFL; TL bit set |
| tc_rx_huge_frame_enable | Frame > MAXFL; HUGEN=1; verify full frame received |
| tc_rx_preamble_removal | Verify 7-byte preamble + SFD stripped; only payload in memory |
| tc_rx_short_preamble | PHY sends frame with <7-byte preamble (only SFD); verify accepted |
| tc_rx_delayed_crc | DLYCRCEN=1; verify CRC checking starts 4 bytes after SFD |
| tc_rx_dribble_nibble | PHY sends odd-nibble-count frame; verify DN and CRC bits in RX BD |
| tc_rx_invalid_symbol | PHY sets MRxD=0xE (100Mbps error code); verify IS bit in RX BD |
| tc_rx_phy_error | MRxErr asserted mid-frame; verify frame aborted, no error in BD |
| tc_rx_overrun | Throttle WISHBONE master; RX FIFO overrun; verify OR bit in RX BD |
| tc_rx_ifg_minimum | Two back-to-back frames with IFG < 960ns; verify second frame dropped |
| tc_rx_ifg_disable | r_IFG=1 in MODER; verify frames accepted regardless of IFG |
| tc_rx_multi_bd | Receive frames filling multiple RX BDs; verify wrap using WR bit |
| tc_rx_no_empty_bd | All RX BDs marked non-empty; verify frame discarded, BUSY interrupt |
| tc_rx_interrupt_rxb | IRQ=1 in RX BD; verify RXB interrupt after frame received |
| tc_rx_interrupt_rxe | Receive with error, IRQ=1; verify RXE interrupt |
| tc_rx_late_collision | Late collision during reception; verify LC bit in RX BD |
| tc_rx_unaligned_ptr | RX buffer pointer at non-word-aligned address; verify correct byte select |
| tc_rx_only_4byte_min | Frame ≤ 4 bytes; verify received with CRC error (per spec note) |

Group 5: Address Recognition Tests

| Test | Description |
| --- | --- |
| tc_addr_unicast_match | Frame dest = MAC_ADDR0/1; verify accepted |
| tc_addr_unicast_mismatch | Frame dest ≠ MAC address; PRO=0; verify dropped (RxAbort) |
| tc_addr_promiscuous | PRO=1; send frame with foreign dest; verify accepted + M bit set in RX BD |
| tc_addr_broadcast_accept | BRO=0; send broadcast (FF:FF:FF:FF:FF:FF); verify accepted |
| tc_addr_broadcast_reject | BRO=1; send broadcast; PRO=0; verify rejected |
| tc_addr_multicast_hash | IAM=1; load HASH0/HASH1; send multicast matching hash; verify accepted |
| tc_addr_multicast_hash_miss | IAM=1; multicast not in hash table; verify dropped |
| tc_addr_miss_bit | PRO=1; frame doesn't match MAC; verify M=1 in RX BD |

Group 6: Flow Control Tests (Full Duplex)

| Test | Description |
| --- | --- |
| tc_fc_rx_pause_frame | PHY sends valid PAUSE frame (type=8808, op=0001); RXFLOW=1; verify TX paused for timer value |
| tc_fc_rx_pause_timer_expire | PAUSE timer counts down to 0; verify TX resumes |
| tc_fc_rx_pause_passall_0 | PASSALL=0, RXFLOW=1; PAUSE frame not stored to memory; RXC interrupt set |
| tc_fc_rx_pause_passall_1 | PASSALL=1, RXFLOW=1; PAUSE frame stored to memory + CF bit set + RXC interrupt |
| tc_fc_rx_pause_passall_1_rxflow_0 | PASSALL=1, RXFLOW=0; stored as normal frame, RXB set, no pause |
| tc_fc_rx_pause_both_0 | PASSALL=0, RXFLOW=0; PAUSE frame silently dropped |
| tc_fc_tx_pause_request | Write TXPAUSERQ=1 to TXCTRL with TXPAUSETV; verify PAUSE frame transmitted with correct timer value |
| tc_fc_tx_pause_frame_format | Verify PAUSE frame structure: dest=01-80-c2-00-00-01, type=8808, op=0001, 42B reserved, CRC |
| tc_fc_tx_pause_blocked | TXFLOW=0; write TXPAUSERQ=1; verify no PAUSE frame sent |
| tc_fc_rx_invalid_pause | Receive frame with wrong type/opcode; verify not treated as PAUSE |
| tc_fc_slot_timer | Verify slot timer generates correct pulse intervals (64-byte slot time at 10/100 Mbps) |
| tc_fc_txc_interrupt | TXFLOW=1; send PAUSE frame; verify TXC interrupt in INT_SOURCE |
| tc_fc_rxc_interrupt | RXFLOW=1; receive PAUSE frame; verify RXC interrupt in INT_SOURCE |

Group 7: Half-Duplex / CSMA-CD Tests

| Test | Description |
| --- | --- |
| tc_hd_collision_backoff | FULLD=0; inject collision; verify jam pattern (0x99999999) + binary exponential backoff |
| tc_hd_collision_retry | Inject up to 15 collisions; verify each retry uses wider backoff window; TxRetry signal |
| tc_hd_collision_window | Verify ColWindow = COLLVALID bytes from preamble |
| tc_hd_nobckof | NOBCKOF=1; verify immediate retransmission after collision (no backoff) |
| tc_hd_carrier_sense_defer | MCrS asserted; verify TX defers; DF bit set after eventual success |
| tc_hd_carrier_sense_ifg | Carrier appears within IPGR1 window; verify counter reset |

Group 8: Interrupt System Tests

| Test | Description |
| --- | --- |
| tc_int_mask_all | INT_MASK=0x00; trigger all events; verify INTA_O never asserts |
| tc_int_unmask_txb | INT_MASK.TXB_M=1; transmit frame; verify INTA_O asserts |
| tc_int_unmask_rxb | INT_MASK.RXF_M=1; receive frame; verify INTA_O asserts |
| tc_int_clear_by_write1 | Write 1 to INT_SOURCE bits to clear; verify bits clear and INTA_O deasserts |
| tc_int_busy | Fill all RX BDs; receive new frame; verify BUSY bit in INT_SOURCE |
| tc_int_multiple_sources | Trigger TX + RX simultaneously; verify all bits set, single INTA_O |

Group 9: Reset & Clock Domain Tests

| Test | Description |
| --- | --- |
| tc_rst_hard_reset | Assert RST_I; verify all registers return to reset values; TX/RX halt |
| tc_rst_mid_tx | Assert RST_I during frame transmission; verify graceful abort |
| tc_rst_mid_rx | Assert RST_I during frame reception; verify BD not corrupted |
| tc_rst_bd_preservation | BD RAM holds values after RST_I (per design doc section 3.4.1) |
| tc_clk_10mbps | Set PHY clocks to 2.5MHz (10Mbps); full TX/RX test |
| tc_clk_100mbps | Set PHY clocks to 25MHz (100Mbps); full TX/RX test |
| tc_clk_no_link | MRxClk random 2-40MHz (no link); verify MAC does not lock up |

Group 10: Stress & Corner Case Tests

| Test | Description |
| --- | --- |
| tc_stress_all_bd_tx | Fill all 128 TDs as TX BDs (TX_BD_NUM=0x80); transmit burst |
| tc_stress_all_bd_rx | TX_BD_NUM=0x00; all BDs as RX; receive burst |
| tc_stress_simultaneous_txrx | Full duplex: simultaneous TX burst and RX burst |
| tc_stress_wb_wait_states | Add random wait states to WISHBONE slave; verify no deadlock |
| tc_stress_bd_boundary_addr | Access TX BDs at 0x400, RX BDs ending at 0x7FC; boundary check |
| tc_stress_random_frame_size | Randomize frame sizes from 5 to 1600 bytes; run 1000 frames |
| tc_stress_fifo_depth | Fill TX FIFO to capacity (16 words) then drain; verify no stall |

1- tc_reg_reset_values

After asserting RST_I, every one of the 21 configuration registers must read back its documented reset value as per the specification. Uses the UVM RAL built-in reset sequence so no manual register reads are needed.

Components used

wb_slave_agenteth_reg_block (RAL)uvm_reg_hw_reset_seqeth_coverage

Test flow

1-TestAssert RST_I=1 for 5 clock cycles via wb_slave_agent

2-TestDeassert RST_I=0, wait 2 clock cycles

3-uvm_reg_hw_reset_seqIterates every register in eth_reg_block; calls reg.read() via frontdoor WISHBONE

4-wb_slave_driverDrives ADDR_I, WE_I=0, STB_I, CYC_I; waits for ACK_O

5-RAL predictorCompares read value against reg.get_reset() for every field

6-eth_coverageSamples reg_reset_cg covergroup — all register names

Pass criteria

MODER reset = 0x0000_A000 (NOPRE, NOBCKOF, FULLD bits check)

IPGT reset = 0x12, IPGR1 = 0x0C, IPGR2 = 0x12

PACKETLEN reset = 0x0040_0600

COLLCONF reset = 0x000F_003F

TX_BD_NUM reset = 0x40

MIIMODER reset = 0x64

All status/interrupt regs (INT_SOURCE, MIISTATUS) reset = 0x0

uvm_reg_hw_reset_seq reports zero mismatches

### uvm_reg_hw_reset_seq

It is a pre-built UVM sequence

Its entire job is one thing:

Read every register in your RAL model via the actual bus (frontdoor), compare each value against the reset value your RAL model declares, and report any mismatch as a UVM error.

### The Actual Source Code (Simplified)

Here is what the sequence actually does internally, stripped to its essence:

systemverilog

class uvm_reg_hw_reset_seq extends uvm_reg_sequence;

uvm_reg_block model;   // you set this — points to your eth_reg_block

task body();

uvm_reg regs[$];          // queue to hold all register handles

uvm_reg_data_t rdata;     // value read from hardware

uvm_reg_data_t exp;       // expected reset value from RAL model

uvm_status_e   status;

// Step 1: collect every register from the block

model.get_registers(regs);

// Step 2: for each register

foreach (regs[i]) begin

// Step 3: skip registers with no reset value defined

if (!regs[i].has_reset()) continue;

// Step 4: read the register from actual hardware

//         this goes through your adapter → WISHBONE bus

regs[i].read(status, rdata, UVM_FRONTDOOR);

// Step 5: get what the reset value SHOULD be

exp = regs[i].get_reset();

// Step 6: compare — only check bits that have reset values

//         (reserved bits are masked out automatically)

if ((rdata & regs[i].get_reset("HARD"))

!== regs[i].get_reset("HARD")) begin

`uvm_error("REG_HW_RESET",

$sformatf("Register '%s' expected 0x%0h after reset, got 0x%0h",

regs[i].get_full_name(), exp, rdata))

end

end

endtask

endclass

For MODER (offset 0x00): CYC_I = 1 STB_I = 1 WE_I = 0 ← read SEL_I = 4'hF ADDR_I = 0x00

DUT responds: ACK_O=1, DATA_O = current register value

adapter captures DATA_O, returns to RAL

RAL compares against 0x0000_A000

2-tc_reg_walking_one

Shifts a single 1-bit through every RW-accessible bit position in every register. For each bit position: write 1 to that bit only, read back, verify; then restore to reset value. Uses uvm_reg_bit_bash_seq.

Components used

wb_slave_agenteth_reg_block (RAL)uvm_reg_bit_bash_seq

Test flow

1uvm_reg_bit_bash_seqFor each RW register: writes all zeros (frontdoor), reads back, checks

2uvm_reg_bit_bash_seqFor each RW bit position N: writes 1 << N, reads back

3wb_slave_driverEach write: drives ADDR_I=reg_offset, DATA_I=value, WE_I=1, STB_I, CYC_I; waits ACK_O

4RAL predictorVerifies read data matches written value; flags W1C or read-only bits that differ

5uvm_reg_bit_bash_seqWrites all ones, reads back; checks RO bits unchanged, RW bits set

Pass criteria

Every RW bit in all 21 registers accepts 0→1 and 1→0 transitions

Read-only bits (MIIRX_DATA[15:0], MIISTATUS[2:0]) never change on write

No ERR_O asserted for valid register addresses with SEL_I=4'hF

ERR_O asserted when SEL_I ≠ 4'hF on any register access

### What uvm_reg_bit_bash_seq Actually Does Internally

The UVM library sequence does more than just walking 1. It runs four sub-patterns on every register. Here is its actual internal logic:

systemverilog

// Inside uvm_reg_bit_bash_seq (UVM library)

task body();

uvm_reg regs[$];

model.get_registers(regs);    // collect all registers

foreach (regs[i]) begin

bash_kth_reg(regs[i]);    // test this register completely

end

endtask

task bash_kth_reg(uvm_reg rg);

int n_bits;

uvm_reg_field fields[$];

uvm_reg_data_t  dc_mask;     // don't-care mask (reserved + RO bits)

uvm_reg_data_t  reset_val;

n_bits    = rg.get_n_bytes() * 8;   // 32 for your registers

reset_val = rg.get_reset();

rg.get_fields(fields);

// Build a mask of bits we should NOT touch or check

// (reserved bits, read-only bits, write-only bits)

dc_mask = 0;

foreach (fields[i]) begin

if (fields[i].get_access() != "RW")

dc_mask |= fields[i].get_mask();

//            ^ set those bit positions in the don't-care mask

end

// ── Pattern 1: Walking 1 ──────────────────────────────

for (int k = 0; k < n_bits; k++) begin

if (dc_mask[k]) continue;   // skip reserved/RO bits

bash_bit(rg, k, 1, reset_val, dc_mask);

//            ↑  ↑

//         bit  value to write

end

// ── Pattern 2: Walking 0 ──────────────────────────────

// First write all 1s to every RW bit

rg.write(status, ~dc_mask);

for (int k = 0; k < n_bits; k++) begin

if (dc_mask[k]) continue;

bash_bit(rg, k, 0, reset_val, dc_mask);

//            ↑  ↑

//         bit  value to write (0 this time)

end

endtask

task bash_bit(uvm_reg    rg,

int         k,          // which bit position

bit         val,        // 1 for walking-1, 0 for walking-0

uvm_reg_data_t reset_v,

uvm_reg_data_t dc_mask);

uvm_reg_data_t  wr_val;

uvm_reg_data_t  rd_val;

uvm_reg_data_t  exp_val;

uvm_status_e    status;

// ── Step A: Build the write value ──

if (val == 1)

wr_val = (1 << k);           // only bit k is 1, all others 0

else

wr_val = ~(1 << k) & ~dc_mask; // only bit k is 0, all RW bits 1

// ── Step B: Write to hardware (frontdoor WISHBONE) ──

rg.write(status, wr_val);

// ── Step C: Read back from hardware (frontdoor WISHBONE) ──

rg.read(status, rd_val);

// ── Step D: Build expected value ──

// For RW bits: expect exactly what you wrote

// For RO/reserved bits: expect their current value (unchanged)

exp_val = (wr_val & ~dc_mask) | (reset_v & dc_mask);

// ── Step E: Compare ──

if ((rd_val & ~dc_mask) !== (exp_val & ~dc_mask)) begin

`uvm_error("REG_BIT_BASH",

$sformatf("Reg '%s' bit %0d: wrote 0x%0h, expected 0x%0h, got 0x%0h",

rg.get_full_name(), k, wr_val, exp_val, rd_val))

end

// ── Step F: Restore register to reset value ──

rg.write(status, reset_v);

endtask

### Walking Through a Concrete Example — MODER Register

MODER is 32 bits wide. Reset value = 0x0000_A000.

Let us map out which bits are RW vs reserved:

Bit 31–17:  Reserved   → dc_mask bits 31–17 = 1  (skip these)

Bit 16:     RECSMALL   → RW  → test this

Bit 15:     PAD        → RW  → test this

Bit 14:     HUGEN      → RW  → test this

Bit 13:     CRCEN      → RW  → test this  (reset=1)

Bit 12:     DLYCRCEN   → RW  → test this

Bit 11:     Reserved   → dc_mask bit 11 = 1  (skip)

Bit 10:     FULLD      → RW  → test this

Bit 9:      EXDFREN    → RW  → test this

Bit 8:      NOBCKOF    → RW  → test this  (reset=1)

Bit 7:      LOOPBCK    → RW  → test this

Bit 6:      IFG        → RW  → test this

Bit 5:      PRO        → RW  → test this

Bit 4:      IAM        → RW  → test this

Bit 3:      BRO        → RW  → test this

Bit 2:      NOPRE      → RW  → test this

Bit 1:      TXEN       → RW  → test this

Bit 0:      RXEN       → RW  → test this

dc_mask = 0xFFFF_0800   (bits 31–17 and bit 11)

Now the sequence walks through 32 bit positions for MODER. Let us trace every interesting one:

#### Bit 0 — RXEN

k = 0

Step A: wr_val = (1 << 0) = 0x0000_0001

Step B: WISHBONE write

ADDR_I = 0x00   (MODER offset)

DATA_I = 0x0000_0001

WE_I   = 1

SEL_I  = 0xF

STB_I  = 1

CYC_I  = 1

→ wait for ACK_O

Step C: WISHBONE read

ADDR_I = 0x00

WE_I   = 0

→ DUT returns DATA_O

Step D: exp_val = (0x0000_0001 & ~0xFFFF0800)

| (0x0000A000 &  0xFFFF0800)

= 0x0000_0001 & 0x0000_F7FF

| 0x0000_A000 & 0xFFFF_0800

= 0x0000_0001

| 0x0000_A000  ← bits 13 and 8 are reset=1, in dc_mask? NO

wait — bits 13,8 are RW, not reserved

dc_mask only covers reserved bits

= 0x0000_0001

(reserved bits stay 0, RW bits we only wrote RXEN=1)

Step E: DUT read returns 0x0000_0001 → PASS ✓

Step F: restore: write 0x0000_A000 (MODER reset value)

Wait — step D needs clarification. The expected value for RW bits is exactly what you wrote. You wrote 0x0000_0001. Only RXEN=1. All other RW bits including CRCEN and NOBCKOF are 0. So you expect to read back 0x0000_0001. The restore in Step F puts MODER back to 0x0000_A000 before the next iteration.

#### Bit 11 — Reserved

k = 11

dc_mask[11] = 1   → continue (skip this bit entirely)

No write. No read. No compare.

The sequence just moves to k=12.

#### Bit 13 — CRCEN (reset = 1)

k = 13

Step A: wr_val = (1 << 13) = 0x0000_2000

Step B: WISHBONE write

ADDR_I = 0x00

DATA_I = 0x0000_2000   ← only CRCEN=1, everything else 0

WE_I   = 1

Step C: WISHBONE read

DUT returns DATA_O = 0x0000_2000

Step D: exp_val = 0x0000_2000   (just CRCEN bit set)

Step E: 0x0000_2000 === 0x0000_2000 → PASS ✓

Step F: restore: write 0x0000_A000

### The Walking-0 Pattern (Part 2 of the Same Sequence)

After all 32 walking-1 iterations for MODER, the sequence runs walking-0. First it writes all 1s to every RW bit:

Write all-RW-ones:

DATA_I = ~dc_mask = ~0xFFFF_0800 = 0x0000_F7FF

MODER now = 0x0000_F7FF

(every RW bit is 1: RECSMALL, PAD, HUGEN, CRCEN, DLYCRCEN,

FULLD, EXDFREN, NOBCKOF, LOOPBCK, IFG, PRO, IAM, BRO,

NOPRE, TXEN, RXEN all = 1)

Then for each RW bit position, it clears that one bit to 0:

Walking-0 for bit 0 (RXEN):

k = 0, val = 0

Step A: wr_val = ~(1 << 0) & ~dc_mask

= 0xFFFF_FFFE & 0x0000_F7FF

= 0x0000_F7FE

(every RW bit = 1 EXCEPT RXEN which is 0)

Step B: write 0x0000_F7FE to MODER

Step C: read back from DUT

Step D: exp_val = 0x0000_F7FE

Step E: compare → PASS ✓

Step F: restore to 0x0000_A000

### Now Let Us See Every WISHBONE Transaction For Just IPGT

IPGT is simpler — only 7 bits, all RW, reset = 0x12 = 0001_0010.

dc_mask = 0xFFFF_FF80   (bits 31–7 are reserved or don't exist)

Walking-1 iterations: 7 total (bits 0–6)

Walking-0 iterations: 7 total (bits 0–6)

Total WISHBONE cycles: 7×(write+read+restore) + setup + 7×(write+read+restore)

= 7×3 + 1 + 7×3 = 43 WISHBONE cycles just for IPGT

Here is every transaction:

WALKING-1 FOR IPGT:

─── Bit 0 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000001  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000001  ACK

compare 0x01 vs 0x01 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 1 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000002  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000002  ACK

compare 0x02 vs 0x02 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 2 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000004  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000004  ACK

compare 0x04 vs 0x04 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 3 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000008  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000008  ACK

compare 0x08 vs 0x08 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 4 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000010  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000010  ACK

compare 0x10 vs 0x10 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 5 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000020  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000020  ACK

compare 0x20 vs 0x20 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

─── Bit 6 ───────────────────────────────────────────────

Write: ADDR=0x0C  DATA=0x00000040  WE=1  SEL=0xF → ACK

Read:  ADDR=0x0C  WE=0             → DATA=0x00000040  ACK

compare 0x40 vs 0x40 → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

Bits 7–31: all in dc_mask → skipped entirely

WALKING-0 SETUP FOR IPGT:

Write: ADDR=0x0C  DATA=0x0000007F  WE=1  SEL=0xF → ACK

(all 7 RW bits = 1: 0111_1111)

─── Bit 0 (RXEN walking-0) ──────────────────────────────

Write: ADDR=0x0C  DATA=0x0000007E  WE=1  SEL=0xF → ACK

(0111_1111 with bit 0 cleared = 0111_1110)

Read:  ADDR=0x0C  WE=0             → DATA=0x0000007E  ACK

compare 0x7E vs 0x7E → PASS

Write: ADDR=0x0C  DATA=0x00000012  WE=1            → ACK  (restore)

... (repeat for bits 1–6)

### What Bugs This Test Catches

Each failure points to a very specific RTL problem:

Failure type                    What it means in RTL

────────────────────────────    ──────────────────────────────────────────

Read back all zeros always      Register not connected to DATA_O path

Wrong address decode in eth_registers.v

Read back all ones always       DATA_O stuck high

Missing output enable

Bit N always reads 0            That specific flip-flop not written

Bit N of DATA_I not connected to FF input

Bit N always reads 1              FF reset value wrong, or bit stuck

Walking-1 PASS but              Register has bit coupling — writing one

walking-0 FAIL on same bit      bit affects another (logic bug in DUT)

Reserved bits change            Reserved bit positions not properly

when RW bits written            masked in eth_registers.v

Value does not restore          Restore write not working

after Step F                     (same as write failure but only on

the reset value pattern)

ERR_O asserts on valid          SEL_I checking too strict, or address

register address                decode bug in eth_wishbone.v

### Total Bus Transactions Across All 21 Registers

Per register:

Walking-1: (number of RW bits) × 3 transactions (write + read + restore)

Walking-0: 1 setup write + (number of RW bits) × 3 transactions

For your 21 registers (approximate RW bit counts):

MODER       15 RW bits → 15×3 + 1 + 15×3 = 91

INT_SOURCE   7 RW bits →  7×3 + 1 +  7×3 = 43

INT_MASK     7 RW bits →  43

IPGT         7 RW bits →  43

IPGR1        7 RW bits →  43

IPGR2        7 RW bits →  43

PACKETLEN   32 RW bits → 32×3 + 1 + 32×3 = 193

COLLCONF    14 RW bits →  85

TX_BD_NUM    8 RW bits →  49

CTRLMODER    3 RW bits →  19

MIIMODER     9 RW bits →  55

MIICOMMAND   3 RW bits →  19

MIIADDRESS  10 RW bits →  61

MIITX_DATA  16 RW bits →  97

MAC_ADDR0   32 RW bits → 193

MAC_ADDR1   16 RW bits →  97

HASH0       32 RW bits → 193

HASH1       32 RW bits → 193

TXCTRL      17 RW bits → 103

──────────────────────────────

Total ≈ 1,663 WISHBONE transactions

At ~3 clock cycles each with no wait states ≈ 5,000 clock cycles

At 100MHz system clock = 50 microseconds of simulation time

This is why uvm_reg_bit_bash_seq is always run early in regression — it is fast, completely automatic, and catches a huge class of RTL connectivity bugs before you even attempt functional tests.

3- tc_bd_walking_one

Exercises all 1024 bytes of the internal BD RAM using uvm_mem_walk_seq. Verifies that the RAM correctly stores and returns data at every address without corruption. BD RAM is only accessible when the MAC is not in reset.

Components used

wb_slave_agenteth_bd_mem (uvm_mem)uvm_mem_walk_seq

Test flow

1TestEnsure MAC is not in reset (RST bit in MODER cleared)

2uvm_mem_walk_seqWrites walking-1 pattern to every 32-bit word from offset 0x400 to 0x7FC

3wb_slave_driverEach write: ADDR_I = 0x400+offset, DATA_I = pattern, WE_I=1, SEL_I=0xF

4uvm_mem_walk_seqReads back every location and compares

5TestAssert RST_I, then read BD RAM — verify values preserved

Pass criteria

All 256 32-bit words (0x400–0x7FC) store and return correct values

No ACK timeout for any BD address within 0x400–0x7FF range

Addresses outside 0x400–0x7FF do not alias into BD RAM

RAM values preserved across soft reset (RST_I) as per spec

READY bit (RD/E) prevents host access once set — verify ERR_O or no effect when written

4- tc_tx_bd_num_split

Writes 0x32 (50) to TX_BD_NUM register and verifies that exactly 50 TX BDs are accessible at 0x400–0x58C and 78 RX BDs are accessible at 0x590–0x7FC. Tests the boundary address for each region.

Components used

wb_slave_agenteth_reg_block (RAL)eth_bd_mem (uvm_mem)eth_tx_bd_num_seq

Test flow

1eth_tx_bd_num_seqWrite TX_BD_NUM = 0x32 via RAL frontdoor

2eth_tx_bd_num_seqWrite all 50 TX BDs: address range 0x400 to 0x58C (8 bytes each × 50 = 0x190)

3eth_tx_bd_num_seqWrite all 78 RX BDs: address range 0x590 to 0x7FC

4eth_tx_bd_num_seqRead back all 128 BDs and verify no overlap or corruption

5TestWrite TX_BD_NUM = 0x80 (all TX): verify RXEN auto-disabled in MODER

6TestWrite TX_BD_NUM = 0x00 (all RX): verify TXEN auto-disabled in MODER

Pass criteria

TX BD at 0x400 (first) and 0x58C (last) correctly stored and read

RX BD at 0x590 (first) and 0x7FC (last) correctly stored and read

RXEN auto-disables when TX_BD_NUM = 0x80

TXEN auto-disables when TX_BD_NUM = 0x00

Values > 0x80 are ignored (register retains previous value)

5- tc_miim_write_phy_reg

Programs a PHY configuration register by setting up MIIADDRESS, MIITX_DATA, then asserting WCTRLDATA in MIICOMMAND. Verifies the full 64-bit MIIM frame appears on the MDC/MDIO interface with correct preamble, address, and data fields.

Components used

wb_slave_agentmdio_agenteth_reg_block (RAL)eth_miim_write_seqmdio_monitoreth_scoreboard

Test flow

1eth_miim_write_seqRAL write: MIIMODER.CLKDIV = 0x28 (MDC = CLK/40)

2eth_miim_write_seqRAL write: MIIADDRESS.FIAD = 0x01 (PHY addr), RGAD = 0x00 (control reg)

3eth_miim_write_seqRAL write: MIITX_DATA.CTRLDATA = 0x2100

4eth_miim_write_seqRAL write: MIICOMMAND.WCTRLDATA = 1

5wb_slave_driverIssues all four WISHBONE write cycles with ACK_O handshake

6mdio_monitorCaptures MDC edges and samples MDIO bit stream

7mdio_monitorDecodes: 32-bit preamble(1s), ST=01, OP=01(write), FIAD, RGAD, TA=10, DATA

8eth_miim_write_seq poll MIISTATUS.BUSY via RAL read until cleared

9mdio_agentSimulates PHY acknowledging write (drives MDIO during TA)

10eth_scoreboardVerifies decoded frame fields match programmed values

Pass criteria

MDIO preamble = 32 ones (unless MIINOPRE set)

ST field = 2'b01

OP field = 2'b01 for write

FIAD[4:0] matches MIIADDRESS.FIAD written

RGAD[4:0] matches MIIADDRESS.RGAD written

TA = 2'b10 for write

DATA[15:0] matches MIITX_DATA.CTRLDATA

MIISTATUS.BUSY deasserts at end of operation

6- tc_miim_clkdiv_all

Iterates through all valid CLKDIV values (1–255) and for each value triggers a MIIM read operation, measures the MDC frequency, and verifies it equals CLK/(2×CLKDIV).

Components used

wb_slave_agentmdio_agenteth_reg_block (RAL)eth_miim_clkdiv_seq

Test flow

1eth_miim_clkdiv_seq For CLKDIV = 1 to 255: RAL write to MIIMODER.CLKDIV

2eth_miim_clkdiv_seq Trigger MIICOMMAND.RSTAT = 1 to start a read operation

3mdio_monitor Measures MDC period: records rising edge timestamps, computes frequency

4mdio_monitor Verifies MDC_freq = SYS_CLK_freq / (2 × CLKDIV) ± 1 cycle tolerance

5eth_miim_clkdiv_seqWait for BUSY = 0 before next iteration

Pass criteria

MDC frequency matches formula for all 255 values

No MDC glitches (spurious edges) between operations

BUSY correctly asserted for full duration of each read

CLKDIV=0 treated as 1 or ignored (per implementation)

We need read operation as MDC does not run continuously. It only toggles when the MIIM module is actively performing an operation.

MDC is a divided version of CLK. The counter counts CLKDIV system clock cycles, then toggles MDC. One full MDC period = 2 × CLKDIV system clock cycles.

7- tc_miim_scan_linkfail

Asserts SCANSTAT in MIICOMMAND to start continuous PHY status polling. The mdio_agent (PHY model) first returns link-OK, then drives LINKFAIL=1 in the status register. Verifies MIISTATUS.LINKFAIL reflects the PHY state and NVALID clears after first scan completes.

Components used

wb_slave_agent mdio_agenteth_reg_block (RAL)eth_miim_scan_seq

Test flow

1eth_miim_scan_seqWrite MIICOMMAND.SCANSTAT = 1; MIIADDRESS.FIAD = PHY_ADDR

2mdio_agentFirst scan response: bit[2]=1 (link OK) in PHY status register

3eth_miim_scan_seqRAL read MIISTATUS: verify NVALID=1 (invalid until first scan done)

4mdio_monitorDetects end of first scan frame; NVALID should clear

5eth_miim_scan_seqRAL read MIISTATUS: verify NVALID=0, LINKFAIL=0

6mdio_agentSecond scan response: bit[2]=0 (link fail) in PHY status register

7eth_miim_scan_seqRAL read MIISTATUS: verify LINKFAIL=1

8eth_miim_scan_seqWrite MIICOMMAND = 0x00 to stop scan; verify BUSY deasserts

Pass criteria

NVALID=1 until first complete scan frame received

NVALID=0 after first scan completes

LINKFAIL tracks PHY bit[2] of status register

BUSY stays asserted during entire scan operation

BUSY deasserts only after scan explicitly stopped

### 5. Scoreboard Architecture

wb_master_monitor  ──► tx_frame_fifo (uvm_tlm_fifo) ──►┐

eth_scoreboard

mii_monitor        ──► rx_frame_fifo (uvm_tlm_fifo) ──►┘

↕

frame comparison + status check

+ BD status readback verification

The scoreboard:

- Reconstructs full frames from WISHBONE master writes (TX path) and MII nibble captures (RX path)

- Verifies CRC correctness, frame length, padding

- Compares BD status fields after each transfer (RTRY, RL, LC, DF, CS for TX; CRC, SF, TL, OR, IS, DN for RX)

- Checks interrupt generation matches enabled masks in INT_MASK